3GPP TSG-SA2 Meeting #169	S2-2505529

19- 23 May 2025, Fukuoka, Japan	(revision of S2-2504659)

Source:	China Telecom, MediaTek Inc, CSCN

Title:	23.700-19: New solution of combined CP/UP in a PDN connection for KI#1

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This paper propose solution of combined CP+UP in a PDN connection for KI#1

1Discussion

Based on previous discussion, we can see that either CP or UP based solution has its pros and cons, e.g. CP by default has higher priority than UP data which may mean less call setup time, UP has less overhead than CP which means more efficient transport for voice data.

This solution addresses Key Issue 1 and proposes to leverage the SRB to carry IMS signalling and establish additional DRB to carry voice packets upon voice call is initiated. The IMS PDN connection only uses one DRB (for IMS voice) so there is still one more DRB left for Internet PDN connection (i.e., simultaneously IMS voice call and Internet data transfer is supported within 2 DRBs). With this solution, the priority arrangement for IMS Signaling, IMS voice and Internet data transfer can be configured as a desirable manner: IMS Signaling priority > IMS voice priority > Internet data priority.

2 Proposal

It is proposed to agree the solution of combined CP/UP bearers in a PDN connection for KI#1 into TR 23.700-19 on FS_5GSAT_Ph4_ARC as follows

* First change *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.101: "Service aspects; Service principles".

[x]	3GPP TS 23.203: "Policy and charging control architecture".

* Second change*

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 |  |  |  |
| X | x |  |  |  |

* Third change (all New)*

6.X	Solution #X: Combined CP/UP bearers in a PDN connection

6.X.1	High-level solution Principles

This solution utilizes the benefits of both CP and UP for separate transmission of IMS signalling and vioce data, by combining a CP bearer and a UP bearer in a IMS PDN connection. The solution only uses one dynamically allocated radio UP bearer for GEO call, so the remaining radio UP bearer (s) can be used for other services during the voice call, if the NB-IoT UE is assumed to support up to 2 UP radio bearers.

6.X.2	Description

This solution focus on the establishment of CP and UP bearer for IMS PDN connection. Other aspects for KI#1, e.g. how the UE decides to request IMS PDN connection based on information from EPS, is not described and is assumed to be solved by other solutions.

The CP bearer is the default bearer and established when the IMS PDN connection is setup, by using existing Control Plane CIOT optimization procedure. The CP bearer is used for transmission of IMS signalling. The UP bearer for transmission of voice data is dedicated bearer dynamically established when the voice path needs to be setup, e.g. during the SIP negotiation between two UEs. In this solution User Plane CIOT EPS Optimisation is optional but not preferred, considering it only benefits for occasional data transmission. The UP bearer can be either Non-GBR or GBR bearer which will be aligned with RAN decision.

To support combined CP and UP bearers in IMS PDN connection, the SGW needs to simultaneously maintain corresponding S11-U path and S1-U path during the IMS call.

6.X.3	Procedures

6.X.3.1	Network architecture

As specified in Figure 6.x.2.1-1, the internet PDN connection can be established to provide the internet data transfer service and the DRB1 is associated with the default EPS bearer context of the internet PDN connection, thus at most one extra DRB can be established due to non-internet services (NB-IOT only supports 2 DRBs). The UE establishes the IMS PDN connection with default EPS bearer over the SRB (e.g., the SRB1 in Figure 6.x.2.1-1), and when the voice call is initiated the dedicated EPS bearer for voice is established with the DRB2 in Figure 6.x.2.1-1. The RB carries the IMS signalling and the RB carries the voice can use different RLC mode (e.g., IMS signalling over SRB1 is RLC ACK mode, and IMS voice over DRB2 is RLC UNACK mode or RLC TRANSPARENT mode).

Figure 6.x.3.1-1: Bearer mapping for IMS voice over GEO via NB-IoT NTN connected to EPC using CP CIoT

The benefit of this approach is the priority can be configured appropriately, e.g.,: priority of [RRC signalling, NAS signalling, IMS signalling] > priority of IMS voice > priority of Internet data; as illustrated in Table 6.x.2.1-1.

| Traffics | Bearer | Configuration | Priority Level |
| --- | --- | --- | --- |
| RRC signalling, NAS signalling, IMS signalling | SRB1(-bis) (ACK Mode) | SRB by default has higher priority than any DRB | Highest |
| IMS voice | DRB2 (not ACK Mode) | e.g., Priority Level 2 if configured with QCI 1 as specified in TS 23.203 [x] Table 6.1.7-A | Middle |
| Internet data | DRB1 (ACK Mode) | e.g., Priority Level 9 if configured with QCI 10 as specified in TS 23.203 [x] Table 6.1.7-A | Low |

Table 6.x.3.1-1: Example Priority level for different type of traffics

Editor’s note:	Whether a new GBR or non-GBR QCI is needed for the dedicated EPS bearer used for voice, to fit for the characteristics of voice over GEO link is FFS.

Editor’s note:	Whether QCI=1 is sufficient or a new QCI (GBR or non-GBR) value needs to be introduced in order to express the difference between the usual voice media that assumes ptime=20ms and the GEO voice media that assumes different longer ptime value.

6.X.3.2 Combined CP/UP bearers establishment

Figure 6.X.3.2-1: Combined CP/UP bearers establishment

- The UE initiates the PDN Connectivity Request to MME to request IMS PDN connection setup. It may be an initial attach request with PDN connectivity or UE requested PDN connectivity. The UE can indicates its support of combined CP/UP bearers(it is a new capability indication) in Preferred Network Behaviour in initial attach request or TAU.

NOTE: How the UE make the decision to request IMS PDN connection for voice service, e.g. based on information from RAN or EPC, is assumed to be solved by other solutions.

- Based on UE support of combined CP/UP bearers, RAT type(NB-IoT access), UE subscription of using GEO call and network capability, the MME determines to setup the IMS PDU session and allocates S11-U resource for default bearer(CP bearer). The MME sends a Create Session Request to SGW. Before this, the MME selects the SGW supporting combined CP/UP bearers via DNS query.

The MME includes S11-U info(IP address and TEID) and new indicator of combined CP/UP bearers in Create Session Request. MME can include the QoS for default bearer applied for IMS signalling.The exact QoS value(QCI) for CP bearer can be aligned with RAN decision.

During the PDN connection setup procedures, the PGW employs an IP-CAN Session Establishment procedure with the PCRF as defined in TS 23.203 to get the default PCC rules for the UE. The PCRF may configure the QoS parameters (QCI) associated with the default bearer, based on RAT type.

- The SGW returns a Create Session Response to MME, including S11-U info of default bearer, following existing procedure.

- If separation of S11-U from S1-U is required, the SGW shall additionally include the SGW IP address and TEID for S1-U, as described in TS 23.401. This is different from the S1-U info for dedicated UP bearer applied for subsequent voice data at step 6.

If indicator of combined CP/UP bearers is received in step 2 from MME, the SGW should not change the default bearer from S11-U to S1-U.

- The MME sends Activate Default EPS Bearer request to UE. This message is contained in S1-AP Downlink NAS transport message to eNB. In the NAS message the MME includes combined CP/UP bearers indicator to the UE.

If indicator of combined CP/UP bearers is received, the UE should not change the default bearer from S11-U to S1-U.

Above steps 1-4 follows the existing procedures of PDN Connectivity establishment for Control Plane CIoT EPS Optimisation, with enhancement on the IMS PDN connection to support combined CP/UP bearers.

After the PDN Connectivity is established, the UE uses the CP bearer to transport the SIP signalling to IMS(P-CSCF)

- If a call is received in IMS, following the existing IMS procedures, or the enhanced IMS procedures for GEO NB-IoT NTN access described in other solutions, the IMS(P-CSCF) requests the PCRF to authorize the resources necessary to establish a dedicated bearer for voice data transmission. Then the PCRF follows the Dedicated Bearer Activation Procedure as described in clause 5.4.1 of TS 23.401 by sending to PGW a PCC decision provision (PCC rule with QoS policy) message to modify the IP-CAN session.

- The SGW shall allocate S1-U resource for the bearer if indicator of combined CP/UP bearers is received in step 2 from MME. The SGW should not request creation of additional CP bearer . The SGW sends the Create Bearer Request to MME including the S1-U info for the Dedicated Bearer.

- The MME sends Activate dedicated EPS bearer context request message to UE, if the number of currently activated UP bearers for the UE dose not reach the maximum number of user plane radio bearers(existing NB-IoT specification supports 2 bearers at most). In the message the MME includes QoS(QCI) and TFT for the dedicated UP bearer to the UE.

Activate dedicated EPS bearer context request message is contained in S1-AP Bearer Setup Request message to eNB. The eNB establishes the corresponsing UP bearer, based on QoS info in Bearer Setup Request message .

NOTE: The UP bearer can be either Non-GBR or GBR bearer which will be aligned with RAN decision. The exact QCI value will be aligned with RAN decision.

- The MME sends Create Bearer Response to SGW, including EPS Bearer ID and S1-U info for UP bearer from eNB

After the activation of dedicated UP bearer, the UE can transport the voice data over UP bearer based on TFT.

NOTE: Although the existing Modify Bearer Request used between MME and SGW dose not support both S1-U and S11-U, for NB-IoT via GEO access scenario there is no need to consider the mobility case which may change the both S1-U and S11-U.

6.X.4	Impacts to Services, Entities and Interfaces

The following impacts are foreseen by this solution:

MME:

- Establishment of combined CP/UP bearers in IMS PDN session, with CP for default bearer and UP for dedicated bearer.

- Indicating to UE and SGW the attribute of combined CP/UP for the IMS PDN connection

SGW:

- Establishment of combined CP/UP bearers in a PDN session, with CP for default bearer and UP for dedicated bearer

PCRF:

- Decides QoS for default CP bearer and dedicated UP bearer in IMS PDN session

UE:

- Establishment of combined CP/UP bearers in a PDN session, with CP for default bearer and UP for dedicated bearer

- The UE should not change the default bearer from S11-U to S1-U, if the IMS PDN connection is CP/UP combined.

eNB:

- Support of activating both CP bearer and UP bearer for a UE, with respective bearer QoS

* End of change *