3GPP TSG-SA WG2 Meeting #169	S2-2505580

Fukuoka, Japan, 19 – 23 May 2025	(rev of S2-2505271)

Source:	ETRI

Title:	New solution for KI#1: New sensing NFs and functionalities

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This contribution proposes a solution for KI#1 with introducing a new Sensing Management Function (SeMF) designed for deployment per coverage area for sensing.

# 1. Discussion

<Revision History – S2-2505580>

- Title is revised with more specific feature

- Architectural feature (distributed per sensing coverage) is clarified

- NEF provides sensing service provisioning and result exposure (rather than introducing new NF: SePF)

- Authorization of sensing services are done by NEF interacting with UDR

- Purpose of storing sensing data/results is clarified

- Ambigous text (i.e., "FFS", SeMF selection by AMF for sensing entity registration, multiple SeMF) was removed for further discussions in the corresponsing key issues

This contribution addresses the work tasks identified in KI#1 of TR 23.700-14 by proposing new NFs and functionalities for sensing as summarized as follows:

- New NFs

  - SeMF: Sensing Management Function

  - SePF: Sensing Provisioning Function

- NF Roles

  - SeMF

    - Sensing entity management

      - registration, discovery/selection, configuration

    - Sensing data management

      - collection, processing, storage, content

  - SePF

    - Sensing service provisioning

      - authorization

      - policy, QoS enforcement

      - SeMF discovery/selection, sensing request

    - Sensing result exposure

      - content

      - type: one-time, periodic, event

- Key functionalities

  - Sensing entity management (SeMF)

    - registration

      - gNB/UE registers itself to SeMF via N2/N1

      - with sensing capability (examples below)

        - coverage: TA, Cell ID

        - role: transmitter or receiver or both

        - QoS: range, resolution, accuracy, latency

        - resource: energy

        - feature: pre-processing

      - serving SeMF is discovered/selected by AMF with TA, Cell ID

      - support of UE (as a sensing entity) mobility is FFS

    - discovery/selection

      - SeMF discovers/selects sensing entities

      - based on the sensing capabilities to meet the sensing request from SePF

    - configuration (as per KI#6, FFS)

      - SeMF configures sensing entnties (parameter provisioning)

      - based on the sensing request from SePF, or operator's policy

  - Sensing data management (SeMF)

    - collection

      - SeMF requests/collects sensing data to/from sensing entities

      - data transport:

        - CP or UP

    - processing/reply

      - SeMF generates sensing result from sensing data collected, and reply the result to the requestor (SePF or other NFs)

      - with contextual information

    - storage

      - SeMF manages sensing data collected in its own stoage

      - for re-use with different TTLs

      - for analytics by NWDAF (FFS)

  - Sensing service provisioning (SePF)

    - Authorization

      - SePF authorizes sensing service (request) via UDM, PCF

    - policy, QoS enforcement

      - SePF enforces QoS, policies on the sensing service (request) via PCF

    - SeMF discovery/selection

      - SePF discovers/selects SeMF via NRF

      - with sensing capabilities: TA, Cell IDs, etc.

    - sensing request triggering

      - SePF receives sensing request from AF via NEF

        - content: type (object detection, tracking, environment monitoring, etc.), target (object, area), QoS (accuracy, resolution, latency), assistance information (object information, area information)

      - SePF requests sensing results to the selected SeMF

      - with target information, assistance information from AF

  - Sensing result exposure (SePF)

    - SePF reply AF with sensing result

      - content: object type, location, velocity, direction, etc.

    - with contextual information

    - type: one-time, periodic, event

# 2. Proposal

It is proposed to add the following text to TR 23.700-14.

* * * Start of Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #y> |
| #1 |  |  |
| #2 |  |  |
| #x | X |  |

* * * Next Change * * * *

6.X	Solution #X: Distributed Sensing Management Architecture per Sensing Coverage

6.X.1	Description

This solution addresses Key Issue #1 by proposing a distributed sensing management architecture per sensing coverage for supporting sensing capabilities within the 5G System. The architecture introduces a new Network Function (NF): the Sensing Management Function (SeMF) which is responsible for managing sensing entities and sensing data within a specific sensing coverage (e.g., Tracking Area, Cell ID list); and leveraging NEF functionalities for sensing service provisioning.

The SeMF is responsible for the core functionalities related to managing sensing resources and handling sensing data. Its key roles include:

- Sensing Entity Management:

This involves the registration of sensing-capable entities (gNBs or UEs or both), their discovery and selection for specific requests (as per KI#3), and their configuration (parameter provisioning, as per KI#6). Sensing entities register their capabilities including coverage (e.g., TA, Cell ID), role (e.g., transmitter, receiver, or both), sensing-related performance parameters (e.g., range, resolution, accuracy, latency), resource information (e.g., energy consumption), and features (e.g., pre-processing).

- Sensing Data Management:

This covers the collection of sensing data from sensing entities (e.g., via N1/N2 or N3), processing the collected data to generate sensing results. It also includes storing the sensing data and/or result for purposes such as enabling analytics for predictions, historical analysis, or re-use for sensing with static objects minimizing resource consumption (as per KI#4).

The NEF manages sensing services requested by 3rd-parties (e.g., AFs). Its primary roles are related to provisioning and exposing sensing results. Its key functionalities include:

- Sensing Service Provisioning:

This involves authorizing sensing service requests (as per KI#2), enforcing policies and service-specific requirements, discovering and selecting the appropriate SeMF(s) to handle a request (e.g., via NRF, using sensing coverage like TA), and triggering the sensing request to the selected SeMF(s) (as per KI#3). It also includes mapping between sensing target areas (e.g., geographical area) specified in AF requests and network-internal identifiers (e.g., TAs or Cell IDs).

- Sensing Result Exposure:

This involves receiving sensing results from SeMF(s) and exposing them to the requesting entity (e.g., AF as per KI#5). The content of exposed results may include, for example, object type, location, velocity, direction, etc. Results may be provided as one-time reports, periodically, or based on events.

6.X.2	Procedures

Figure 6.X.2-1: Overall procedure for handling sensing service request

A high-level procedure for a sensing service request may involve the following steps:

- (Assimption) Sensing entities are already registered to SeMF with their sensing capabilities, which may be covered in KI#3.

- An Application Function (AF) initiates a sensing service request towards the 5G System via NEF. The request includes details such as the sensing type (e.g., object detection, tracking, environment monitoring, etc.), target (e.g., object, area), required service-specific performance (e.g., accuracy, resolution, latency), and potentially assistance information (e.g., object/area details).

- The NEF performs authorization based on the request details and sensing service profile information (e.g., interacting with UDR).

- Based on the target area/object and required capabilities specified in the request, the NEF discovers and selects one or more suitable SeMFs. This involves mapping the sensing target area (if specified) to sensing coverage (e.g., TAs or Cell IDs) and querying the NRF for SeMF(s) matching the sensing coverage and required sensing capabilities.

- The NEF triggers the sensing request towards the selected SeMF(s), including the relevant sensing target information and assistance information derived from the AF request.

- Upon receiving the sensing request, the SeMF discovers and selects the specific sensing entities (gNBs/UEs) under its management that are capable of fulfilling the request for the specified target area/object and service-specific performance.

- The SeMF configures the selected sensing entities with necessary parameters for the sensing request.

- The SeMF requests and collects sensing data from the configured sensing entities. Transport for the sensing data collection may occur over CP (N1/N2) or UP (N3).

- The SeMF processes the collected sensing data to generate the sensing results. The collected sensing data and/or sensing results may be stored by the SeMF for further use.

- The SeMF reports the sensing results, potentially with contextual information, back to the requesting NEF.

- The NEF receives sensing results from the SeMF and exposes the sensing result to the requesting AF.

6.X.3	Impacts on Services, Entities and Interfaces

The following NFs and interfaces are impacted:

SeMF:

-	a new network function for sensing entity management and sensing data management under its sensing coverage

-	new services and interfaces for sensing entity registration, sensing data collection, and sensing request

NEF:

-	enhancement for sensing service provisioning and sensing result exposure

-	new services and interfaces for sensing service request (including subscription) from AF

NRF:

-	updates to NF profile management for discovery of SeMF

UDR:

-	updates to interfaces for sensing service authorization and storage

* * * End of Change * * * *