SA WG2 Meeting #169	S2-2505583

19 – 23 May 2025, Fukuoka, Japan

Title:	[DRAFT] LS on static energy consumption estimation

Response to:	-

Release:	Rel-19

Work Item:	EnergySys

Source:	SA2

To:	SA5

CC:

Contact person:	Dan Wang

wangdanyjy@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2503121

# 1	Overall description

During SA2 discussions on the energy consumption models defined in Annex T of TS 23.501 to be used by EIF, there isa proposal for a calculation model update  where, the consumed energy from gNB and UPF are consisted of two-part, i.e., basic energy consumption and processing energy consumption [S2-2503121 and reference 1].

- Basic energy consumption refers to the energy consumed to power up the node without any load (idle mode)

- Processing energy consumption refers to the energy consumed to serve the traffic.

The energy consumption information for the UE on UPF can be calculated by following formula:

ECUE=E'UPF×DVUEDVUPF

where

E'UPF=EUPF-EUPF,0

EUPF,0 is the basic energy consumption information, which is supposed to be provided by OAM.

This method reduces the influence of basic energy consumption which is not caused by traffic, and should not be considered for estimation of the energy consumption for the UE.

SA2 kindly ask SA5 to take the above method into consideration, and to answer the following questions:

1) Whether the basic energy consumption information of UPF and gNB in a given time window can be provided from OAM.

2) When the UPF is scaled in/out, whether the basic energy consumption information of the corresponding scaling level can be provided from OAM.

[1] Vishwanath, Arun, et al. "Modeling energy consumption in high-capacity routers and switches." IEEE Journal on Selected Areas in Communications 32.8 (2014): 1524-1532.

# 2	Actions

To SA5

ACTION: 	SA2 kindly asks SA5 to provide the answers for the above questions..

# 3	Dates of next TSG SA WG2 meetings

SA2#170	August 25 - 29	Goteborg, Sweden

SA2#171	October 13 - 17	TBD, China