3GPP TSG-SA WG2 Meeting #169	S2-2505608

Japan, Fukuoka, May 19 – 23, 2025

Source:	MediaTek Inc.

Title:	Solution for KI#3: Support of UP path adjustments for the UE

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes solution to the key issue for KI#3 on Support of UP path adjustments for the UE.

* * * Start of Change * * * *

6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |
| 1 | X |  |  |
| 2 | X | X |  |
| 3 |  | X |  |
| 4 |  | X |  |
| 5 |  | X |  |
| 6 |  | X |  |
| 7 |  | X |  |
| X |  |  | X |

6.X	Solution #X: Support of UP path adjustments

6.X.0	High-level solution principles

The solution is to address how UPF is selected or reselected by SMF

-	the energy usage information of renewable energy per slice or a set of slices and corresponding priority of renewable energy per slice in NF profile in NRF

-	after PCF updates the PCC rules based on the energy related information from EIF

The information of the renewable energy included in NF profile can be configured by operator policy by considering one or more of the energy related information.

The SMF can subscribed to EIF to obtain the energy resource usage information per slice or for a set of slices, i.e., whether the renewable energy is used or not per slice. The SMF can be notified by EIF whether renewable energy per slice is used and then the SMF is triggered to query NRF by considering whether to reselect a UPF or not for the slice related to the existing PDU Sessions

6.X.1	Description

This solution is to address the issue for the UP path adjustments by SMF for the UE based on the energy related information from EIF and query NRF for the candidates of UPFs.

The SMF can subscribe to EIF for a set of Network slices of the information whether the renewable energy is used for those slices. After subscribed by SMF, the EIF can notify the SMF if the renewable energy is used for those slices requested by SMF

When the SMF is notified by EIF for the requested slices whether the renewable energy is used, the SMF can query NRF for proper UPFs

The NF profile can contain the energy related information that can be used by SMF together with the existing parameters as shown example in below Table:

| Parameter | Description |
| --- | --- |
| Indication of renewable energy is used or not | 1: used 0: not used |
| NF Priority | Determine the relative priority among NF instances (existing parameter in clause 6.2.6.2 of TS 23.501[2]) |
| NF capacity | Static capacity information within the range 0 to 65535, expressed as a weight relative to other NF instances of the same type (existing parameter in clause 6.2.6.2 of TS 23.501[2]) |
| NF load | Dynamic load information, within the range 0 to 100, indicates the current load percentage of the NF.The existing parameter (existing parameter in clause 6.2.6.2 of TS 23.501[2]) |
| Slice ID | S-NSSAIs of the Network Function. The existing parameter (existing parameter in clause 6.2.6.2 of TS 23.501[2]) |

Example1:

SMF A is serving PDU Session 1, 2 and 3 which is respectively related to Slice a for Application a

SMF A is notified by EIF that Slice d and Slice b for Application a is using renewable energy.

SMF A queries NRF for proper UPFs to be re-selected.

NRF provides the candidates of UPFs which are UPF 1 and UPF 2.

UPF 1:

Indication of renewable energy: 1

NF Priority: 1

NF Capacity: 1

NF load: 50

Slice ID: b

UPF 2:

Indication of renewable energy:1

NF Priority: 2

NF Capacity: 2

NF load: 50

Slice ID: b

The SMF A decides to reselect UPF 1 and then perform PDU Session modification for PDU Session 1, 2 and 3.

Example 2:

UPF 1:

Indication of renewable energy is: 1

NF Priority: 1

NF Capacity: 1

NF load: 50

Slice ID: b

UPF 2:

Indication of renewable energy: 1

NF Priority: 0

NF Capacity: 0

NF load: 30

Slice ID: b

The SMF A decides to reselect UPF 2 and then perform PDU Session modification for PDU Session 1, 2 and 3.

The above examples 1 and 2 is shown how SMF can make decision for the candidate UPFs provided by NRF using the energy related information and the existing parameters in NF profile of UPF.

NOTE 1: How to consider the energy related information along with the existing parameters in NF profile is up to the operator policy.

NOTE 2: The use of NF priority and NF Capacity can be as defined in IETF RFC 2782 [yy]

The  above trigger events for SMF to (re)select a UPF for one of more PDU Sessions related to the slice(s) can be:

-	from PCF due to PCC rules change/update after PCF interacts with EIF

-	from  NF profile with a new value ("energy related value in NR profile") calculated according to operator policy by considering one or more of energy related information

6.X.2	Architectural assumption

The architecture can be referred to clause 4.2.18 of TS 23.501 [2] to enhance the service operations from EIF to other 5G NFs.

6.X.3	UP path adjustment

The UPF can be (re)selected by SMF as specified in clause 6.3.3 of TS 23.501 [2] due to e.g., UE mobility, UE traffic offloading. The UPF (re)selection can be triggered also due to the update of the energy related information. The SMF can interact with EIF to obtain the energy related information, and the SMF can (re)select a UPF based on the obtained energy related information.

On the other hand, the SMF can also utilize the NRF to (re)select a UPF based on the value calculatedby considering the energy related information notified by EIF and queries NRF.

The UPF is (re)select based on the calculated value in the parameters included in NF profile for the UPF candidates provided by NRF when the SMF queries with NRF. When a SMF is triggered to (re)select a UPF (i.e. is notified by EIF), the SMF queries NRF to get the list of candidates of UPFs, by considering the NF profiles in each of those candidates UPFs has one energy related value in NF profile. The SMF basically can select a UPF considering the parameters in NF profile of the UPFs.

As examples 1 and 2 mentioned, the SMF selects a UPF by considering energy related information along with the existing parameters.

The energy related information that can be included in NF profile can be calculated with the weight ratio defined by operators considering one or more of the energy related information listed below, e.g., :

-	whether renewable energy is used or not per slice per application or for a set of slices

-	energy status of NG-RAN

- energy efficiency

When the SMF decided to (re)select a UPF for UE traffic routing, the SMF can trigger PDU Session establishment modification which includes the N4 Session modification procedure in clause 4.4.1.3 of TS 23.502 [3] to reselect a UPF and update the corresponding N4 rules to the re-selected UPF.

Besides the above, it is possible that after performing the SM policy association modification procedure with PCF, the SMF may also trigger UP path adjustment to reselect a UPF

6.X.4	Procedures for UP path adjustment

The existing procedures as specified in TS 23.502 [3] such as PDU Session modification procedure can be re-used by SMF to reselect a UPF for UE traffic routing.

6.X.5	Impacts on existing services, entities and interfaces

SMF:

-	may reselect a UPF for a PDU Session for UE traffic routing based on

-	after interacting with PCF

-	interacting with NRF to reselect a proper UPF

PCF:

-	generate PCC rules to SMF that may trigger to reselect a UPF after interacting with EIF

EIF:

-	interact with PMF such that if subscribed by SMF, it will notify SMF the renewable energy is used per slice or for a set of slices may trigger to reselect UPF for UE traffic routing due to policy change

* * * End of Change * * * *