3GPP TSG SA WG2 Meeting #169	S2-2505413

Fukuoka, Japan, 19-23 May, 2025	(revision of S2-220xxxx)

Source:	Xiaomi

Title:	New solution for KI#2 Authorization and revocation based on sensing service conditions

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing-ARC / Rel-20

Abstract: This paper proposes a new solution for KI#2.

1. Discussion

A new key issue on Authorization and Revocation to Support Sensing Service was added as key issue #2 in TR 23.700-14. This pCR proposes a new solution to address this key issue.

2. Proposal

The following changes are proposed for TR 23.700-14.

* * * Start of Change (ALL NEW TEXT) * * *

6.X	Solution #X: Authorization and revocation based on sensing service conditions

6.X.0	High-level solution Principles

In this solution it proposes following high-level principles:

- There are two types of Authorization involved in sensing service：

- Authorization is performed for the sensing service request.

- Authorization may be performed during the sensing service operation, due to e.g., Sensing Entity(ies) reselection,

- NEF checks the authorization of sensing service consumer that sending the sensing service request.

- SF performs the authorization for the sensing service request, by checking the conditions with NF (e.g., UDR, or CCF) configured with service profile (i.e., service information containing specific conditions).

- NEF may obtain the permission/consent from the owner of the sensing object, if the user/owner is the subscriber of the network, and also the user/owner information is included in the request, for some case, house for intruder detection, body for sleep monitoring, or factory of AMR collision avoidance

- SF may re-select the Sensing Entity(ies) by checking the sensing service conditions information (e.g., a flight route, sensing target area, etc.); SF may revoke the ongoing sensing service operation when the conditions is not met.

6.X.1	Description

This solution addresses Key Issue #2 "Authorization and Revocation to Support Sensing Service".

Unlike the legacy mobile communication services which the operators strive to provide to its subscribers anytime anywhere, most of the use cases of sensing services described in TR 22.837 [x] are provided with some case specific conditions, e.g., specific flight route for UAV sensing, specific sensing geographical area/zone, specific time or duration for sensing, specific owner for in-building sensing, etc. These are also captured in TS 22.137 [2] clause 5.2.2 and the first requirement for network exposure in TS 22.137 [2] clause 5.2.3 to meet specific requested conditions.

To ensure that specific service conditions are met, this solution proposes that authorization is performed:

- for the Sensing Service request, after receiving sensing service request from sensing service consumer and before the sensing service operation is triggered in the network;

- Optionally, for the specific sensing service (e.g., tracking the UAV, etc), authorization may be also needed to check whether sensing target is aligned with the configured conditions (e.g., planned flight route for UAV), for example, when reselecting the Sensing entity(ies), or detecting the target moving out of the sensing target area, if the authorization fails, the sensing service needed to be revoked.

The criteria for sensing service request authorization and sensing operation revocation are based on the conditions of specific sensing services.

6.X.2	Procedures

6.X.2.1	Authorization for the  sensing service request

Sensing service request is sent from a sensing service consumer (e.g., a 3rd UAV operator or UTM) to an operator network. Once the NEF receives a sensing service request, it needs to  check whether the sensing service consumer is allowed to access the requested sensing service. For this check, it is proposed that the NEF needs to do the following, in order to meet different conditions of specific sensing services:

- A sensing service profile (i.e., the service-level information containing specific conditions) is configured in a NF (e.g., UDR or CCF), The service profile can also be locally configured in the NEF.

- NEF checks on condition compliance, e.g., whether the condition information from received sensing service request implies specific conditions (e.g., a flight route, an area, a duration, a houseowner, etc.) in the NF (e.g., UDR or CCF).

Editor’s Note: If the NEF supports CAPIF framework for authorization, the details of CAPIF-based authorization is to be addressed by SA WG3.

For some case, house for intruder detection, body for sleep monitoring, or factory of AMR collision avoidance, in order to protect the interests or privacy of the owner of the sensing objects or required by regulations,  additional check with the owner may still be needed to ensure the service request sent by the authorized sensing service consumer (e.g., the owner or the user authorized by the owner). Hence, it is proposed that:

The NEF may need to obtain the permission/consent from the owner of the sensing object, as long as the information of user or owner is contained in the service request explicitly or implicitly.

- If the owner is a subscriber of the network receiving the service request, the NEF may trigger the network to obtain the permission/consent from the subscriber via the UE.

- If the owner cannot be mapped to a subscriber of any mobile network, an owner list or allowed user list associated with the sensing object needs to be configured in the service profile for the NEF to retrieve.

Editor’s Note: The details of user consent mechanism is to be addressed by SA WG3.

Editor’s Note: The details of authorization of owner or user is to be addressed by SA WG3.

6.X.2.2	Authorization during the sensing target tracking

For some cases, e.g., sensing target tracking, after the sensing service is triggered by the network, the condition compliance still needs to be checked by the SF, e.g., whether the movement of the sensing service target still aligns with the planed trajectory. SF is responsible for selecting the sensing entities or controlling sensing activation/deactivation during the sensing service operations. Hence, it is proposed that:

- The SF needs to store the information implying sensing service conditions (e.g., a flight route, an area, a duration, a houseowner, etc.) once received from the sensing service request.

Sensing entities may need to be reselected due to the Sensing target’s movement. It is proposed that:

- The SF needs to be aware of the trajectory of the moving object, e.g., notified by the sensing entities or requested from the sensing entities.

- The SF may check the condition information when reselecting the sensing entities while the sensing object is moving.

NOTE x:	For example, during the flying course, if a UAV is detected to have left the coverage of an old Sensing Entity and entered into the coverage of a new Sensing Entity, the old base station needs to stop radio sensing and the new base station needs to be selected for radio sensing. If the flight route is received and stored by the SF, the SF needs to check the flight route to reselect the base station one after another along the flight route.

- The SF may revoke the ongoing sensing operation when the check of condition compliance fails.

NOTE y:	For example, if the SF detects that a flying UAV enters into the coverage of a new base station residing away from the flight route, the SF needs to deactivate the sensing operation to avoid serving the UAVs flying in an unpermitted route.

6.X.3	Impacts on Services, Entities and Interfaces

NEF:

-	Need to retrieve service profile containing service specific conditions to authorize the service request

-	Need to trigger the network to obtain the permission/consent from the subscriber via the UE

SF:

-	Need to store the information implying service conditions

-	Need to select or reselect sensing entities based on service conditions

-	Need to revoke ongoing sensing operation when the check of condition compliance fails

* * * End of Change * * *