SA WG2 Meeting #S2-169	S2-2505652

19 - 23 May, 2025, Fukuoka, Japan	(revision of S2-2504971)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6308 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifying the Femto cell sharing the access control. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Femto |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the SA2#167 meeting, SA2 sent the LS (S2-2502787) to RAN3 on NR Femto node shared by PLMN and PNI-NPN. RAN3 sends the LS response (S2-2504519) to clarify: “Since Rel-16, a physical cell shared between a PLMN and a PNI-NPN is served by separate logical RAN nodes and cells with different identities, one for PLMN and one for PNI-NPN.” So the NR femto cell can be shared by PLMN and PNI-NPN according to existing mechanism define in the 5.18. In the agreed RAN3 CR, it clarified: If NR Femto node activates a PLMN cell, legacy UE can access. If NR Femto node activates a PNI-NPN cell, the UEs whose allowed CAG list includes a CAG-ID broadcasted by the NR Femto cell can access. This CR proposes to clarify the above consideration accroding to RAN3 LS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying the Femto cell access control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent with RAN3 spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.50.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 5.50	Support for NR Femto

### 5.50.1	Overview

This clause provides an overview of 5GS functionalities and architecture to support NR Femto deployments.

NOTE:	For NR Femto deployments, it is considered that NR Femto nodes are connected to 5GC, not EPC.

For the support of NR Femto in 5GS, the architecture is described in clause 5.50.2.

For the UE access control to the NR Femto cell, the CAG concept defined in the clause 5.30.3 for PNI-NPN may be used. The Femto cell may broadcast one or multiple CAG Identifiers as specified in the clause 5.30.3 if CAG is used for access control.

The CAG owner or an authorized administrator may control/provision subscribers to allow accessing their NR Femto/CAG cells as described in clause 5.50.3.

An NR Femto Hosting Party plays the role of a CAG owner.

END OF CHANGES