SA WG2 Meeting #169	S2-2505815

Fukuoka, Japan, 19 – 23 May 2025

Source:	Intel, NTT Docomo

Title:	5GS Architecture Enhancements for Integrated Sensing

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This paper is to add new solution to key issue 1.

# 1. Introduction/Discussion

Integrated Sensing and Communication is a new feature that requires the 5GS to support. pCR proposes the architecture introducing SSMF (Sensing Service Management Function) as a new core control function with new interfaces: SSMF–RAN (N2 extensions ), SSMF–NEF (using CAPIF exposure), SSMF–UDM (to retrieve subscriber consent/capabilities), SSMF–PCF (policy and QoS control), and optionally SSMF–NWDAF/ADRF (for data analytics storage).

# 2. Text Proposal

It is proposed to agree on the following solution in TR 23.700-14.

* * * * Start of Next Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> |
| #X | X |  |

* * * * Start of Next Change * * * *

## 6.X	Solution #X:  5GS Architecture Enhancements for Integrated Sensing

### 6.X.1	High-level solution Principles

-  This solution introduces a new network function SSMF (Sensing Service management Function) to coordinate sensing services between 5GC, NG-RAN, and external applications.

-  The SSMF function includes all signalling aspects, including sensing-service authorisation, configuration of sensing entities and ongoing session control. The solution proposes to reuse DCCF defined in TS 23.288 for sensing data collection and exposure i.e., the aspects relevant collects the raw measurements, coordination with additional data sources.

### 6.X.2	Description

The 5G System is enhanced with new functionalities to support integrated sensing in a Release-20 compliant manner. Figure 6.X-1 illustrates the 5G architecture with integrated sensing support. A new Core Network function SSMF is introduced. The SSMF is responsible for coordinating sensing services between 5GC, NG-RAN, and external applications.

Figure 6.X.2-1 System architecture for network-based sensing (SBI representation)

Figure 6.X.2-2 System architecture for network-based sensing (Reference Point Representation)

SSMF communicates with existing network functions as follows:

- with the AMF over NS4 interface (service-based) or via N2 (NGAP) for configuration of sensing tasks and retrieval of sensing measurements;

- with the NEF over an Nnef_SensingExposure service for exposure of sensing services to external AFs;

- with the UDM to fetch subscription data (e.g. in case of UE based sensing UE consent information, UE capabilities);

- with the PCF for applying policies (e.g. QoS for sensing data transport, usage authorization); and

- with possible analytics functions (e.g. NWDAF/ADRF) for processing or storing sensing data.

The SSMF may be implemented as two logical components:

- Sensing-Control Function (SCF): handles all signalling aspects, including sensing-service authorisation, configuration of sensing entities and ongoing session control.

- Data-Collection and Coordination Function (DCCF): re-used for sensing use; it collects the raw measurements, coordinates any additional data sources, and (optionally) stores intermediate or final data in an ADRF.

These components can be collocated or distributed depending on deployment needs (e.g. an operator may deploy SSMF instances per region for low-latency local sensing, while a central SSMF handles large-scale aggregation).

6.X.3	Procedures

#### 6.X.3.1	General

This clause specifies procedures for end-to-end sensing-service operation using the Sensing-Service-Management Function (SSMF) described in clause 6.X.2. Following delivery path is supported:

-  SSMF-to-RAN via AMF/NGAP, as defined in clause 6.X.3.2

Editor’s Note: The procedure described in clause 6.X.3.2 with the call flow towards NWDAF/DCCF/ADRF is FFS.

#### 6.X.3.2	Procedure to request Sensing-Service (via AMF/NGAP)

Figure 6.X-3.2-1 depicts an overview of the high-level procedure for a sensing service. The procedure is summarized as follows:

Figure 6.X.3.2-1  Sensing-Service Execution via AMF/NGAP

- 1. A sensing service consumer (either an external AF or an internal network function) triggers a request for a sensing service (e.g., detecting the presence of an object in a geographic area). If the Sensing consumer is external to the operator network, the request is received via NEF using a CAPIF-compliant API. If internal, an NF (e.g. NWDAF or AMF) may call the SSMF directly via service-based interface.

- 2. The NEF authenticates and authorises the  sensing consumer and forwards the subscription to the SSMF.

- 3.[Optional] The SSMF queries UDM for any user-consent (as specified in TS 33.501[y] Annex V) or UE-policy data (provisioned as part of subscription) relevant to the requested sensing service. The SSMF retrieves subscriber consent and (optionally) UE-sensing capability information via Nudm_SDM_Get; it may subscribe to Nudm_SDM_Subscribe to receive consent-revocation events. SSMF triggers Npcf_SMPolicyControl a request to PCF for UE policy rules (QoS, regulatory constraints). If permitted, the SSMF selects one or more sensing entities (gNBs and/or authorised UEs) according to coverage, required sensing modality, network topology, and UE capabilities. The UE sensing capability is registered with the network during the registration procedure and has an established PDU session as defined in TS 23.502[x].

NOTE: Whether the user consent check specific to UE-based sensing is required depends on the June plenary decision on the scope of this study.

- 4. The SSMF (Control component) authorizes the request (ensuring the requesting entity is allowed and the target of sensing is permissible) and selects appropriate sensing entity(ies) and method. Selection criteria include the location/coverage area, required sensing type (e.g. imaging, radar, etc.), network topology, and UE capabilities. For instance, the SSMF may select one or more gNBs covering the area, instruct them to use specific beams or signals for sensing, and/or identify UEs in the area that have necessary sensors or can participate (if they consent).

Editor’s Note: The exact mechanism by which the SSMF discovers, filters, selects sensing entities is FFS and dependent on the solution for key issue 3.

- 5. The SSMF sends Namf_SensingCfg_Request configuration instructions to the chosen sensing entities. For NG-RAN nodes, this can be done via an NGAP message through the AMF. The configuration parameters include: type of sensing to perform (e.g. waveform or measurement type), duration or interval, area or target identifiers, involved UEs (if any), and reporting criteria. If UEs are involved, the gNB, upon receiving the configuration, uses RRC signalling to configure those UEs for sensing (e.g. scheduling sounding reference signals or requesting sensor data upload). UEs acting as sensors may be instructed to report measurements or raw data to the gNB or the core (depending on the procedure).

- 6. The sensing entities perform the sensing as configured. A gNB may collect reflection data from its signals or measurements from UEs. UEs may send sensor readings (radio or otherwise) either to the gNB (which forwards them) or via a user plane connection to the core. The raw sensing data is delivered to the SSMF (Processing component), either encapsulated in new NGAP messages (for small reports) or via user plane tunnels (for large-volume data, e.g. I/Q samples). The AMF delivers an NGAP SensingConfiguration message to each selected gNB; the gNB, in turn, may use RRC to configure participating UEs for sensing and reporting.

- 7. The gNB (and any UE sensors) perform the instructed sensing, then encapsulate measurements in NGAP SensingDataReport messages sent to the AMF.

- 8. The AMF forwards each SensingDataReport to the SSMF via Namf_SensingDataTransfer.

- 9. The SSMF processes the data as needed – this could involve aggregating multiple reports, running detection algorithms, or filtering noise – to produce a sensing result that meets the service request.

10. The final sensing result (which may include detected object info, positions, radio environment maps, etc., along with optional confidence metrics or timestamps) is made available to the requester. If the requester is an external AF, the SSMF provides the result to the AF via the NEF (which in turn uses the exposed API to deliver it to the AF; e.g. an API callback or response message). If the requester is internal, the result is delivered over the service-based interface. The exposure can be one-time (a single result) or streaming/periodic based on the request parameters.

Once the sensing task is completed (or if it is terminated early), the SSMF instructs involved network entities to cease the specific sensing activity and free any reserved resources. In case of a long-running periodic sensing service, the SSMF manages the lifecycle until a stop is requested or a pre-configured end condition is met.

### 6.X.4	Impacts on Services, Entities and Interfaces

-  SSMF: It is a control-plane function and does not replace any existing NF.

-  New/Extended Interfaces:

- -  NEF: The Nnef interface (between NEF and SSMF) is extended to support exposure of sensing services where NEF invokes an SSMF service for incoming request from the sensing consumer.

- -  UDM: Uses the existing interface and may require new information elements e.g., subscription data in UDM for “sensing service authorization information”

- -  PCF: Uses the existing interface and may require new information elements e.g., policy data in PCF for sensing QoS handling.

- - AMF: The AMF interface with the SSMF is supported to send the sensing configuration request for sensing entities and the sensing data response from the sensing entities to the SSMF.

END OF CHANGES