SA WG2 Meeting #169	S2-2505819

19 - 23 May, 2025, Fukuoka, Japan	(revision of S2-2505600)

Source:	ZTE, Nokia, CATT, China Mobile, Ericsson, OPPO

Title:	pCR to TS 23.369: Update Protocol Stacks

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The paper updates AIoT protocol stacks, and clarifies that the NGAP protocol is used to transport the AIoT Reader Control messages and removes related ENs.

1. Discussion

In clause 4.6 of TS 23.369, there are following ENs regarding protocol stack and AIoT message transportation:

Editor's note:	The protocol stacks between RAN and 5GC in the following clauses need to align with RAN specifications.

Editor's note:	Whether AIoT Reader Control is transported by NGAP or is part of the NGAP protocol will be updated based on RAN WG3 decision.

Based on the LS from RAN3 (S2-2504523/R3-252481), RAN3 has reached the following working agreement regarding how to transport AIOTF information.

- Previous working assumption: “Including AIOTF information containers in NGAP, rather than introducing new protocol carried by NGAP” is changed to an agreement.

- Existing NGAP Interface management procedures will be enhanced to support A-IoT.

Based on RAN3 agreement, it is proposed to remove the ENs and add the description that the AIoT information will be included in the NGAP protocol.

2. Proposal

It is proposed to capture the following changes into TS 23.369.

* * * * Start of Changes * * * *

4.6.2.2	Protocol Stack between AF and AIoT Device for NG-RAN Direct Connectivity

Legend:

-	AIoT Data: It is the application data exchanged between the AIoT Device and the AF.

-	AIoT NAS: The NAS protocol between AIoT Device and the AIOTF.

-	AIoT AS: It is between the AIoT Device and the NG-RAN as specified in TS 38.300 [5].

-	NGAP: Application Layer Protocol between the AIOTF and the NG-RAN as defined in TS 38.413 [x].

-	NGAP AIoT Information: It is the subset of NGAP information and is included in the NGAP messages over AIOT2 reference point.

Figure 4.6.2.2-1: Protocol Stack between AF and AIoT Device for Direct Connectivity option

The AIoT NAS messages between the AIoT Device and the AIOTF are transferred via the NG-RAN transparently.

4.6.2.3	Protocol Stack between AF and AIoT Device for NG-RAN Indirect Connectivity

Legend:

-	AIoT Data: It is the application data exchanged between the AIoT Device and the AF.

-	AIoT NAS: The NAS protocol between AIoT Device and the AIOTF.

-	AIoT AS: It is between the AIoT Device and the NG-RAN as specified in TS 38.300 [5].

-	NGAP: Application Layer Protocol between the AMF and the NG-RAN as defined in TS 38.413 [x].

-	NGAP AIoT Information: It is the subset of NGAP information that the AMF transparently relays between the NG-RAN and the AIOTF, and is included in the NGAP messages and the AIOT3 messages.

Figure 4.6.2.3-1: Protocol Stack between AF and AIoT Device for Indirect Connectivity via AMF option

NOTE X:	From the NG-RAN perspective, there is a single termination of N2 i.e. the AMF.

In the Indirect Connectivity option, the AMF is involved and its functionality is as defined in clause 4.5.7. The AIoT NAS messages between the AIoT Device and the AIOTF are transferred via the NG-RAN and AMF transparently.

* * * * End of Changes * * * *