3GPP TSG-WG SA2 Meeting #169	S2-2505796

Fukuoka, Japan, May 19 - 23, 2025	(revision of S2-2505564)

Source:	Ericsson, Nokia, NTT DOCOMO, Lenovo

Title:	Clause 5.3.2: Update on ADM Discovery and Selection

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: This pCR updates the clause of ADM discovery and selection to resolve EN.

1. Introduction

In clause 5.3.2, there are two factors listed for ADM discovery and selection: the AIoT device permanent ID or domain information, and the AF ID. There is an EN left from SA2#168:

Editor's note:	The further investigation of factors for the ADM selection will be needed and other factors are FFS.

To identify all the possible factors, the call flow for each use case needs to be analysed one by one. The following use cases are for the retrieval or update of device profile data:

- When the AIOTF needs to retrieve the device profile data:

In inventory procedure, after the AIOTF receives the inventory response from the AIoT device, it needs to retrieve the device profile data to perform the validation of the device. In this case, the AIoT device permanent ID or domain information can be used for ADM discovery and selection.

- When the AIOTF needs to update the last known AIOTF for the AIoT device:

After the validation of the device, the AIOTF may further update the last known AIOTF if there is a serving AIOTF change. In this case, the ADM instance is already known in the retrieving device profile step, and it is very likely that there is no need to perform the ADM discovery and selection again. Even if there may arise such needs for some reason, the AIoT device permanent ID or domain information should be used in that case.

- When the NEF needs to obtain the last known AIOTF for the AIoT device:

After receiving an AIoT service request, in case when targeting individual AIoT device(s), the NEF may fetch the last known AIOTF to send the request to this AIOTF as an optimization. In this case, the AIoT device permanent ID or domain information can be used for ADM discovery and selection.

The following use case is for the retrieval of AF authorization data:

- When the AIOTF needs to retrieve AF authorization data:

After receiving an AIoT service request, the AIOTF needs to retrieve the AF authorization data to ensure the AF is authorized to perform the service operation on the target AIoT devices within the target area. In this case, the AF ID is used for ADM discovery and selection.

Based on the analysis above, the EN can be resolved without introducing additional factors.

[Proposal-1] Resolve the EN based on the analysis above.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369 v0.3.0:

* * * Start of Change * * * *

### 5.3.2	ADM Discovery and Selection

The ADM discovery and selection function is supported by the AIOTF to select an ADM instance to retrieve the device profile data or update the last known AIOTF for the AIoT device. The AIOTF may also discover and select an ADM to retrieve AF authorization data. Similarly, the NEF uses the ADM discovery and selection function to select an ADM to obtain the last known AIOTF for the AIoT device.

When the ADM discovery is performed, the AIOTF or the NEF utilizes the NRF to discover the ADM instance(s) unless the ADM information is available by other means, e.g., locally configured. The AIOTF or the NEF selects an ADM instance based on the available ADM instances (obtained from the NRF or locally configured).

The following factors may be considered for the ADM discovery and selection for AIoT device profile retrieval or update:

-	The domain information or the AIoT device permanent ID.

NOTE X:	Based on local configuration, the AIOTF or the NEF can determine whether to use the domain information or the AIoT device permanent ID.

NOTE Y:	In case the domain information is empty, the AIOTF or the NEF uses AIoT device permanent ID for ADM discovery and selection.

The following factors may be considered for the ADM discovery and selection for AF authorization data retrieval:

-	The AF ID.

* * * End of Change * * * *