3GPP TSG-WG SA2 Meeting #169	S2-2505825

Fukuoka City, Fukuoka, JP, 19th May – 23rd May, 2025	(revision of S2-2505336, 5565, 5795)

Source:	Huawei, HiSilicon, OPPO, ZTE, Samsung

Title:	AIOTF and Reader Selection Updates

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: Update of AIOTF Selection and Reader Selection, taking into account RAN3 agreements and providing description of the process and parameters.

# 1. Introduction/Discussion

In RAN3 the following agreements have been captured:

General - Turn previous “WA: Introduce a NGAP Class 1 Inventory Request procedure.” and “WA: Introduce a NGAP Class 1 Command Request procedure” to agreements. - Define the Reader Index as INTEGER (1..65536, …). - Introduce a new “A-IoT Area”. The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message. - The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB. About Inventory - Introduce a “Correlation ID” in the respective NGAP inventory at least inside the transfer containers. - WA: Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages. - Include the AIOTF Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages. - The Inventory Request Transfer IE, also includes the following: - Device Identification for Paging (to enable paging for single device, a group of devices, all devices) - Requested Service Area Information (A-IoT Area list and/or Reader list) - Inventory Assistance Information (Approximates Number of Target A-IoT Devices). - Reader indexes and the gNB ID are mandatory included in Inventory Report Transfer IE.

The bullets highlighted in yellow above in yellow are relevant to RAN Node and Reader Selection.

Based on the above agreements clause 5.3.3 can be updated to align with them and the following ENs can be removed:

Editor’s note:	NG-RAN and RAN reader information needs to coordinate with the RAN WG(s). Details are pending RAN WG feedback.

Editor’s note:	The AIOTF or AMF configuration of NG-RAN and RAN reader information over NGAP needs to coordinate with RAN WG(s).

Editor’s note:	It is FFS how NG-RAN transfers and updates its information to the AIOTF in indirect connectivity.

The information is shared between NG-RAN and the AIOTF by OAM, and can include the list of A-IoT Areas, list of readers and the areas they serve.

In the overall process from an AF request to reader selection in the AIOTF External Target Area information and Target Area information terms were introduced, with the External Target Area information used between the AF and NEF, and the Target Area information used in service operation requests towards the NRF and AIOTF, as described in the following diagram drafting offline at SA2#168, as shown in Figure 1.

Figure 1: Naming from SA2#168

RAN Area information has been superseded by the RAN3 term Requested Service Area Information.

When an AF makes a request for an operation it is expect it can provide a parameter which describes the physical area that the operation should be performed in. For example, a geographic region (whether defined by a shape or a civic address) or an identifier for where to perform the operation.

Figure 2: Overview of different areas.

Figure 2 illustrates an area that an AIOTF can serve and different possible A-IoT Areas defined by RAN3 and as can be seen, the scope of the request from an AF can be vastly different the area that an AIOTF can handle. Ultimately, the AIOTF is responsible for determine which gNBs (and readers/A-IoT areas) to use for a request and it needs to understand the area that the AF is asking for, to enable optimal handling of the request.

The network should not expose details of gNBs, Readers or A-IoT Areas to an AF, so the network should translate an AF request into a network internal representation - Target Area information. The NF to do this translation is the NEF.

The NEF is also responsible for determining an AIOTF to handle the request, either through local configuration or through discovery via the NRF. The NRF does not do complex transformations, or understands the relationships between areas (e.g., whether a geographic shape is within another shape etc.), therefore the lookup in the NRF needs to be simple. A simple representation of the area an AIOTF can cover is required – an AIOTF Service Area.

The NEF does understand the relationships and translations from External Target Area information to Target Area Information. It can also understand how to convert from an External Target Area information into an AIOTF Service Area, very much like it does today with geographic areas to tracking areas.

As mentioned previously, it is important that the AIOTF understands what area an AF is requesting in a service operation otherwise there will be a lot of resource wastage. Take for example and AF asking for a geographic area withing the coverage of A-IoT Area 3 in the diagram. If the NEF translates that into an AIOTF Service Area and only provides the AIOTF Service Area to the AIOTF, the AIOTF will have no choice but to select all 3 gNBs in this example, and all the readers/A-IoT Areas covered it. Keep in mind this is a simple example, but it can easily be that a single AIOTF covers a large campus area or wider with multiple building with multiple floors. The resource wastage is obvious.

To enable the AIOTF to operate in the area matching, and as the areas can be complex such that a single Target Area information not describe this request with sufficient detail, to either avoid resource wastage or to ensure that the entire area requested is used providing a list of Target Area information is required. For example, if a External Target Area information requests “the 3rd floor”, but there is not a Target Area information that matches “the 3rd floor”, the AIOTF can be provided with a list of Target Area information’s of each area that is part of “the 3rd floor”, e.g., “room 1”, “room 2”, “room 3” etc.

Overall:

-	Nnef Interface: uses an External Target Area information, a preconfigured area identifier or a geographic area (e.g., a civic address or a shape),

-	Nnrf Interface: uses AIOTF Service Area information consisting of a list of preconfigured AIOTF Service Area ID(s), which the NEF determines from the External Target Area information.

-	AIOTF Interface: Uses a list of Target Area information consisting of a list of preconfigured area identifiers, which the NEF generates from the External Target Area information.

The relevant clauses are updated to add more information about what the parameters above are and the overall process of AIOTF selection including conversion from External Target Area information to a list of Target Area information to Requested Service Area Information as the AF request is processed by the system.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.369.

# * * * * First change * * * *

### 4.5.6	NRF

In addition to the functions defined in TS 23.501 [3], the NRF performs the following functions:

-	Support of new NF types AIOTF and their corresponding NF profiles. The NF profile includes the AIOTF ID or IP address, NF type and the serving area information of the AIOTF.

-	Support of AIOTF discovery based on Target Area information.

# * * * * Second change * * * *

### 5.3.1	AIOTF Discovery and Selection

The AIOTF discovery and selection functionality is to determine an AIOTF(s) to handle an AIoT service operation request.

The NEF determines AIOTF instances(s) by providing the NRF Target Area information and the NRF returning AIOTF instance(s) that match the Target Area information, or by using local configuration.

A service operation request received by the NEF from an AF may include External Target Area information and the NEF uses it to determine the Target Area information that is provided NRF, if used. The External Target Area information is a pre-configured External Area Identifier or geographic area (e.g., a civic address or shapes). The Target Area information is a list of A-IoT Areas.

NOTE x:	The mapping between A-IoT Areas and External Area Identifiers provided by an AF is deployment specific.

When the information about the target AIoT Device(s) in a service request indicates individual AIoT Device(s), the AIOTF instances(s) may be selected by taking into account the last known AIOTF instance(s) (e.g., AIOTF ID/address) for those AIoT Device(s) obtained from the ADM.

# * * * * Third change * * * *

### 5.3.3	NG-RAN Node and RAN Reader Selection

The AIOTF selects NG-RAN node(s) and optionally RAN readers based on the configured NG-RAN node selection information. The AIOTF may also select the RAN reader based on the stored last known RAN Reader information.

The AIOTF obtains the NG-RAN selection information (A-IoT Area list, RAN Reader list, and, optionally, the location served by each RAN Reader and each A-IoT Area) via OAM or local configuration.

When the AIOTF receives an AIoT service request, based on the received Target Area information in the request and the NG-RAN selection information the AIOTF selects the NG-RAN node(s) to handle the request, and determines a Requested Service Area Information for each selected NG-RAN node. If the RAN Readers within an A-IoT Area is configured in AIOTF, the AIOTF can directly select the RAN Reader(s).

The Requested Service Area Information provided to each NG-RAN node can be empty, a list of A-IoT Areas, a list of RAN Readers, or both a list of A-IoT Areas and a list of RAN Readers.

NOTE 1:	The Target Area information in a received request can span multiple NG-RAN node(s) or can be a subset of the supported AIoT areas of a single NG-RAN node.

The AIOTF sends the AIoT service request to each selected NG-RAN node, either directly or through the selected AMF, along with the corresponding Requested Service Area Information for the NG-RAN node.

If the AIOTF provides an empty Requested Service Area Information in the AIoT service request to the NG-RAN node, then the NG-RAN node may use all available RAN Readers under its coverage.

Editor’s note:	It is FFS how NG-RAN transfers and updates its information to the AIOTF in indirect connectivity.

If an AIoT service request includes information about  individual target AIoT Device(s), the AIOTF may consider the last known serving RAN Reader(s) from the AIoT Device context to determine the NG-RAN node and RAN Reader(s) for the request.

NOTE 2:	The RAN Reader ID is not exposed to the AF.

NOTE 3:	An Inventory Report from an NG-RAN node includes a RAN Reader ID that represents the AIoT Device’s location at Reader granularity.

NOTE x:	The AIOTF uses the RAN Reader ID and the NG-RAN node to update the last known serving RAN reader information in the local AIoT Device context.

# * * * * Forth change * * * *

### 6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory(AF ID, [External Target Area information], information about the target AIoT Device(s), [Approximate number of AIoT Devices]) service operation request to the NEF.

Information about the target AIoT Device(s) may include Filtering Information, as described in clause 5.8, or include complete AIoT Device Identifier(s).

The approximate number of AIoT Devices, if provided, is used to determine the number of AIoT Devices expected to respond to this inventory service operation, which is sent by AIOTF to the NG-RAN in the assistance information for NG-RAN in step 7 for proper radio resource allocation.

Editor's note:	The parameters for the inventory service operation need further definition.

2.	The NEF may further authorize the AF request as specified in clause 5.6.

The NEF determines the Target Area information from the External Target Area information, and selects one or multiple AIOTF(s) to handle the request as specified in clause 5.3.1.

3.	The NEF invokes the Naiotf_AIoT_Inventory(AF ID, [Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices]) service operation towards to the selected AIOTF(s).

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. The AIOTF may perform authorization as specified in clause 5.6. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a Correlation ID corresponding to this AF service operation request.

The AIoT Identification Information to be provided to NG-RAN can include Filtering Information, as defined in clause 5.8, or a single AIoT Device Identifier.

AIOTF performs Reader Selection, see clause 5.3.3.

The AIOTF may also use the last serving Reader to assist with determining which Readers to use for an AFs request targeting for a specific AIoT Device.

The AIOTF determines assistance information as described in clause 5.4.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT Inventory service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT Inventory service operation request as specified in clause 8.3.

7.	The AIOTF sends the Inventory Request message including the Correlation ID, the AIoT Identification Information to be included in the paging message, and assistance information to the selected NG-RAN.

Editor's note:	Whether the Inventory Request sent to NG-RAN includes indication about whether there will be a follow up command or not needs to be determined.

8.	The NG-RAN sends an Inventory Response to the AIOTF with the Correlation ID indicating that the Inventory Request is received successfully and will perform the service operation accordingly.

9.	Upon reception of the Inventory Request message from the AIOTF, the RAN Reader(s) will execute the inventory operation. The RAN Reader(s) broadcast the paging message that includes the AIoT Identification Information.

The AIoT Device determines whether it matches the AIoT Identification Information, as described in clause 5.8.

If an AIoT device matches the AIoT Identification Information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its AIoT Device ID.

Editor's note:	Whether and how the Device ID is concealed or encrypted will be determined and aligned with SA WG3.

10.	NG-RAN sends one or more Inventory Report messages to the AIOTF including the Correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s). The AIOTF stores the mapping between the Reader ID and AIoT Device ID(s).

NOTE:	When to erase the stored mapping between the Reader ID and AIoT device ID(s) is up to implementation and local configuration.

11.	The AIOTF validates the results, using local stored device information or device profile data retrieved from the ADM. The AIOTF may aggregate the results.

12.	Optionally, if the NG-RAN detects that no more AIoT Devices will respond to the inventory procedure, the NG-RAN informs the AIOTF that the procedure is complete.

Editor's note:	The details how interactions between NG-RAN and AIOTF to indicate no further Inventory Reports will be sent and completion of the procedure need to be aligned with RAN.

13.	The AIOTF reports the progress of the AIoT inventory request to the NEF by sending the Naiotf_AIoT_Notify message including a list of AIoT Device Permanent Identifier (s). The AIOTF may send multiple reports. The AIOTF in the final Naiotf_AIoT_Notify message indicates it is the last report for this operation. If multiple AIOTFs are involved in the procedure, the NEF may receive the AIoT_Notify from multiple AIOTFs.

14.	When receiving the Naiotf_AIoT_Notify message from AIOTF, the NEF informs the AF of the outcome of the AIoT_Inventory request by sending the Nnef_AIoT_Notify message(s) including the AIoT Device Permanent Identifier(s). The NEF in the final Nnef_AIoT_Notify message indicates that it is the last report for this operation.

# * * * * End of changes * * * *