3GPP TSG SA WG2 Meeting #169	S2-2505952

# 19-23 May 2025, Fukuoka, Japan										revision (S2-2505706)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1601 | rev | 7 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: Corrections to IMS Subscribe/Notify Framework Architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Ericsson, Nokia, ZTE, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Corrections to the IMS SUB/NOT Framework. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove event category from call flows. Clarified the interworking event. Include AF Identifier as additional input to the subscribe request by DC AS so that NEFand HSS can perform necessary authorization prior to hanlding the request. Clarify the current status as optional output in subscribe responses. Alignment with CT3 for procedures. In the rev07: HSS subscribe with IMS AS for only those subscribers for which IMS DC service is enabled. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification for some aspects, and confusing specification for some other aspects. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.1.2.1, AA.2.1.5.1, AA.2.2.3, AA.2.4.5.1, AD2.4.5.2, AA.2.4.5.3, AC.2.2.6, AD.1, AD.2.3.2, AD.2.2.3, AD.2.4, AD.2.5.2, AD.3.1, AD.3.2, AD.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### AA.2.1.2.1	Nhss_ImsUECM_Registration service operation

Service operation name: Nhss_ImsUECM_Registration

Description: If S-CSCF is the consumer, this service operation registers the serving S-CSCF assigned to an IMS User. If authentication is not to be performed, this operation also sets the registration state. The S-CSCF is implicitly subscribed to be notified when it is deregistered in HSS. This notification is done by means of Nhss_ImsUECM_DeregistrationNotification operation.

If IMS AS is the consumer, this service operation registers the IMS AS instance assigned to an IMS user.

Inputs, Required: Public Identity, S-CSCF name and Registration Type (e.g. Initial Registration, Unregistered) if the S-CSCSF is the consumer, IMS AS instance Id if the IMS AS is the consumer.

Inputs, Optional: Private Identity.

Outputs, Required: Result indication.

Outputs, Optional: List of registered Private Identities sharing the same Public Identity which is being registered, S-CSCF Restoration indication.

**** Next Change ****

#### AA.2.1.5.1	Nhss_ImsEE_Subscribe service operation

Service operation name: Nhss_ImsEE_Subscribe

Description: The NF consumer subscribes for a specific subscriber related IMS event.

Inputs, Required: IMS Public Subscriber identity, Event-ID(s) representing the event of interest, Notification Target address, Event Reporting Information as defined in Table 4.15.1-1 of TS 23.502 [94].

Event-ID represents one or multiple events as defined in clauses AD.2.4, and AD.2.5.

Inputs, Optional: Notification Correlation ID, Expiry time, Event Filters, Subscription Correlation ID (in the case of modification of the event subscription), AF Identifier.

NOTE X: The semantics of the Event ID and Event Filters are transparent to the HSS.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can expire based on the operator's policy).

Outputs, Optional: Current status related to the subscribed event type is included, if Immediate Reporting Flag is included in the request within Event Reporting Information and if available (see clause 4.15.1 of TS 23.502 [94]).

**** Next Change ****

### AA.2.2.3	Mapping of Sh messages to HSS SBI services

The following table defines the mapping between stage 2 Sh messages and HSS SBI services and service operations:

Table AA.2.2.3-1: Sh messages to HSS SBI services and service operations mapping

| Sh message | Source | Destination | HSS SBI service operation name |
| --- | --- | --- | --- |
| Sh-Pull | AS | HSS | Nhss_ImsSDM_Get |
| Sh-Update | AS | HSS | Nhss_ImsSDM_Update |
| Sh-Subs-Notif | AS | HSS | Nhss_ImsSDM_Subscribe Nhss_ImsSDM_Unsubscribe Nhss_ImsSDM_Get |
| Sh-Notif | HSS | AS | Nhss_ImsSDM_Notification |

NOTE  : 	Sh procedures to HSS or IMS AS are defined in TS 29.328 [79].

**** Next Change ****

#### AA.2.4.5.1	Nimsas_ImsEE_Subscribe service operation

Service operation name: Nimsas_ImsEE_Subscribe

Description: The NF consumer subscribes for a specific subscriber related IMS Event.

Inputs, Required: Event-ID(s) representing the event of interest, Notification Target address, Event Reporting Information as defined in Table 4.15.1-1 of TS 23.502 [94].

Event-ID represent one or multiple events as defined in clauses AD.2.4, and AD.2.5.

Inputs, Optional: IMS Public Subscriber identity if the subscribe request is for a subscriber specific event, Notification Correlation ID, Expiry time, Event Filters, Subscription Correlation ID (in the case of modification of the event subscription).

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can expire based on the operator's policy).

Outputs, Optional: Current status related to the subscribed event type is included, if Immediate Reporting Flag is included in the request within Event Reporting Information and if available (see clause 4.15.1 of TS 23.502 [94]).

**** Next Change ****

#### AA.2.4.5.2	Nimsas_ImsEE_Unsubscribe service operation

Service operation name: Nimsas_ImsEE_Unsubscribe

Description: The NF consumer unsubscribes for updates to Requested event.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

**** Next Change ****

#### AA.2.4.5.3	Nimsas_ImsEE_Notify service operation

Service operation name: Nimsas_ImsEE_Notify

Description: This service operation is used by the IMS AS to report the information associated with a subscribed Event.

Inputs, Required: Event ID, Event Reporting information, Notification Correlation ID.

The Event ID parameter defines the event for which the notification applies.

Inputs, Optional: Event information (defined on a per Event ID basis).

Outputs, Required: Operation execution result indication.

**** Next Change ****

### AC.2.2.6	HSS

In order to support data channel service, the HSS is additionally enhanced with the following functionalities:

-	The HSS stores IMS Data Channel subscription data as transparent data.

-	The HSS interacts with DCSF during service data retrieval by DCSF via N72/Sc.

-	The HSS interacts with IMS AS during service data retrieval by IMS AS via N71/Sh.

-	The HSS interacts with IMS AS to subscribe to event notifications on behalf of other NF, e.g. NEF or DC application server.

-	The HSS supports IMS DC event subscription as specified in Annex AD.2.3.2.

-	HSS authorizes subscription requests for subscriber specific IMS DC service events before forwarding them to IMS AS.

**** Next Change ****

# AD.1	General

This annex describes the Subscribe/Notify framework to support event monitoring in the IMS architecture. The framework enables third party AF and network functions to subscribe to IMS DC events.

IMS event exposure for event monitoring allows a consumer (e.g. a DC AS) to subscribe to and being notified about subscriber specific and non-subscriber specific IMS events and to invoke IMS services exposed by the IMS network via the NEF.

A non-subscriber specific event monitoring request does not contain any target subscriber. Non-subscriber specific event monitoring requests are sent by the NEF directly to all IMS AS instances which support monitoring of the requested IMS event. The NEF may be configured with available IMS AS instances, or the NEF can discover IMS AS instances via NRF.

A subscriber specific event monitoring request includes the targeted subscribers. The targeted subscribers can be either one distinct subscriber or multiple subscribers indicated by their IMS public user identities. The events in the subscriber specific event monitoring request are common to all targeted subscribers. The NEF sends one event subscription request per targeted subscriber to the HSS serving the given subscriber. The NEF may determine the serving HSS based on configuration or based on HSS discovery as depicted in clause AA.3.3.

Once an IMS subscriber successfully registers in IMS, and one of the IMS AS is assigned to the subscriber, the IMS AS which supports monitoring of IMS events registers its address in the HSS.

For subscriber specific event monitoring requests received from the AF (e.g. DC AS), the NEF uses Nhss_ImsEE service to manage the subscription requests (e.g. subscribe/unsubscribe) via HSS. HSS uses Nimsas_ImsEE service to manage the subscriber specific subscription requests with the IMS AS assigned to the subscriber.

**** Next Change ****

### AD.2.3.2			HSS

HSS handles subscriptions to IMS DC events related to registered and unregistered IMS subscribers and ensures that a subscription is persistent when UE registers, deregisters and re-registers in IMS.

HSS receives subscription requests for subscriber specific events from NEF via N91 reference point using the Nhss_ImsEE_Subscribe service operation.

HSS stores the incoming subscription request and sends the subscription request to the IMS AS instance supporting the requested event and assigned to the IMS subscriber via N71 reference point, using the Nimsas_ImsEE_Subscribe service operation, or via Sh reference point.

The IMS AS assigned to an IMS subscriber is registered in HSS via N71 reference point using the Nhss_ImsUECM_Registration service operation, or via Sh reference point. If the IMS subscriber in the incoming subscription request is not registered, HSS ensures that when the IMS subscriber registers, any pending subscription request is sent to the applicable IMS AS instance assigned to the IMS subscriber.  For subscriptions to IMS DC events, HSS sends the subscription request to an IMS AS for a subscriber that has the IMS DC service enabled in the subscription profile.

NOTE: HSS determines that a subscriber has the IMS DC service enabled in the user subscription service profile based on implementation means.

HSS provides the NEF address in the notification end points while subscribing with IMS AS so  that corresponding notifications are directly sent to the NEF as subscribing entity.

**** Next Change ****

### AD.2.3.3	IMS AS

Once an IMS subscriber successfully registers in IMS, any IMS AS allocated to the subscriber that supports monitoring of IMS events and DC services shall register in HSS its IMS AS instance via N71 reference point using the Nhss_ImsUECM_Registration service operation, or via Sh reference point.

The IMS AS receives subscription requests from HSS related to supported subscriber specific IMS DC events via the N71 reference point using the Nimsas_ImsEE_Subscribe service operation, or via the Sh reference point. IMS AS also receives subscription requests from NEF related to non-subscriber specific events via the N92 reference point using the Nimsas_ImsEE service operation.

IMS AS sends notification events directly to the NEF as subscribing entity included in the subscription request via N92 reference point using the Nimsas_ImsEE_Notify service operation.

**** Next Change ****

## AD.2.4	Determination of IMS Events supported by an IMS AS

The NF profile registered in NRF of an IMS AS that supports monitoring of IMS events may include support for the Nimsas_ImsEE service and related service operations. Additionally, the NF profile of the IMS AS may include the list of IMS events the IMS AS instance supports. NF consumers of the Nimsas_ImsEE service (i.e., HSS and NEF) can make use of this information in the NF profile of the IMS AS to determine the IMS AS instances that support monitoring of a given IMS event.

**** Next Change ****

### AD.2.5.1	General

Subscription requests for the notification of an IMS event type may include event filter information, e.g. condition for notifying the event. As an optimization option, a subscriber can subscribe to a number of events in a single subscription. In this case, event filters are applied to each event independently for event detection. Additionally, notification to detected events are handled individually.

The event filter depends on the event type. The IMS events and corresponding event filters are only meaningful for the AF and the IMS AS. The involved NF consumer of the IMS AS (e.g. HSS/NEF) determines which IMS AS supports a given IMS event type as described in clause AD.2.4 or by implementation means.

NOTE 1:	The actual value of IMS event types and corresponding event filters and reports are transparent for the NRF, HSS and NEF. This allows that additional standardized as well as non-standardized event types can be supported without the need to upgrade the NRF, HSS or NEF.

Subscription requests for the notification of an IMS event shall include Event Reporting information. The Event Reporting information defines the type of reporting requested (e.g. event reporting mode, number of reports, maximum duration of reporting). The contents of the Event Reporting Information along with the presence requirement of each information element is described in Table 4.15.1-1 of TS 23.502 [94].

NOTE 2:	Not all of the Event Reporting Information described in Table 4.15.1-1 of TS 23.502 [94] is applicable to the Exposure Events in IMS.

NOTE X: 	If Immediate Reporting Flag is included in the subscription request within Event Reporting Information, the current status related to the subscribed event type is included in the response, if available (e.g. whether the bootstrap data channel is currently established or not). The current status related to the subscribed event type included in the response is not considered as an event report (i.e. it is not considered a notification on event detection) and it is not deducted from the maximum number of reports if included in the request within the Event Reporting Information.

**** Next Change ****

### AD.2.5.2	Event Monitoring

Table AD.2.5.3-1 enumerates the IMS DC exposure events and their detection criteria:

Table AD.2.5.3-1 List of events for monitoring capability

| Event | Description and Detection criteria | Which NF detects the event |
| --- | --- | --- |
| Bootstrap Data Channel established | Detected when a Bootstrap data channel is established. Network detects that a Bootstrap data channel is established when the SDP for an established IMS DC includes the media for a Bootstrap data channel with the corresponding Bootstrap DC stream ID according to TS 26.114 [76]. | IMS AS |
| Bootstrap Data Channel Release | Detected when an established Bootstrap data channel is released. Network detects that a Bootstrap data channel is released when an IMS DC that includes an SDP with the media for a Bootstrap data channel with the corresponding Bootstrap DC stream ID according to TS 26.114 [76] is released, or an IMS DC is updated to remove the media for a Bootstrap data channel with the corresponding Bootstrap DC stream ID according to TS 26.114 [76]. | IMS AS |
| Application Data Channel established | Detected when an Application Data Channel is established. Network detects that an Application data channel is established when the SDP for an established IMS DC includes the media for an Application data channel with the corresponding Application DC stream ID according to TS 26.114 [76]. The Application DC stream ID can be explicit or any stream ID. | IMS AS |
| Application Data Channel Release | Detected when an Application data channel is released. Network detects that an Application data channel is released when an IMS DC that includes an SDP with the media for an Application data channel with the corresponding Application DC stream ID according to TS 26.114 [76] is released, or an IMS DC is updated to remove the media for an Application data channel with the corresponding Application DC stream ID according to TS 26.114 [76]. | IMS AS |
| Application Data Channel requires interworking is established | Detected when an Application Data Channel that requires interworking is in the process of establishment. Network detects that an P2A Application data channel is in process of establishment and the MDC2 resources are assigned for the Application data channel. | IMS AS |

Event Filters are used to specify the conditions to match for notifying the events (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following list provides an example how the conditions to match for event reporting can be specified for various Event IDs for IMS DC exposure events:

-	Calling ID: Identifies the public identity of calling subscriber in the targeted IMS session if present.

-	Called ID: Identifies the public identity of called subscriber in the targeted IMS session if present.

-	Media Information Set: If present, includes a set of media information indicating how the media used in the created targeted session for the selected event. Each media information contains:

-	Media Type: It indicates the media type used in the created session, e.g., Data Channel.

-	Media Parameter: It indicates the related media parameters. Different media, includes different parameters.

-	When the Media Type is Data Channel, it indicates the Data Channel Type (ADC or BDC).

-	When the Data Channel Type is BDC, it indicates local or remote, for local UE or for remote UE.

-	When the media type is ADC, Data Channel application binding information is included.

-	Session ID: Indicates the targeted Session ID if present.

**** Next Change ****

## AD.3.1	IMS AS instance Registration in HSS

Figure AD.3.1-1depicts a signalling flow diagram for an IMS AS registering in HSS the IMS AS instance assigned to a registering UE.

Figure AD.3.1-1: IMS AS Instance Registration in HSS

1.	UE performs initial IMS Registration (see clause 5.2).

2.	S-CSCF performs third party Registration with the IMS AS instance assigned to the registering UE.

3.	IMS AS assigned to the UE/IMPU registers in HSS using Nhss_ImsUECM_Registration service operation or using Sh interface.

NOTE X:	Figure AD.3.1-1 shows only SBA services. Corresponding Sh procedure is not included in the Figure.

NOTE Y:  Multiple IMS AS instances can be assigned to a given IMPU (e.g. MMTel AS, SCC AS). In this release only IMS AS supporting MMTEL AS having IMS DC capability registers its instance in HSS.

**** Next Change ****

## AD.3.2	Subscribe/Notify Procedure for subscriber specific IMS events

Figure AD.3.2-1depicts a signalling flow diagram for establishing a subscription for notification of a subscriber specific IMS event.

Figure AD.3.2-1: SUBSCRIBE/NOTIFY Procedure for Subscriber specific IMS events

The steps in the call flow are as follows:

1.	AF subscribes to NEF initiating the Nnef_imsEE_Subscribe Request for a subscriber specific IMS event. The AF may include one or more IMS subscriber IDs in the subscription request.

2.	If the subscription to the subscriber specific IMS event is authorized by the NEF, the NEF records the association of the event and the AF identity. Otherwise, the NEF continues in step 8 indicating failure.

NEF initiates for each IMS subscriber in the incoming subscription request a separate subscription request towards HSS for the requested event via the Nhss_ImsEE_Subscribe Request service operation. The NEF creates a Notification Target address, and Notification Correlation ID and includes it to the Nhss_ImsEE_Subscribe Request.

3.	If the subscription to the subscriber specific IMS event is authorized by the HSS, the HSS records the association of the event and the NEF identity. Otherwise, the HSS continues in step 7 indicating failure.

HSS locates the IMS AS instance serving the UE based on the IMS AS registration in HSS defined in clause AD.3.1. The HSS determines if the IMS AS instance for the IMS subscriber (IMPU) supports the requested IMS event based on the NF profile of the registered IMS AS instances stored in NRF.

If the UE is not IMS registered in HSS, steps 4 and 5 are skipped.

NOTE 1:	If the UE is not IMS registered as in step 1, then the HSS will store the event subscription request per IMPU.

4.	HSS subscribes to the IMS AS instance serving the UE and supporting the requested IMS event, using Nimsas_ImsEE_Subscribe Request for the requested IMS event including the IMPU, and the NEF address as notification endpoint. Alternatively, HSS can use Sh interface to send the requested IMS event to the IMS AS instance assigned to the UE.

5.	IMS AS returns to HSS with the Nimsas_ImsEE_Subscribe Response or returns to HSS via Sh interface.

6.	HSS returns to NEF the Nhss_ImsEE_Subscribe Response.

7. NEF returns to AF the Nnef_ImsEE_Subscribe Response.

8.	At some point, the requested event for the UE is detected by the IMS AS.

9.	The IMS AS sends an Nimsas_ImsEE_Notify Request to NEF.

10.	NEF sends to IMS AS Nimsas_ImsEE Notify Response to the IMS AS.

11.	The NEF maps the Notification Correlation ID to the subscription in the NEF. NEF sends Nnef_ImsEE_Notify Request to AF using the Notification Target Address and Notification Correlation ID received in step 2.

12.	The AF sends an Nnef_ImsEE_Notify Response to the NEF.

NOTE 2:	Figure AD.3.2-1 shows only the SBA services. Corresponding Sh procedure is not included in the Figure.

**** Next Change ****

## AD.3.3	Subscribe/Notify Procedure for Non-subscriber specific IMS events

Figure AD.3.3.1-1 depicts a signalling flow diagram for establishing a subscription for notification of a non-subscriber specific IMS event.

Figure AD.3.3-1: SUBSCRIBE/NOTIFY Procedure for non-subscriber specific IMS events

The steps in the call flow are as follows:

1.	AF subscribes to NEF initiating the Nnef_imsEE_Subscribe Request for non-subscriber specific IMS event.

2.	If the subscription to the subscriber specific IMS event is authorized by the NEF, the NEF records the association of the event and the AF identity. Otherwise, the NEF continues in step 8 indicating failure. NEF locates the IMS AS instances that support the requested IMS event via NRF or by local configuration.

3.	For each IMS AS instance that supports the requested IMS event, NEF subscribes to the IMS AS using Nimsas_ImsEE_Subscribe Request for the requested IMS event.

4.	IMS AS returns to NEF the Nnef_ImsEE_Subscribe Response.

5.	NEF returns to AF the Nnef_ImsEE_Subscribe Response.

6.	At some point, the requested event for the UE is detected by the IMS AS.

7.	The IMS AS sends an Nimsas_ImsEE_Notify Request to NEF.

8.	NEF sends to IMS AS Nimsas_ImsEE Notify Response.

9.	NEF sends Nnef_ImsEE_Notify Request to AF.

10.	The AF sends an Nnef_ImsEE_Notify Response to the NEF.

**** End of Changes ****