TSG SA WG2 Meeting #169	S2-2506097

19th - 23rd May, 2025, Fukuoka, Japan

Title:	Reply LS on Transport Level Marking on N3/N9

Response to:	LS on Transport Level Marking on N3/N9 (C4-250629/S2-2502815/S2-2504574)

Release:	Release 19

Work Item:	XRM_Ph2

Source:	SA2

To:	CT4

Cc:	-

Contact Person:

Name:	Lizhuo Zheng

E-mail Address:	zhenglz4@lenovo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	23.501 CR (6193)

1. Overall Description:

SA2 has discussed CT4 (C4-250629/S2-2502815) LS on Transport Level Marking on N3/N9 and would like to provide the following answers and attached the agreed CR (6193):

5.8.2.7    PDU Session and QoS Flow Policing

ARP is used for admission control (i.e. retention and pre-emption of the new QoS Flow). The value of ARP is not required to be provided to the UPF.

For every QoS Flow, the SMF shall determine the transport level packet marking value (e.g. the DSCP in the outer IP header) based on the 5QI, the Priority Level (if explicitly signalled) and optionally, the ARP priority level and provide the transport level packet marking value to the UPF.

For QoS Flows that are configured for PDU Set QoS handling, the SMF may additionally take into account the PDU Set Importance when determining the transport level packet marking values. In this case, the SMF provides a list of transport level packet marking values for the downlink direction to the UPF in a FAR, each of the transport level packet marking values corresponding to one or more PDU Set Importance values. When an I-SMF/I-UPF is inserted in the PDU Session, the I-SMF may instruct the I-UPF to derive the transport level packet marking of the outgoing N3 downlink packet based on the transport level packet marking of the incoming N9 downlink packet.

CT4 would like to ask if the highlighted requirement in yellow implies a new requirement for N4 interface specific for XRM_Ph2?

SA2 reply: The I-SMF may provide an indication in the FAR to derive the transport level marking of the outgoing packet based on the transport level marking value of the incoming packet . Then, I-UPF shall derive the transport level marking value of the outgoing N3 packet based on the incoming marking value of the N9 packet.

If so, how can the I-SMF learn from the anchor SMF to instruct the I-UPF for a QoS flow to derive the transport level packet marking of the outgoing N3 downlink packet based on the transport level packet marking of the incoming N9 downlink packet?

SA2 Reply: For downlink, the anchor SMF may send an indication that transport level marking is being applied and consistency of transport level marking is needed for the corresponding QoS Flow to the I-SMF via N16a interface.

More generally, could SA2 clarify how transport level marking should be performed by I-UPF?

SA2 reply:

In general,

- If the I-SMF receives an indication from anchor SMF that transport level marking is being applied and consistency of transport level marking is needed for the corresponding QoS Flow, then it instructs the I-UPF to derive the transport level marking value of the outgoing N3 packet based on the transport level marking value of the incoming N9 downlink packet.

- If no such indication is received at I-SMF,

  - The I-SMF can provide a transport level marking value in the FAR, and then the I-UPF performs the transport level marking for the QoS flow based on this transport level marking value.

  - If no transport level marking value is included in the FAR, the I-UPF performs transport level marking based on its pre-configured logic, e.g., using a pre-configured value, or deriving the outgoing marking value based on the incoming marking value, or mapping tables or others.

2. Actions:

To CT4:

ACTION: 	SA2 kindly asks CT4 to take the above into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #170		25-29 August 2025	Goteborg, Sweden

TSG-SA2 Meeting #171		13-17 October 2025	Wuhan, China