SA WG2 Meeting #169	S2-2504586

19 - 23 May, 2025, Fukuoka, Japan

Title:	Reply LS on Questions on stage 2 requirements for AIML_CN

Response to:	S2-2504575/C4-250630 LS on Questions on stage 2 requirements for AIML_CN

Release:	Rel-19

Work Item:	AIML_CN

Source:	Oracle (to be SA2)

To:	CT4

Cc:	CT3

Contact person:	Uri Baniel

uri.baniel@oracle.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA2 thanks CT4 for the LS C4-250630 on Questions on stage 2 requirements for AIML_CN.

After discussion, SA2 provides answers for the 4 Questions:

Q1: Given that the metrics can be reported by OAM APIs, is it necessary for the NRF APIs to also support reporting the same metrics, and if so, can SA2 explain why?

A1: It is common across 23.288 that NWDAF may get similar inputs from multiple alternative resources. If one resource cannot provide the inputs (e.g OAM) then another resource is needed to provide the inputs (e.g NRF). SA2 will be willing to define a new Event Exposure API on the NRF, so that it can provide NWDAF with inputs required by Table 6.22.2-5.

Q2: If the answer to Q1 is yes, this required functionality does not fit with the current scope of the Nnrf_NFManagement_NFStatusSubscribe service operation. CT4 kindly asks SA2 to clarify which service operation is suitable for this purpose?

A2: SA2 prefers to introduce a new event exposure service for NRF. The Nnrf_EventExposure new service will include: Notify, Subscribe and Unsubscribe service operations.

Q3: Accordingly, should NF metrics related to signalling storm information be reported per Service Name (i.e. with the NF instance aggregating metrics of all NF service instances supporting the same service name) or per NF service instance?

A3: It can be reported on a level of NF service name (for a specific NF instance) or on a level of NF instance (all service names) or on a level of NF service instance.

NOTE: Knowing the type and number of requests from a specific NF instance can help identify the exact cause for the signaling storm; However, it can create an overhead. Hence, flexible approach is proposed.

Q4: Does “Service Type” refer to “service name”? If the answer is yes, should SCP metrics related to signalling storm information be reported per "Service Name" or "Service instance"? If the answer is no, what does "Service Type" refer to?

A4: SCP metrics can be reported either per NF service instance or per service name. Note in A3 above provides more info.

NOTE: SCP can identify the NF service instance based on HTTP headers. CT4 can provide more info on that.

# 2	Actions

To CT WG4:

ACTION: 	SA2 kindly asks 3GPP CT4 to take the above information into consideration

# 3	Dates of next TSG SA WG meeting

SA2#170				        25-29 Aug 2025				     Goteborg, Sweden

SA2#171				        13-17 Oct 2025				     China, Wuhan