3GPP TSG-SA2 Meeting #169	S2-2504663

19- 23 May 2025, Fukuoka, Japan

Source:	China Telecom, Thales, TNO, Eutelsat, Novamint

Title:	23.700-19: Discussion and Interim conclusion for UE-SAT-UE communication for non-IMS services

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This paper discusses the existing mechanism and the issues of session and service continuity to support UE-SAT-UE communication for non-IMS services, and concludes the Interim conclusion.

1Discussion

-	WT-2: UE-SAT-UE communication enhancement for 5GS under NR NTN NGSO constellation with feeder link available.

-	WT-2.2 UE-SAT-UE communication via UPF only onboard satellite for non-IMS services, e.g. a 5G-LAN service with IP and Ethernet PDU Session type.

NOTE 8: Whether existing mechanism can be reused to support WT-2 needs to be checked.

NOTE 9: No UE impact is expected for WT-2.

The Work Task#2.2 of the FS_5GSAT_Ph4_ARC SID is copied as below:

The existing R18 mechanism (described in 5.43.3 of TS 23.501) supports UE-to-UE communications via UPF(s) deployed on GEO satellite(s) used for backhauling. In that mechanism, UE to UE traffic may be locally routed by UPF(s) deployed onboard GEO satellite(s) based on traffic forwarding feature of 5G VN. If the UEs connect to different satellites, then UPFs deployed on different satellites can use N19 tunnel or N6 to carry traffic between them. The SMF can use DNAI derived from a GEO satellite ID to determine the UPF onboard satellite, then setup the traffic routing between UPFs for traffic of UE-to-UE communications by using 5G VN mechanisms. Change of UPF onboard satellite is not considered in R18 for cases when GEO satellite is used for backhauling.

In R18 only a single SMF is supported for local switching and N19 forwarding, i.e. both UEs are served by the same SMF. Since 5G VN already supports multiple SMFs within an SMF set, by using implementation based mechanisms between SMF(s) of the same SMF set (as described in 5.8.2.13 of TS 23.501), then it can be easily extended to support SMF set deployment for the case of Work Task#2.2 of the FS_5GSAT_Ph4_ARC.

In R18 two scenarios are supported: PSA UPF onboard satellite as described in 5.43.3.2 and PSA UPF on the ground as described on 5.43.3.3. For the case of PSA UPF onboard the PDU session is established for UE-to-UE communication. For case of PSA UPF on the ground, the UL CL/BP and local PSA UPF of the PDU session is dynamically inserted on satellite when UE-SAT-UE communication is detected and trigged. In this case the UE connecting satellite can also commuicate with the UE connecting ground via PSA UPF on the ground.

5G VN already supports both IP PDU Session type and Ethernet PDU Session type. For Work Task#2.2, both IP PDU Session type and Ethernet PDU Session type can also be supported by reusing existing 5G VN mechanisms.

Based on above analysis, following principles can be concluded for Work Task#2.2:

Principle 1: The existing R18 mechanism (Local switch for UE-to-UE communications via UPF deployed on GEO satellite) can be reused for Work Task#2.2 to trigger, select UPF(s) onboard satellite(s) and establish UE-SAT-UE communication via UPF(s) only onboard NGSO satellite(s).

Principle 2: A single SMF or a single SMF set is supported for Work Task#2.2, i.e. both UEs are served by the same SMF or same SMF set. Multiple SMF sets deployment support is optional.

Principle 3: The UEs are in the same 5G VN group.

Principle 4: N6 and N19 tunnel can be used between UPFs on-board different satellites to locally switch UE to UE traffic.

Principle 5: Both IP PDU Session type and Ethernet PDU Session type are supported.

Since Work Task#2.2 applies for UPF onboard NGSO satellite, then service/session continuity due to change of UPF onboard satellite caused by UE or satellite mobility needs to be supported. It includes the case where UE-SAT-UE communication continues on subsequent satellites (e.g. through ISL(s) between target satellites), and the case UE-SAT-UE communication falls back to the ground.. These cases correspond to the ones already studied in R19 for UE-SAT-UE IMS call.

Existing specification for 5G VN supports both unicast communication between two UEs and multicast communication from one UE to many other UEs, as described in 5.8.2.13 of TS 23.501. However, the mobility with service/session continuity is only supported for unicast not multicast. To support multicast, the complexity can be anticipated considering the frequent change of connections among multiple UPFs onboard multiple satellites, due to UE or serving satellite mobility. There maybe not enough time for this study, since the TU assigned is only 0.5. Accordingly, it is proposed to only support unicast communication for Work Task#2.2.

There are three Session and Service Continuity modes in 3GPP and all the three SSC modes are support in 5G VN, as described from 5.29.3 of 23.501. For the scenario of PSA UPF onboard satellite, it can use SSC mode 2/3 since the serving satellite always changes. For the scenario of PSA UPF on the ground, it can use all three SSC modes. To simplify the analysis, we can assume the scenario of PSA UPF on the ground uses SSC mode 1. Below is the comparation of different SSC modes for 5G VN deployed on satellite to support satellite change:

|  | SSC mode 1 (with PSA UPF on ground) | SSC mode 2 (with PSA UPF on satellite) | SSC mode 3 (with PSA UPF on satellite) |
| --- | --- | --- | --- |
| Packet loss during serving satellite change? | NO | Yes 1. Frequently due to frequent satallte change 2. More study can be estimated to support no packet loss | NO |
| Commecial UE support? | Yes | No 1. Optionally supported (described in 5.6.9.2 of 23.501) | No Same as SSC mode 2 |
| AF/service layer interaction needed? | No | Yes for IP No for Ethernet 1.Interaction between UEs/AF to sync the IP change | Yes Same as SSC mode 2 |
| IP type support? | Yes | Yes | Yes |
| Ethernet type support? | Yes | Yes | No 1. As described in 5.6.9.2.3 of 23.501 |

It can be observed that SSC mode 2/3 with PSA UPF on satellite have many issues, then it is proposed to support SSC mode 1 with PSA UPF on ground as baseline for Work Task#2.2.

There are requirements to deploy PSA UPF on satellite. It works for the scenario that all the 5G VN group members are accessing NTN (no ground DN connectivity while UE-Sat-UE with PSA UPF).

Based on above analysis, following can be concluded for Work Task#2.2:

Principle 6: PSA UPF on ground and UL CL/BP/local PSA UPF onboard satellite is supported as baseline.

Principle 7: PSA UPF on satellite is optionally supported, by using SSC mode 2. It is assumed that all the 5G VN group members are accessing NTN (no DN connectivity while UE-Sat-UE with PSA UPF). Enhancement to reduce packet loss is out of 3GPP scope.

Principle 8: Only unicast UE-SAT-UE communication is supported.

2 Proposal

It is proposed to agree the interim conclusion and Key issues for TR 23.700-19 on FS_5GSAT_Ph4_ARC as follows(By using the new TR_Skeleton template)

* First change (all New)*

7	Interim Conclusions

7.Y	Agreed principles for KI#Y of UE-SAT-UE communication via UPF only onboard satellite for non-IMS services

To supports KI#Yof UE-SAT-UE communication via UPF only onboard satellite for non-IMS services, the following principles are concluded:

Principle 1: The existing R18 mechanism (Local switch for UE-to-UE communications via UPF deployed on GEO satellite) can be reused for Work Task#2.2 to trigger, select UPF(s) onboard satellite(s) and establish UE-SAT-UE communication via UPF(s) only onboard NGSO satellite(s).

Principle 2: A single SMF or a single SMF set is supported for for Work Task#2.2, i.e. both UEs are served by the same SMF or same SMF set. The usage of multiple SMF sets for same VN group is optional if there exists requirement to extend UE-SAT-UE communication in a wide area.

Principle 3: The UEs are in the same 5G VN group.

Principle 4: N6 and N19 tunnel can be used between UPFs on-board different satellites to locally switch UE to UE traffic.

Principle 5: Both IP PDU Session type and Ethernet PDU Session type are supported.

Principle 6: PSA UPF on ground and UL CL/BP/local PSA UPF onboard satellite is supported as baseline.

Principle 7: PSA UPF on satellite is optionally supported, by using SSC mode 2. It is assumed that all the 5G VN group members are accessing NTN (no DN connectivity while UE-Sat-UE with PSA UPF). Enhancement to reduce packet loss is out of 3GPP scope.

Principle 8: Only unicast UE-SAT-UE communication is supported.

* Second change (all New)*

7.2	Topics for further consideration

7.2.Y	Topics for further consideration for KI#Y of UE-SAT-UE communication via UPF only onboard satellite for non-IMS services

1. How to support service/session continuity due to satellite change after UE-SAT-UE communication is established, including UE-SAT-UE communication continues on satellite or falls back to ground.

* Third change (all New)*

5.Y	Key Issue #Y: UE-SAT-UE communication via UPF only onboard satellite for non-IMS services

This key issue studies:

-	How to support UE-SAT-UE communication via UPF only onboard satellite for non-IMS services (e.g. 5G-LAN services with IP and Ethernet PDU Session type).

NOTE 1:	Whether existing mechanism can be resued will be considered in this KI.

NOTE 2:	Any UE impact will not be considered in this KI

* End of change *