# 3GPP SA WG2 Meeting #169	S2-2504675

# May 19 - 23, 2025, Fukuoka, Japan

# Title:	Reply LS on clarification of SCP data collection from CT4

# Response to:	LS out Clarification on SCP data collection (S2-2504509_C4-251477)

# Release:	Release 19

# Work Item:	AIML_CN

Source:	SA2

To:	CT4

Cc:

Contact Person:

Name:	Uri Baniel

E-mail Address:	uri DOT baniel @ oracle DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: None.

1. Overall Description:

SA2 thanks CT4 for the LS on clarification of SCP data collection. SA2 would like to provide the following reply.

Question 1: Regarding the "UE related Context Data collection" specified in Table 6.22.2-1 of TS 23.288:

Requiring the SCP to provide information with UE granularity means that the NFs in the network would be required to be configured to use the custom header 3gpp-Sbi-Correlation-Info which is optional to support and may be subject to security/regulatory requirements (see TS 29.500). Furthermore, different NFs may use different UE identities, e.g., SUPI, GPSI, etc, for the same UE within this header Also the NFs of earlier releases that do not support this header do not provide UE information to the SCP, which would impact the accuracy of the statistics.

Considering these limitations, CT4 would like to ask SA2 to confirm whether the information with UE granularity is required to be obtained by the SCP for the event exposure purpose.

SA2 reply: SA2 agrees with CT4’s consideration and analysis, thus recommend an operator who intends to use Signaling Storm analytic, to configure NFs in their NW to provide 3gpp-Sbi-Correlation-Info header with the UE ID that is received by the AMF from the UE in the NAS signaling (for most of the signaling this would be SUPI). This can assist NFs that are challenged with retrieving UE context in update or delete type of requests (e.g., Nsmf_PDUSession_ReleaseSMContext). In addition, NF Service related signaling is moved out from table 6.22.2-1 into a new table. Consequently, SCP and NF entries (other than AMF and SMF entries) are deleted in this table.

Question 2: According to TS 23.288 clause 6.22.2, the SCP is required to distinguish between different types of communications and signalling and provide information, for example, to collect the data of "Number of redundant signalling of NF" and "A posterior Request type of NF (0..max)" in Table 6.22.2-1, and "SCP Signalling statistics" in Table 6.22.2-3. Currently there is no mechanism to enable SCP to differentiate between different types of communication SA2 refers to, e.g., whether the signalling is redundant or an "A posterior Request".

Therefore, CT4 would like to ask SA2 to consider reverting the requirements on "A posterior Request type of NF (0..max)" and "Number of redundant signalling of NF", unless SA2 could provide clarification on how these requirements can be implemented.

SA2 reply: SA2 agrees with CT4’s suggestion and decides to remove the SCP from the source of “Number of redundant signalling of NF” and “A posterior Request type of NF” specified in Table 6.22.2-1 of TS 23.228, i.e., remove NOTE 3 from the description of the two items.

Question 3: CT4 would also appreciate SA2 to clarify what "different types of signalling" corresponds to for "SCP Signalling statistics".

SA2 reply: The signalling types of “SCP Signalling statistics” include Message Type (Request/Response) and Service Type.

Question 4: "Heart-beat related information", "NF load status information", "Capacity and priority information" which are listed in TS 23.288 Table 6.22.2-2, may be provided by NRF. If required, SCP may get this information from NRF. Therefore, CT4 would like to ask for clarification on why the SCP is also required to provide this information.

SA2 reply: SCP seats on the path of these NF-NRF messages, and so is able to detect messages such as Heart-beat, Capacity and priority information etc. Moreover, for some of these messages SCP may have a better assessment than the NRF itself. For example, if an NF is very loaded it may not be able to handle NRF traffic at all, while at the same time it may demonstrating loaded behaviour on other signalling interfaces, e.g not responding to requests from other NFs or responding late.

2. Actions:

To CT4 group

ACTION:

SA2 asks CT4 to take SA2’s feedback into account for future work.

3. Dates of next TSG SA WG 2 meetings:

TSG-SA2 Meeting #170		August 25 – August 29, 2025		Goteborg, SE

TSG-SA2 Meeting #171		October 13 – October 17, 2025		TBD, CN