3GPP TSG-SA2 Meeting #169	S2-2504782

# 19 - 23 May, 2025, Fukuoka, Japan		(revision of )

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1469 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment for VFL service operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The VFL related procedures are almost complete and stable, however, the service operation descriptions are not align with the VFL procedures. Thus, this CR is proposed to update the parameters of service operation descriptions to align with the VFL procedures. For the VFL training notification from VFL client to the VFL server, the delta list, VFL training iteration number and Local ML model accuracy moitoring information are missing. For the VFLTraining_Request service, which is used for preparation, the input parameters and the output parameters are missing. For the Nnef_VFLNFDiscovery_NwdafDiscovery service, some optional parameters are missing. And some Editor’s Note can be removed since the procedures are complete. And as defined in clause 6.2H, it is VFL training instead of VFL ML Model training. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align the parameters of service operations descriptions with the parameters in VFL procedures, including VFL training notify, preparation (VFL training request) and VFLNFDiscovery, etc. Remove some ENs. Clarify the name of VFL training. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The service operations descriptions are misalign with the VFL procedures, and it is unclear for implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.12.3, 7.12.4, 7.12.5, 7.13.1, 11.2.3, 11.2.4, 11.2.5, 11.4.1, 11.5.3, 11.5.4, 12.2.3, 12.2.4, 12.2.5, 12.4.1, 12.4.2, 12.5.1, 12.6.3, 12.6.4. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 7.12.3	Nnwdaf_VFLTraining_Unsubscribe service operation

Service operation name: Nnwdaf_VFLTraining_Unsubscribe

Description: Terminate NWDAF VFL training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code.

### 7.12.4	Nnwdaf_VFLTraining_Notify service operation

Service operation name: Nnwdaf_VFLTraining_Notify

Description: NWDAF notifies the consumer of client intermediate training result of the local ML model.

Inputs, Required:

-		Notification Correlation Information.

Inputs, Optional:

-		Client intermediate training result.

-	Delta list, indicate which samples will not be part of the rest of training procedure.

-	Local ML model accuracy monitoring information.

-	VFL training iteration number

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### 7.12.5	Nnwdaf_VFLTraining_Request service operation

Service operation name: Nnwdaf_VFLTraining_Request

Description: In preparation of VFL training, requests NWDAF VFL client to check if it can support requirements for VFL.

Inputs, Required:

-	Vertical Federated Learning preparation request indication.

-	Analytics ID.

-	Suggested VFL Interoperability Information.

Inputs, Optional:

-	Suggested list of sample IDs.

-	Suggested Feature ID(s).

-	Time window of the data samples.

-	Required minimum sample size.

Outputs Required: When the request is accepted: indicate of accepting the ML Model training requirements.

When the request is not accepted, an error response with cause code

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional:

-	Supported Feature ID.

-	VFL Interoperability information that the VFL Client supports.

-	The list of sample IDs accepted within the sample IDs suggested by the VFL Server.

-	The list of supported sample IDs.

# * * * * Next change * * * *

### 7.13.1	General

Service Description: This service is provided by an NWDAF acting as VFL client and enables an VFL server as consumer to request or subscribe/unsubscribe for a VFL inference.

When the subscription is accepted by the NWDAF containing AnLF, the consumer receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

# * * * * Next change * * * *

### 11.2.3	Naf_VFLTraining_Unsubscribe service operation

Service operation name: Naf_VFLTraining_Unsubscribe

Description: Terminate AF VFL training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code

### 11.2.4	Naf_VFLTraining_Notify service operation

Service operation name: Naf_VFLTraining_Notify

Description: AF notifies the consumer of client intermediate training result of the local ML mode (for VFL).

Inputs, Required:

-	Notification Correlation Information.

Inputs, Optional:

-	Client intermediate training result.

-	Delta list, indicate which samples will not be part of the rest of training procedure.

-	Local ML model accuracy monitoring information.

-	VFL training iteration number.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### 11.2.5	Naf_VFLTraining_Request service operation

Service operation name: Naf_VFLTraining_Request

Description: In preparation of VFL training, requests AF VFL client to check if it can support requirements for VFL.

Inputs, Required:

-	Analytics ID.

-	Vertical Federated Learning preparation request indication.

-	Suggested VFL Interoperability Information.

Inputs, Optional:

-	Suggested list of sample IDs.

-	Suggested feature ID(s).

-	Time window of the data samples.

-	Required minimum sample size.

Outputs Required: When the request is accepted: indicate of accepting the ML Model training requirements. .

When the request is not accepted, an error response with cause code (e.g. AF does not meet the VFL training requirements).

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional:

-	Suggested Feature ID.

-	VFL Interoperability information that the VFL Client supports.

-	The list of sample IDs accepted within the sample IDs suggested by the VFL Server.

-	The list of supported sample IDs.

# * * * * Next change * * * *

### 11.4.1	General

Service Description: This service is provided by AF acting as VFL server and enables an NWDAF or an NEF acting on its behalf as consumer to request or subscribe/unsubscribe for a VFL inference.

# * * * * Next change * * * *

### 11.5.3	Naf_Training_Unsubscribe service operation

Service operation name: Naf_Training_Unsubscribe

Description: Terminate AF VFLtraining.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code.

### 11.5.4	Naf_Training_Notify service operation

Service operation name: Naf_Training_Notify

Description: AF notifies the consumer of training progress

Inputs, Required:

-	Notification Correlation Information.

Inputs, Optional:

-	Training is ongoing.

-	Training is complete.

-	VFL training completion time.

-	VFL correlation ID.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * Next change * * * *

### 12.2.3	Nnef_VFLTraining_Unsubscribe service operation

Service operation name: Nnef_VFLTraining_Unsubscribe

Description: Terminate AF VFL training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code.

### 12.2.4	Nnef_VFLTraining_Notify service operation

Service operation name: Nnef_VFLTraining_Notify

Description: NEF notifies the consumer of client intermediate training result of the local ML mode.

Inputs, Required:

-	Notification Correlation Information.

Inputs, Optional:

-	Client intermediate training result.

-	Delta list, indicate which samples will not be part of the rest of training procedure.

-	Local ML model accuracy monitoring information.

-	VFL training iteration number.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### 12.2.5	Nnef_VFLTraining_Request service operation

Service operation name: Nnef_VFLTraining_Request

Description: In preparation of VFL training, requests NEF to check at untrusted AF or NWDAF acting as VFL client if it can support requirements for VFL.

Inputs, Required:

-	Vertical Federated Learning preparation request indication.

-	Analytics ID.

-	For NWDAF as VFL client, external NWDAF ID.

-	Suggested VFL Interoperability Information.

Inputs, Optional:

-	Suggested list of sample IDs.

-	Suggested feature ID(s).

-	Time window of the data samples.

-	Required minimum sample size.

Outputs Required: When the request is accepted: indicate of accepting the ML Model training requirements. When the request is not accepted, an error response with cause code (e.g. NWDAF does not meet the VFL training requirements.

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional:

-	Supported Feature ID.

-	VFL Interoperability information that the VFL Client supports.

-	The list of sample IDs accepted within the sample IDs suggested by the VFL Server.

-	The list of supported sample IDs.

# * * * * Next change * * * *

### 12.4.1	General

Service Description: This service is provided by an NEF towards an untrusted AF acting as VFL server to enable the AF to interact with NWDAFs acting as VFL client. It enables the AF to detect NWDAFs as VFL clients as described in in clause 6.2H.2.1.

### 12.4.2	Nnef_VFLNFDiscovery_NwdafDiscovery service operation

Service operation name: Nnef_VFLNFDiscovery_NwdafDiscovery

Description: The consumer requests the NEF to discover NWDAF(s) acting as VFL client.

Inputs, Required:

-	Analytics ID.

-	required NF type (i.e. NWDAF type).

-	VFL capability type (i.e. VFL client).

-	VFL interoperability indicator(s).

Inputs, Optional:

-	Required feature IDs.

-	Time Period of Interest.

-	Optional Service Area.

Outputs, Required: If the request is accepted, external NWDAF ID(s). When the request is not accepted, an error response.

Outputs, Optional:

-	Information how they relate to corresponding selection criteria provided by the AF.

-	VFL interoperability indicator(s).

-	Time Period of Interest.

-	Feature ID(s).

-	Service Area.

# * * * * Next change * * * *

### 12.5.1	General

Service Description: This service is provided by an NEF on behalf of an AF acting as VFL server and enables an NWDAF as consumer to request or subscribe/unsubscribe for a VFL inference.

# * * * * Next change * * * *

### 12.6.3	Nnef_Training_Unsubscribe service operation

Service operation name: Nnef_Training_Unsubscribe

Description: Terminate AF VFL training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code.

### 12.6.4	Nnef_Training_Notify service operation

Service operation name: Nnef_Training_Notify

Description: AF notifies the consumer of training progress.

Inputs, Required:

-	Notification Correlation Information.

Inputs, Optional:

-	Training is ongoing.

-	Training is complete.

-	VFL training completion time.

-	VFL correlation ID.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * End of changes * * * *