3GPP TSG-SA WG2 Meeting #169	S2-2504818

Fukuoka, Japan, May 19–23, 2025

Source:	Apple

Title:	KI#2, Sol#6: Addressing ENs on policy control

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_ph2/Rel-20

Abstract of the contribution: Addressing ENs in solution#6 on policy control.

# 1. Discussion

TR 23.700-67 has captured solution #6 describing policy control triggered by EIF notifications. This pCR addresses two Editor’s Notes of this solution.

The first EN in the solution reads

Editor's note: It is FFS how to trigger the PCF to subscribe to notifications from the EIF.

The Rel-19 work item has defined an Energy Saving Indicator in UE subscription data in UDM. AMF receives the Energy Saving Indicator in the UE subscription data and provides this information to PCF. This can be the trigger for PCF to subscribe to EIF notifications at UE level for influencing AM policies. Similarly an SMF that is establishing SM Policy association for a PDU Session can include the Energy Saving Indicator it receives with UE subscription data if it wants PCF to subscribe for EIF notifications on a PDU Session level.

Proposal 1: PCF subscribes to EIF Notifications if requested by AMF/SMF during policy association.

The solution described examples of AM policies that can be modified due to the energy consumption information from EIF. One of the examples for AM policy adjustment is to use notifications from EIF on energy consumption for UE above certain threshold as a trigger to modify RFSP to update the RAT priority of the UE to E-UTRAN.

There is an associated EN

Editor's note: It is FFS whether the PCF takes notifications from the EIF into account to decide the RFSP Index value in AM policy.

The RFSP index value is used by NG-RAN for applying specific RRM strategies according to locally defined configuration. Some examples stated in 3GPP TS 23.5015.3.4.3.1 includes deriving cell reselection priorities and for deciding redirection of active mode UEs across different frequency layers/RATs.

Modifying RFSP index influenced by network energy consumption needs further thought.  RFSP index should be a guidance for static long term RRM strategies.The actions are typically affecting  UE idle mode behaviour. Energy consumption going above threshold may not be a scneario that needs RAT/Frequency (De-)prioritization.  Lowering the access priorities of a UE for a perceived higher than normal energy consumption need to be backed with highly accurate energy consumption measurement at UE granularity as this may cause bad user experience. Mitigation actions triggered by a higher than threshold energy consumptoin may be better handled in policies that act on the UE in connected mode that are not related to RFSP index value.

RFSP index value modification and redirecting UE to a different RAT may also expose it to security issues and bidding down attacks (UE registering on E-UTRAN can lead to scenarios where IMSI is sent over air interface, further redirection to less secure RATs ,e.g, 2G) . These security risks should be avoided. It is proposed to remove RFSP index value modification from a possible AM policy control action triggered from an EIF notification.

Proposal 2: EIF notifications do not influence RFSP index value decisions.

# 2. Text Proposal

It is proposed to agree the following updates to 3GPP TR 23.700-67 0.1.0.

# * * * * First change * * * *

## 6.6	Solution #6: Policy control based on notifications from the EIF

### 6.6.0	High-level solution principles

This solution proposes that the PCF takes the energy related information in the notifications from the EIF into account for policy control.

NOTE:	It is required that the energy related information in the notifications from the EIF is accurate enough for the PCF to take into account for policy decision.

### 6.6.1	Description

This solution addresses Key Issue #2.

If the Energy Saving Indicator is included in the UE subscription data, AMF forwards the Energy Saving indicator to the PCF during AM policy association. Similarly if the UE subscription data includes Energy Saving indicator, SMF forwards the Energy Saving indicator to the PCF during an SM policy association establishment.

The PCF can subscribe to notifications from the EIF, e.g. Energy Consumption information at UE, S-NSSAI, PDU Session and Service Data Flow (e.g. per UE per application) granularity as described in clause 5.51 of TS 23.501 [2].

The PCF may determine and update the following policies based on energy related subscription data (i.e. Energy Saving Indicator), notifications from the EIF and operator policy:

-	Access and mobility related policy, e.g. UE-AMBR, RFSP Index.

-	Session management related policy, e.g. Session-AMBR, MA PDU Session Control information.

Examples for AM policy control taking into account the notifications from the EIF are as follows:

-	If notification(s) from the EIF indicates energy consumption for the UE is high based on operator policy (e.g. higher than a predefined UE average energy consumption threshold per PLMN or per network slice), the PCF may modify the UE-AMBR value to be lower than the subscribed UE-AMBR or the UE-AMBR in use for the UE based on operator policy, to reduce the energy consumption for transmitting the Non-GBR QoS Flows of the UE.

Examples for SM policy control taking into account the notifications from the EIF are as follows:

-	If notification(s) from the EIF indicates energy consumption for the UE or for a PDU Session of the UE is high based on operator policy (e.g. higher than a predefined UE average or PDU Session average energy consumption threshold per PLMN or per network slice), the PCF may modify the Session-AMBR value to be lower than the subscribed Session-AMBR or the Session-AMBR in use for the PDU Session based on operator policy, to reduce the energy consumption for transmitting the Non-GBR QoS Flows of the PDU Session.

-	If notification(s) from the EIF indicates energy consumption for the UE over 3GPP access is high based on operator policy (e.g. higher than a predefined UE average energy consumption threshold per PLMN or per network slice), the PCF may modify the MA PDU Session Control information (e.g. Steering Mode and associated threshold values) in the PCC rules to switch some or all the Non-GBR QoS Flows of the MA PDU Session from 3GPP access to non-3GPP access as described in clause 6.1.3.20 of TS 23.503 [4].

Editor's note:	It is FFS whether the PCF takes notifications from the EIF into account to decide the MA PDU Session Control information in the PCC rules.

In addition, based on notifications from the EIF, the PCF may provision and update validity conditions (e.g. validity time period, validity area) in the policies, for network energy saving within a specific time period and/or specific area.

### 6.6.2	Procedures

The existing policy control procedures as specified in TS 23.502 [3] and TS 23.503 [4] apply.

### 6.6.3	Impacts on services, entities and interfaces

PCF:

-	Performs policy control taking into account of notifications from the EIF if requested by AMF and/or SMF during Policy association procedures.

EIF:

-	Sends energy related notifications to the PCF.

# * * * * End of changes * * * *