3GPP TSG-SA2 Meeting #169	S2-25xxyyy

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | Draft | rev | - | Current version: | 19.x.y |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ECN marking for L4S in the UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20 |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | 20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | ECN marking for L4S is enabled per QoS Flow in 5G. In a tethered mode of operation UE may be independently scheduling uplink traffic originating from devices behind the UE. The knowledge of whether ECN marking for L4S is enabled for a QoS Flow in the 5GS is useful for UE to understand whether L4S techniques can be applied. The UE would also benefit from the knowledge of whether a network node is configured to perform ECN marking for L4S. When serving PSA UPF or NG-RAN is changed, e.g., due to inter-NG-RAN handover or PSA UPF relocation, and if neither target NG-RAN nor PSA UPF supports ECN marking for L4S, UE can perform ECN marking also considering Uu link congestion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Included ECN marking for L4S in the UE for uplink direction in the overall description. New sub-clause for detailed description of support of ECN marking for L4S in the UE. UE is informed whether a QoS Flow is enabled for ECN marking for L4S in the uplink. UE is also informed of whether network is performing congestion marking. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature remains sub-optimal since UE may not be able to make use of L4S techniques for indicating congestion information to perform rate adaptation in upper layers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.3.1, 5.37.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 23.502.. CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 5.37.3	Support of ECN marking for L4S to expose the congestion information

#### 5.37.3.1	General

L4S (Low Latency, Low Loss and Scalable Throughput) is described in IETF RFC 9330 [159], IETF RFC 9331 [160] and IETF RFC 9332 [161]. It exposes congestion information by marking ECN bits in the IP header of the user IP packets between the UE and the application server to trigger application layer rate adaptation.

In 5G System, ECN marking for L4S may be supported. ECN marking for L4S is enabled on a per QoS Flow basis in the uplink and/or downlink direction and may be used for GBR and non-GBR QoS Flows. In the case of 3GPP access, ECN marking for the L4S in the IP header is supported in either the NG-RAN (see clause 5.37.3.2 and TS 38.300 [27]), or in the PSA UPF (see clause 5.37.3.3) and/ or in the UE for uplink direction (see clause 5.37.3.x). In the case of untrusted/trusted non-3GPP access, ECN marking for L4S in the IP header is supported in the N3IWF/TNGF (see clause 5.37.3.4, clause 6.2.9 and clause 6.2.9A) and/ or in the UE for uplink direction (see clause 5.37.3.x).

NOTE 1:	Based on operator's network configuration and policies, SMF decides whether NG-RAN or PSA UPF based ECN marking for L4S is used.

In the case of ECN marking for L4S by PSA UPF, the NG-RAN is instructed to perform congestion information monitoring and report to the PSA UPF the congestion information (i.e. a percentage of packets that UPF uses for ECN marking for L4S) of the QoS Flow on UL and/or DL directions via GTP-U header extension to PSA UPF and accordingly, the PSA UPF may mark the UL and/or DL direction packets of the QoS Flow.

NOTE 2:	As for any QoS Flow, QoS rules in the UE and PDRs in the PSA UPF control which packets are bound to the L4S enabled QoS flow. The Packet Filter Set in the QoS rule or PDR can use packet filter(s) in clause 5.7.6.2 (e.g. match packets with ECT(1) or CE (See RFC 9331 [160]) together with IP 5 tuple) to steer traffic to an L4S enabled QoS Flow.

NOTE 3:	A QoS Flow may be enabled with ECN marking for L4S requirement e.g. statically when a PDU session is established based on configuration in SMF or PCF, or dynamically based on detection of the L4S traffic (e.g. match packets with ECT(1) or CE (See RFC 9331 [160]) together with IP 5 tuple) in the IP header whereby SMF or PCF triggers a setup of a QoS Flow enabled for L4S, or by requests by an AF.

NOTE 4:	To support this functionality, the UE needs to support L4S feedback as described in IETF RFC 9330 [159], which is not in the scope of 3GPP.

When serving PSA UPF or NG-RAN is changed e.g. due to inter-NG-RAN handover or PSA UPF relocation, target NG-RAN and target PSA UPF, if supported, should continue to perform ECN marking for L4S for the QoS Flow. However, if not available (i.e. ECN marking for L4S is not supported in both, target NG-RAN and target PSA UPF), AF should be notified that ECN marking for L4S can no longer be performed if ECN marking for L4S had been enabled for the QoS Flow based on AF request. When ECN marking for L4S is supported again either in target NG-RAN or in target PSA UPF, AF should be notified that ECN marking for L4S can be performed again if ECN marking for L4S had been enabled for the QoS Flow based on AF request.

Next Change

#### 5.37.3.X	Support of ECN marking for L4S in the UE

ECN marking for L4S may be supported in the UE.

Uplink traffic can be subject to congestion in the UE. In tethered mode of operation, UE may be scheduling traffic generated from multiple tethered devices in the uplink direction. SMF may indicate to the UE whether ECN marking for L4S has been enabled for  a QoS Flow in the uplink. UE may perform congestion marking in uplink direction on the QoS Flow.

If ECN marking for L4S is enabled for a QoS Flow in uplink, SMF also indicates whether a network node (NG-RAN or PSA UPF) is configured to perform the marking. In case of a change of NG-RAN or PSA-UPF, if ECN marking for L4S is not supported in both target NG-RAN and target PSA UPF, the SMF indicates to the UE that none of the network nodes are performing ECN marking for L4S on the QoS Flow. In this case UE may perform congestion marking also considering the congestion in the Uu interface.

The criteria based on which UE decides to mark ECN bits for L4S are UE implementation specific.

End of changes