3GPP TSG SA WG2 Meeting #169	S2-2504819

Fukuoka, Japan,  19 - 23 May 2025

# Source:	Apple

# Title:	New WID: ECN marking for L4S in the UE

# Document for:	Information

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: ECN Marking for L4S in the UE

Acronym: TEI20_UE_L4S

Unique identifier:

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | x |  | X |  | x |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

L4S as described in IETF RFC 9330, RFC 9331 and RFC 9332 is supported in 5GS from Rel-18. This feature exposes congestion information by marking ECN bits in the header of the IP packets between UE and the application server to help with application layer/transport layer rate adaptation.

ECN marking for L4S can be enabled on a QoS Flow basis for the uplink and/or downlink direction. For 3GPP access, ECN marking for L4S in the IP header is supported either in the NG-RAN or PSA UPF. For non-3GPP access, ECN marking for L4S is supported in the N3IWF/TNGF in either direction. For Wireline access, ECN marking for L4S is also supported in W-AGF and 5G-RG (uplink only).

A UE may be scheduling uplink traffic from independent sources over Uu interface (e.g., in tethered mode of operation). In such cases, UE may have to rely on L4S techniques to provide feedback to the source applications for rate adaptation. The knowledge of whether ECN marking for L4S is enabled for a QoS Flow in the 5GS is useful for UE to understand whether L4S techniques can be applied. UE may then perform ECN marking on the uplink direction based on the congestion it has observed on the uplink direction.

The UE would also benefit from the knowledge of whether a network node is configured to perform ECN marking for L4S. When serving PSA UPF or NG-RAN is changed, e.g., due to inter-NG-RAN handover or PSA UPF relocation, and if neither target NG-RAN nor PSA UPF supports ECN marking for L4S, UE can perform ECN marking also considering Uu link congestion.

# 4	Objective

This new WID aims to enable the following enhancements in 5GS.

WT 1: Enhance the scope of ECN marking for L4S in the 5GS to enable UE to perform ECN marking in the IP headers in the uplink direction.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Adding general description about ECN marking for L4S by the UE in the uplink direction | - |  |
| TS 23.502 | Update SM signalling for informing UE about L4S support per QoS Flow | - |  |

# 6	Work item Rapporteur(s)

Apple

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

Apple