3GPP TSG SA WG 2 Meeting SA2#169	                                                                  S2-2504868

Fukuoka, Japan, 2025-05-19 – 2025-05-23

Title:	[Draft] LS reply on Extending Charging Support in 5GC

Response to:	LS (S2-2504504, C3-251642) on LS on Extending Charging Support in 5GC

Release:	Rel-19

Work Item:	TEI19, 5GS_Ph1-CT

Source:	Ericsson (will be SA2)

To:	CT3

Cc:	SA5, CT4

Contact person:	Belen Pancorbo

belen.pancorbo@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR 1559 23.503

# 1	Overall description

SA2 has discussed the questions in received LS S2-2504504/C3-251642 and have arrived at the following conclusions:

Q1: Is the Primary Charging Address still required as a mandatory parameter to be stored in the UDR and provided to the PCF, SMF and AMF when the CHF Group ID is available?

Answer: When the CHF group ID is used for CHF selection, the Primary Charging Address is not required as part of the subscription information and thus can be omitted as part of the provided Charging Information to the PCF, SMF and AMF. The Policy control subscription information in TS 23.503 is clarified as described in the attached CR.

Q2: Is the CHF Set ID and the CHF group ID mutually exclusive or can they still be both stored in the UDR and provided to the PCF, SMF and AMF together?

Answer: CHF Set ID and CHF group ID are mutually exclusive and only one of them shall be provided to PCF/SMF/AMF.

# 2	Actions

To CT3

ACTION: Based on the answers above, SA2 asks CT3 to take any needed action.

# 3	Dates of next TSG SA WG 2 meetings

SA2#170		2025-08-25 – 2025-08-29		Gothenburg, Sweden

SA2#171		2025-10-13 – 2025-10-17		TBD, China