3GPP TSG-SA2 Meeting #169 	 S2-2504873

# Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0708 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LMF checking LCS privacy profile before collecting input data and ground truth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, AT&T, Lenovo, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 09-05-2025 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The LMF collects both ground truth data and positioning measurements for a SUPI, both are considered UE positioning information, as such in the scope of 23.273 the LCS privacy profile should be considered instead of the user consent. As such, the LMF is considered as a LCS client that needs to be authorized to collect any of these two positioning data by checking the LCS privacy profile. Consequently, the user consent in UDM is considered not applicable (as such not needed) for the case where a LCS client asks for UE positioning information. This is because the LCS privacy profile addresses user consent in relation with UE positioning, rendering further consent procedures redundant. The LMF checks the LCS privacy profile, although the option where the LMF is a LCS client that contacts the GMLC may also be a valid option, if prefered by the meeting (note that updates are needed to 6.22.3 if it is decided that the GMLC checks the LCS privacay profile instead of LMF). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove user consent checks at the LMF and replace them for the LMF checking the LCS privacy profile and related Editor´s Notes for the user consent in relation with training and performance monitoring of a ML Model. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Open issues remain unsolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.0, 5.18.2, 6.22.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************** 1st change **************

### 5.18.0	General

The LMF may calculate the UE location and estimate the achieved accuracy by using LMF-based AI/ML Positioning. When receiving the request from AMF for determining a UE location, the LMF selects an appropriate positioning method as described in clause 5.2 to determine the result of the positioning. The result of the positioning may be calculated by using LMF-based AI/ML Positioning ML model supported by LMF. The LMF collects input data from UE or NG-RAN for the LMF-based AI/ML Positioning to perform location calculation and provide the location to the consumer.

NOTE 1:	Whether to select LMF-based AI/ML Positioning for location result calculation is determined by LMF.

NOTE 2:	The specific measurement data collected by LMF from UE and NG-RAN for LMF-based AI/ML Positioning are in the scope of RAN specifications and not in the scope of this specification.

Editor's note:	What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs.

The ML model that is used for LMF-based AI/ML Positioning may be trained by LMF. The trigger for data collection required for model training in LMF is up to implementation. LMF collects input data from UE for ML model training as described in clause 6.22.2. The LMF collects input data from NG-RAN for ML model training as described in clause 6.22.3.

The LMF may also request a trained ML model for LMF-based AI/ML Positioning from NWDAF containing MTLF as described in clause 6.22.5. The LMF discovers a suitable NWDAF containing MTLF via NRF as described in clause 5.2 of TS 23.288 [37] with the following considerations:

-	The LMF may provide the positioning case information(e.g. UE assisted/LMF-based AI/ML Positioning case, NG RAN node assisted LMF-based AI/ML Positioning case, or both).

-	The LMF provides an Area of Interest, and may include the ML Model Interoperability indicator to discover an NWDAF that can provide an AI/ML Model.

-	The LMF may request a NWDAF with ML Model accuracy checking capability to report when the model is degraded (i.e. for ML model performance monitoring).

NOTE 3:	Other NWDAF discovery parameters listed in clause 5.2 of TS 23.288 [37] such as Analytics ID, FL capability type and related time period, S-NSSAI or any roaming capabilities are not included by LMF.

Editor's note:	The types of LMF-based AI/ML positioning model (e.g., per positioning case) is FFS, which needs to be coordinated with RAN WGs.

The LMF requests the NWDAF containing MTLF to provide an ML Model for LMF-based AI/ML Positioning as described in clause 6.2A of TS 23.288 [37] with the following considerations:

-	The LMF provides the following input parameters in the Nnwdaf_MLModelProvision_Subscribe or Nnwdaf_MLModelInfo_Request:

-	LMF-based AI/ML positioning indication.

-	Optionally, Vendor ID, ML Model Filter Information (e.g. Area of Interest), Positioning case information, Target of ML Model Reporting, ML Model Target Period, Time when model is needed, Inference Input Data information and ML Model Monitoring Information.

-	If vendor specific information is required , then the ML Model Interoperability Information is included.

-	If the LMF supports multiple AI/ML Models, indication of support for multiple ML Models, optionally with Number of ML Models and Accuracy level(s) of Interest.

The NWDAF containing MTLF collects input data to perform the ML model training as described in clause 6.22.4, and the NWDAF containing MTLF performs ML model provision to LMF as described in clause 6.2A of TS 23.288 [37] with the following considerations:

-	The NWDAF containing MTLF provides the ML Model identifier and ML Model Information for the ML Model for UE Positioning, and optionally, the following parameters:

-	ML Model Filter Information and/or Target of ML Model Reporting, if the ML Model provisioning request includes multiple ML Model Filter Information and/or Target of ML Model Reporting;

-	Indication of whether the ML Model identifier is updated (e.g. retrained ML model).

-	Validity period, Spatial validity, Training Input Data Information, ML Model accuracy Information.

Editor's note:	The collected input data for LMF-based AI/ML Positioning model training will be determined by RAN1. How to collect the input data for LMF-based AI/ML Positioning model training need coordination with RAN WGs.

Once the ML model for LMF-based AI/ML Positioning is trained and available in the LMF, the LMF may use it to perform UE Positioning after receiving a location determination request from AMF.

Editor's note:	Whether user consent for UE positioning calculation is needed apart from the existing GMLC check of the LCS privacy profile is FFS.

For ML model training, performance monitoring and UE positioning calculation using LMF-based AI/ML Positioning, the LMF checks with UDM the LCS privacy profile before collecting UE related data, see clause 6.22.3 and clause 6.22.4.

Either LMF or NWDAF containing MTLF may perform performance monitoring for LMF-based AI/ML Positioning. When the ML model that is used for LMF-based AI/ML Positioning is trained by LMF the LMF monitors the performance of the ML model. When the ML model that is used for LMF-based AI/ML Positioning in LMF is trained by NWDAF containing MTLF, then the NWDAF containing MTLF monitors the performance of the ML model. LMF may determine whether to use the LMF-based AI/ML Positioning to perform location calculation based on the model performance monitoring result. LMF or the NWDAF containing MTLF may also trigger the ML model retraining based on the model performance monitoring result.

************** next change **************

### 5.18.2	AI/ML model performance monitoring for LMF-based AI/ML Positioning

When the AI/ML model that is used for LMF-based AI/ML Positioning is trained by LMF, the LMF may perform performance monitoring for AI/ML model by collecting the measurements data and ground truth data of PRU(s)/UE(s) by using the procedures of data collection for LMF-based AI/ML Positioning as specified in clause 6.22.2 or clause 6.22.3. Before collecting the data from UE(s), LMF needs to check the LCS privacy profile in UDM, however LMF does not need to check LCS privacy profile for data collected from PRU. LMF calculates the location information based on the collected measurement data by using AI/ML model. Based on the calculated location information and ground truth data, LMF evaluates the AI/ML model performance to generate the performance monitoring result. The result may trigger LMF to change the positioning method, e.g. from LMF-based AI/ML Positioning to the legacy positioning or retrain the AI/ML model in LMF.

When the AI/ML model that is used for LMF-based AI/ML Positioning in LMF is trained by NWDAF containing MTLF, then the NWDAF containing MTLF may monitor the performance of the AI/ML model as specified in clause 6.2E.4 of TS 23.288 [37] based on the data collection via the LMF as specified in clause 6.22.4. If the data collection is not granted, the LMF does not perform any data collection procedure for the ML model performance monitoring for this UE.

************** next change **************

### 6.22.3	Data collection to train models for LMF-based AI/ML positioning based on NG RAN measurements

The procedure for data collection from NG-RAN is used to e.g. train the ML Model for LMF-based AI/ML positioning.

Figure 6.22.3-1: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements

1.	The LMF determines that data collection from the NG-RAN is required e.g. to train an ML Model for UE positioning for a number of UEs or to monitor the ML Model performance. The LMF may also initiate the data collection upon the request of an NWDAF containing MTLF as described in step 3 in figure 6.22.4-1.

2.	The LMF may know the SUPIs of the UEs for which to collect location measurement data from the NG-RAN, e.g. when training an ML model using the PRU(s) associated to this LMF. The LMF may optionally invoke an Nnrf_NFDiscovery Request service operation to an NRF to discover other PRU serving LMF(s) which has associated PRUs in the area of interest and send an Nlmf_Location_MeasurementData Request service operation to the selected PRU serving LMFs to collect the measurement data from the NG-RAN for the PRUs and the PRU locations.

If the LMF does not know the SUPIs of the UEs for which to collect location measurement data from the NG-RAN, the LMF tries to get the list of SUPIs from the AMF. Before that the LMF discovers the AMF(s) that serves the area of interest via the NRF using Nnrf_NFDiscoveryRequest.

3.	The LMF subscribes to the list of SUPIs in an area of interest from the AMF(s) using Namf_EventExposure_Subscribe request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest").

The AMF sends Namf_EventExposure_Subscribe response or Namf_EventExposure_Notify (list of SUPIs in the area of interest).

For each SUPI in the area of interest, the following steps are performed.

4.	The LMF checks the LCS privacy profile for the SUPI  with UDM using Nudm_SDM_Get including subscription data type set to "LCS privacy" for this SUPI. The UDM returns the LCS Privacy Profile of the target UE identified by its SUPI, if the SUPI is not allowed to be located, then step 5 to 9 are skipped and this target UE input data is not used for model training at LMF.

NOTE:	The LCS privacy profile check for the target UE is not needed when the target UE is a PRU.

5.	The LMF subscribes to UDM to notifications of changes on subscription data type "LCS privacy”" for this SUPI using Nudm_SDM_Subscribe. If the LCS requests is allowed, then step 6 follows, otherwise no data is collected for this SUPI, i.e. Steps 6 to step 9 are not performed.

6.	The LMF requests input data for the UE from the NG-RAN.

7.	The LMF requests ground truth data from the UE.

8.	The LMF may determine that the UE is no longer in the area of interest, based on the AMF notification using Namf_EventExposure service, then the LMF performs step 10 and step 11, the LMF may unsubscribe to be notified on LCS privacyupdates if the UE is not in the area of interest any longer.

9.	The UDM may notify the LMF on changes of the LCS privacy profile at any time after step 5 using Nudm_SDM_Notification including SUPI and Subscription data type set to "LCS privacy". If the LCS request is  is no longer granted for a user for which data has been collected the LMF performs step 10 and step 11. The LMF may unsubscribe to be notified of LCS privacy updates from UDM for each SUPI for which the LCS request has been revoked, using Nudm_SDM_Unsubscribe including SUPI and Subscription data type set to "LCS privacy".

************** End of changes **************