3GPP TSG-WG SA2 Meeting #169	S2-2504894

# Fukuoka, Japan, 2025-05-19 – 2025-05-23		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6299 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Use of UPF EventExposure for Energysys |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | See DP S2-2504893 Main points: - SMF does not know ULI if I-SMF is used. Also mapping of ULI to gNB ID seems not really a task for SMF. - Unecessary signalling via SMF when reporting volume from UPF. - Overlapping filtering information can cause problems if URR is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of how EIF gets wanted information for Energy calculation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EIF may not get right information if I-SMF is used. Unecessary configuration of SMF. Ineffecient signalling if reporting via SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 5495 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 5.51.2.2.2	Energy Consumption information collection from SMF

The serving AMF and SMFs for a UE are retrieved by the EIF from the UDM based on the provided input parameters including the UE ID and (S-NSSAI, DNN).

The EIF invokes Namf_EventExposure_Subscribe to get the gNB ID from AMF. Then the EIF invokes Nsmf_EventExposure_Subscribe as defined in TS 23.502 [3] with the required granularities (UE ID, DNN/S-NSSAI, application information (e.g. Application Identifier, or Packet Filters)) to retrieve the information from UPF, which is shown in Table 5.51.2.2.2-1a. The EIF receives the data volume of the required granularities from the PSA UPF. The information collected from UPF by EIF, is shown in Table 5.51.2.2.2-2.

Table 5.51.2.2.2-1a: Information to SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE ID | SUPI. |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session. |
| application information | Packet filters or Application Identifier . |
| UPF event ID and event input | Event ID of UPF and input for gettingvolume information with right granularity |
| Path identifier indicator | Indicates that 5GC path is wanted for the measurement |
| Default Timer identifier | Indicating that information reporting interval from is the PLMN-wide configurable starting times and interval T |

Table 5.51.2.2.2-1b: Information from SMF to UPF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE ID | SUPI. |
| UE IP address | UE IP address. |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session. |
| application information | Packet filters or Application Identifier. |
| UPF event ID / event input | Event ID and input for UPF event for getting volume information with the right granularity |
| Path identifier indicator | Indicates that per 5GC path reporting is wanted |
| Default Timer identifier | Indicating that information reporting interval is the PLMN-wide configurable starting times and interval T |

Table 5.51.2.2.2-2: Information from UPF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE IP address | UE IP address. |
| UE ID | SUPI. |
| S-NSSAI | Network Slice applicable to a UE |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session. |
| Packet Filters | Packet Filters as in clause 5.7.6 for IP or Ethernet traffic. |
| Application Identifier | Identification for the traffic of the service data flow. |
| List of Data Volume information | The data volume and the associated UPF(s) and gNB(s) serving the UE within the time interval. |
| > UL/DL Data Volume | The UL/DL Data Volume of a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a Service Data flow (UE ID, S-NSSAI, DNN, Packet Filters/Application Identifier). |
| > (I-)UPF ID(s) | Identifier of UPFs in the 5GC user plane path. I.e. any (I-)UPF(s), e.g., NF ID(s) of UPF(s) or userplane address(es) UPF(s) associated to a reported data volume used by a PDU Session identified by UE-ID, S-NSSAI/DNN and optionally Packet Filters/Application Identifier. |
| Reference to Time Interval | Indicate the time interval of the collected information (e.g. time stamps). |

NOTE 2:	Each entry of List of Data Volume information represents the serving UPF(s) corresponding to the same Data Volume regarding the required granularities. A new entry is added when the  UPF(s) is changed. EIF needs to correlate the gNB ID received from AMF with the information received from UPF.

# * * * * End of changes * * * *