SA WG2 Meeting #169	S2-2504896

Fukuoka, Japan, May 19 - 23, 2025	(revision of S2-250xxxx)

Source: 	Ericsson

Title: 	KI #3, New Solution: Report current and anticipated energy related information in the NF profile

Document for: 	Approval

Agenda Item: 	20.4.1

Work Item / Release:	FS_EnergySys / Rel-20

Abstract of the contribution: This contribution provides a solution for KI#3 on the NF selection aspect based on current and anticipated energy related information.

# Introduction

This solution applies to KI#3 on the NF selection and re-selection aspects based on energy related information in TR.700-67:

KI#3: Whether and how to support NF (re)selection and/or UP path adjustment based on energy related information.

The proposal is based on the following key observations:

O1.	The energy consumption behaviour of the NFs may suddenly change with their load

O2.	Other energy related information (ratio of the renewable energy, energy cost, energy storage etc.) may differ per NF, and it may also change sharply

O3.	Selecting NFs only on current energy information may not always be energy efficient. It is beneficial if the consumer NFs get information about the anticipated energy information change of the producer NFs.

Observation O1 highlights the fact that when the NF load reaches some load limits, its relative (e.g., per packet or per PDU Session) energy consumption may change radically. Some examples are listed below:

-	If the SRAM limit on the HW accelerated component on a UPF is reached or the limit for specific UP traffic processing (e.g., deployed on a smart-NIC) is reached, then the energy consumption of the UPF could suddenly increase as new CPUs may need to take over the CPU-intensive processing of the new sessions. Since some of these limits do not depend on the traffic volume to process (i.e., packets processed in a second) but rather on the number of states to keep (e.g., number of PDU Sessions), reaching the above limits may be difficult to relate solely to the UP utilization (e.g., processed packets per second) of the UPF.

-	If the CPUs allocated to the NF get overloaded, then a new CPU needs to be switched on (Upscaling), which also results in an immediate local increase in the energy consumption. Similarly, a sudden decrease of the energy consumption happens at CPU downscaling.

In conclusion, a static value that describes the energy consumption of the NF cannot be pre-configured, since this value is likely to change in an operational network, depending on the traffic volume and other conditions (specific HW or memory limitation) in the NF. Some means to inform the consumer NFs about the current energy information of the producer NFs is thus needed.

Observation 2 can e.g., relate to the scenarios where different NFs are placed to different locations, where the power supplier at least partly differs from the national grid provider. This could be in the places where the cloud provider provides also (parts of) the electricity, as in many cases there are locally placed power plants by the HCP to support the energy requirement of the given DC. It is also possible that the operator or the site owner deploys renewable energy sources and local batteries to partly of fully cover the energy needs of its deployed devices. This also means that the energy information could change dynamically on that site, depending on the renewable energy resource (e.g., solar power) available, and the remaining battery power level.

Observation 3 is exemplified though a simple scenario summarized in Table 1-1 below. The example assumes that there are two NFs, NF1 and NF2 of the same capability to choose for a certain service, and NF2 is optimized for the given service, so its relative (load-dependent) energy consumption is half of that of NF1. However, it is also assumed that, by its design architecture, the relative energy efficiency of NF2 is lost above a certain load (say, 0.5), and NF2 consumption becomes equal to that of NF1.

Table 1-1 compares the relative energy consumption of the system for different total load of the given NF type (NF1+NF2) assuming different selection logic for the Consumer NF:

-	load-aware: the selection logic chooses the less loaded NF (it does not consider energy related information in the selection)

-	current: the selection logic selects the NF with the lowest current energy consumption as a first criteria. If the energy consumption is the same, then the NF load is considered as for the load-aware logic.

-	current + anticipated: the selection logic considers both the current and anticipated energy consumption, including the NF load when the changes occur, and thus trying to avoid that NF2 works in a less energy-efficient regime.

The energy efficiency for “current” and “current + anticipated” are compared to that of the “load-aware” selection. One can see that both the “current” and the “current + anticipated” equally overperform the “load-aware” for low loads, but when the load of NF2 reaches 0.5, its energy consumption equals that of NF1, thus NF1 will be preferred by the “current” logic because of the lower load, which could eventually decrease the load on NF2, getting it again into the low consumption regime, which would again trigger its reselection for the new sessions, and so on. These changes in the selection logic outcome would result in frequent energy consumption changes of NF1 (and corresponding updates on the consumer NFs) so the relative energy consumption for “current” will land between 0.6875 and 1 (depending on how the system load fluctuates) while the “current + anticipated” can maintain the energy consumption of NF2 in the optimal regime and thus obtain a stable energy efficiency gain over the “load-aware”. If the system load increases further, the “current” will perform as the “load-aware”, but the “current + anticipated” will still optimize the energy consumption of NF2 as long as it can be kept below 0.5 utilization.

Table 1-1: Relative energy consumption compared to the load-aware NF selection for the discussed scenario

| Total load | NF load |  |  |  | Relative energy consumption (current) / load-aware selection | Relative energy consumption (current + anticipated) / load-aware selection |
| --- | --- | --- | --- | --- | --- | --- |
|  | NF1, current | NF1, current + anticipated | NF2, current | NF2, current + anticipated |  |  |
| 0.2 | 0 | 0 | 0.4 | 0.4 | (0x2E+0.4xE)/(0.2x2E+0.2xE) = 2/3 | 2/3 |
| 0.4 | 0.3 | 0.3 | 0.5±Δ | 0.5-Δ | [0.6875, 1] | 0.6875 |
| 0.6 | 0.6 | 0.7 | 0.6 | 0.5-Δ | 1 | 0.792 |
| 0.8 | 0.8 | 1 | 0.8 | 0.6 | 1 | 1 |

Related to other types of energy related information as in Observation 2, we note that the time when the energy information is anticipated to change is also important. For example, if it is known in advance when a renewable energy efficiency source will be switched on and off, or it is anticipated when a local battery depletes, this could be indicated to the NF selection logic with an anticipated energy selection preference value and a coupled time attribute when this occurs. This may then enable the consumer NFs gradually off-load the given producer NF if its energy conditions become unfavourable. The scheduled switch-off time could also be indicated in this way, by setting a large value for the anticipated energy preference for the given NF together with a switch-off time value.

Based on the above observations it is concluded that there could be changes in the energy related information in the individual NFs that could impact the overall energy efficiency of the 5GS. To support a more energy-efficient NF (re-)selection, it is thus proposed that information related current and anticipated energy related selection preference information of the NF is maintained in the NF profile at the NRF, indicating also the NF load and/or the time when the anticipated change occurs.

# 2	Proposal

It is proposed to capture the following solution in TR 23.700-67.

First change

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |  |
| X |  |  | X |  |

Second change (all new)

## 6.x	Solution #X: Report current and anticipated energy related information in the NF profile

### 6.x.1	High-level solution principles

It is proposed that the NF profile in the NFR is extended with

‐	Current energy related information of the NF (“Energy Preference” attribute)

-	Anticipated energy related information of the NF (“Anticipated Energy Preference” coupled with either an “Anticipated Energy Preference Load Level” or “Anticipated Energy Preference Time”)

### 6.x.2	Description

This is a solution related to KI#3:

‐	Whether and how to support NF (re)selection and/or UP path adjustment based on energy related information.

It is proposed that the NF profile in the NRF is extended with current and anticipated energy information related to the given NF, to facilitate a more energy-efficient NF (re-)selection, e.g., UPF (re-)selection by the SMF or SMF (re-)selection by the AMF. In the former case, reporting may result in UP path adjustments, based on the decision of the SMF. The NRF is updated either by the NF itself or by the OAM.

During NF discovery, the consumer NFs may receive the current and anticipated energy related information in the NF profile and may use it in the producer NF selection process.

The current energy related information may be represented by an optional “Energy Preference” attribute that indicates the selection preference of the given NF relative to other NFs of the same type and capability considering the energy characteristics of the NF.

The anticipated energy related information may be reported e.g., by these (optional) new attributes in the NF profile:

-	“Anticipated Energy Preference”: anticipated value of the “Energy Preference” attribute.

-	“Anticipated Energy Preference Load Level”: anticipated NF load level of the change of the “Energy Preference” to the value in the “Anticipated Energy Preference”. It should be coupled with an “Anticipated Energy Preference” value.

-	“Anticipated Energy Preference Time”: anticipated time of the change of the “Energy Preference” to the value in the “Anticipated Energy Preference”. It should be coupled with an “Anticipated Energy Preference” value.

All above attributes are optional in the NF profile.

### 6.X.3	Procedures

It is proposed that the existing NRF-related procedures are used to convey the attributes related to the current and anticipated energy related information to the consumer NFs. The consumer NFs may use Nnrf_NFManagement_NFStatusSubscribe to subscribe to NRF for NF status changes, see clause 5.2.7.2.5 of TS 23.502 [3]. If so, then NRF may use Nnrf_NFManagement_NFStatusNotify to notify the consumer NFs of any NF status change, including the energy relate d information, see clause 5.2.7.2.6 of TS 23.502 [3].

The producer NFs may directly provide an update of the attributes to the NRF, by using the Nnrf_NFManagement_NFRegister and Nnrf_NFManagement_NFUpdate operations, see clauses 4.17.1 and 4.17.2 of TS 23.502 [3].

### 6.X.4	Impacts on existing services, entities and interfaces

Consumer NFs:

-	Consider the current and anticipated energy consumption related information received from the in the NF (re)selection.

Producer NFs:

-	Report changes to their current and anticipated energy consumption related information to the NRF (optional feature)

NRF:

-	Supports new attributes related to the current and anticipated energy consumption related information in the NF profile and related SBIs.

End of Changes