3GPP TSG-SA2 Meeting #169	S2-2504899

Fukuoka, JP, 19-23 May, 2025

Source:	China Mobile

Title:	New solution for KI#4 Sensing Data Collection and Transportation

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The Sensing data collection and transportation for FS_Sensing_ARC are proposed.

1. Introduction

This pCR proposes the solution for Key issue#4 Sensing data collection and transportation

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X |  |  |  | X |  |  |

# * * * * Second change (All Text New) * * * *

6.X	Solution #X:Sensing Data Collection and Transportation

6.X.1	Description

This solution aims to address key issue#4 on how to collect sensing data for sensing result generation.

The Sensing Control Function (SCF) requests Sensing Entity/Entities for sensing data and the associated information for sensing data collection and transportation. Meanwhile, SCF could get the the associated information for sensing data collection and transportation from Sensing Procession Function (SPF) and align those information with selected Sensing Entity/Entities, Multiple sensing data can be collected by SPF, the SCF determines the type of the sensing data based on the sensing task requested by the AF.

6.X.2	Procedure for Sensing Data Collection and Transportation

Figure 6.X.2: Sensing Data Collection and Transportation procedure.

- The AF sends Nnef_Sensingservice Request message to the NEF, the request message contains sensing service requirement parameters (e.g. Sensing service type, Target Sensing Service Area, one-time report/periodical report/event-triggered,).

- The NEF sends the Nscf_Sensingservice Request message to the selected SCF. The request message contains the sensing service requirement parameters.

- To ensure that the selected SPF suitable for the required sensing service, the SCF determines based on the supported service area of the SPF and Target Sensing Service Area requested by the AF. Then the SCF sends a Sensing Task Initiation Request to the seleted SPF and SPF replies the IP/TEID information for data reporting.

- The SCF determines the type of sensing data uploaded by the sensing entity/entities based on the sensing service type which requested by the AF. The SCF sends a StartSenseTask Request to the Sensing Entity, the request message contains sensing service requirement parameters (e.g. Sensing data type, one-time report/periodical report/event-triggered).The 3GPP Sensing data may include the following type:

-  I/Q streams of the raw signal,

-  Delay spread spectrum, Doppler spectrum,

-  Point cloud information of the sensing target,

-  Distance, velocity, trajectory, and associated information of the target.

NOTE x: The scope of sensing data are determined by RAN WG.

5.Upon receiving the sensing service request message from the SCF, the Sensing Entity/Entities create sensing data path for 3GPP Sensing Data reporting. The corresponding Endpoint of the sensing data path are defined as follows:

-  Source Tunnel Endpoint: Tunnel information provided by the Sensing Entity/Entities.

-  Target Tunnel Endpoint: Tunnel information for data reporting provided by the selected SPF.

6. The Sensing Entity/Entities performs the Sensing measurement and obtains 3GPP sensing data based on the sensing service requirement.

7. The Sensing Entity/Entities transmits the 3GPP sensing data to the SPF.

8. After receiving the 3GPP sensing data, the SPF performs the generation of the Sensing result.

9. The SPF exposes the sensing result to AF which is addressed by solutions to Key Issue #5.

6.X.3	Impacts on services, entities and interfaces

NEF:

-  Support receive sensing service requirements and expose sensing result.

SPF:

-  Support sensing data collection and transport.

-  Support sensing result generation based on the 3GPP sensing data.

-  Support sensing result exposure.

SCF:

-  Support sensing result reception and exposure.

# * * * * End of changes * * * *