3GPP TSG-SA WG2 Meeting #169	S2-2504932

Fukuoka, Japan, 19-23 May 2025

Source:	China Mobile

Title:	Solution for IMS signaling performance optimization to support voice over GEO

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes solutions for KI#2.

# 1	Discussion

To achieve SA1’s IMS voice call setup time KPI over GEO satellites, the end-to-end signaling process needs adjustments for satellite link limitations. Main issues are unavoidable delays and limited bandwidth, which slow down signaling and increase retransmissions.

The solution focuses on optimize signaling flows, adapting protocol parameters, and compressing message to reduce end-to-end latency.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-19 V0.1.0.

* * * First Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| x |  | x |  |  |

* * * Next Change * * * *

6.X	Solution #X: <Support of IMS signalling performance optimization>

6.X.1	High level principles

This solution resolves Key Issue #2 IMS enhancement for GEO NB-IoT NTN access.

This solution is focus on simplifying SIP signaling and optimizing IMS call procedures.

6.X.2	Description

To meet SA1 performance requirements for IMS voice call setup time using GEO satellite, the IMS signaling messages exchanged between UE and IMS network via GEO satellite need to be optimized.

The IMS signaling flow always requires high reliability. It’s a big challenge to support IMS signaling exchange between UE and IMS network in the scenarios of voice services via GEO satellite. The transmission data rate of NB-IoT via GEO satellite is very restricted, e.g., 1-3 kbps as described in TR 22.887 [10], which causes long transmission delay for the IMS signaling message delivery. The propagation delay via satellite is associated with these orbit ranges. The UE to ground max propagation delay is estimated to be 272 ms, as specified in TS 22.261 [11]. In addition, the re-transmission of IMS singling via GEO satellite is more frequently than terrestrial access due the radio resource condition.

Considering the above, the message size and number of exchanged signaling messages between UE and IMS network need to be reduced to meet SA1 requirements on IMS call setup time. At the same time, due to the constraints of long transmission delays under GEO satellite access, adjustments must be made to the existing SIP timer settings. In GEO satellite access scenarios, the delay mainly comes from the SIP request and response signaling exchanged between the UE and P-CSCF over the Gm interface. To reduce transmission delay, the size of SIP messages needs to be simplified. At the same time, to reduce one-way propagation delay, the IMS call flow can be optimized. The following solutions focus on optimizing the SIP URI, SIP header, SIP body, and SIP timers.

6.X.3	Procedures

6.X.3.1	 SIP URI simplification

The SIP URI appears multiple times in SIP messages (e.g., in the request line, Via, From/To, Contact headers, etc.), leading to redundant transmission and parsing overhead. The table below shows the mandatory and common fields where SIP URIs appear：

Table 6.X.3.1-1: SIP URI in SIP Headers

| Header | Example | Presence |
| --- | --- | --- |
| Request-Line | sip:460080123456789@ims.mnc008.mcc460.3gppnetwork.org | M |
| From | From: <sip:+8613512345678@ims.mnc008.mcc460.3gppnetwork.org>; tag=12345678 | M |
| To | To: <sip: +8613912345678@ims.mnc008.mcc460.3gppnetwork.org>;tag=12345678 | M |
| Contact | Contact: "Mr. Watson"<sip: +8613512345678@ims.mnc008.mcc460.3gppnetwork.org>; q=0.1; expires=60 | M |
| Via | Via: SIP/2.0/UDP +8613512345678@ims.mnc008.mcc460.3gppnetwork.org:5060;rport;branch=z9hG4bK123456 | M |
| Route | Route: <sip:server10.ims.mnc008.mcc460.3gppnetwork.org;lr> | O |

SIP URIs may also appear in the SIP SDP message body (e.g., in the <o> and <c> fields). Therefore, simplifying SIP URIs should be prioritized. Typically, a SIP URI is approximately 45–55 bytes in length. Through optimization, this can be reduced to 30–40 bytes, achieving a compression rate of around 30%. Below is an example of a simplified SIP URI:

Table 6.X.3.1-2: Example of SIP URI Simplification

| SIP URI | SIP URI after Simplification | Reduction Size / Rate |
| --- | --- | --- |
| 460080123456789@ims.mnc008.mcc460.3gppnetwork.org | 460080123456789@i.008.460.3g.org | 17 B / 34.7% |

6.X.3.2	 SIP header simplification

In terms of simplifying SIP header fields, an efficient compression strategy can be achieved by streamlining the message headers. Specifically, based on the requirements of RFC 3261, only the essential core header fields (such as Via, From, To, Call-ID, CSeq, Contact, and Max-Forwards) should be retained in SIP messages, as they are mandatory for message routing, transaction integrity, and session control. Non-essential headers (e.g., User-Agent, Allow, Supported) can be dynamically removed depending on the actual scenario to reduce redundant data.

Furthermore, for the retained essential headers, RFC 3261 permits the use of abbreviated forms to further compress the message size. This optimization can reduce the size of a single header field by 1 to 14 bytes. A simplified example is provided in the following table:

Table 6.x.3.2-1: Simplified SIP headers by referencing RFC 3261

| Full Name | Short Form | Reduction |
| --- | --- | --- |
| Call-id | i | 6B |
| Contact | m | 6B |
| Content-length | l | 14B |
| Content-type | c | 11B |
| From | f | 3B |
| Session-Expires | x | 14B |
| Supported | k | 8B |
| To | t | 1B |
| Via | v | 2B |

When UE sends a simplified SIP message to the P-CSCF in the IMS network, the P-CSCF shall perform standardized message reconstruction: First, abbreviated header fields (e.g., "v" restored to "Via") shall be converted to their original format to ensure backward compatibility with downstream IMS network functions. Additionally, mandatory header fields (e.g., Record-Route, Path, P-Asserted-Identity) shall be inserted in accordance with operator-specific policy requirements to enable proper signaling routing, user authentication, and service continuity.

6.X.3.3	 SIP message body simplification

The message body in SIP messages shall use the SDP protocol. The SDP protocol supports the description of various session parameters, including media type, media format, media address, etc. For voice communication services, the message body shall retain only the mandatory SDP fields—v, o, s, c, t, m, and a—while omitting unnecessary fields such as e, k, and p to reduce message body length. The specific content of each retained line shall remain unchanged. Below is an example of a simplified SDP message body:

v=0 o=Alice 45698 121678 IN IP4 1.1.1.1 s=- c=IN IP4 10.10.10.40 t=0 0 m=audio 10052 RTP/AVP 8 a=rtpmap:8 PCMA/8000

Figure 6.x.3-3: Example of simplified SDP

6.X.3.4	 Comparison of SIP Message Simplification

The methods in sections 6.X.3.1, 6.X.3.2, and 6.X.3.3 simplify the encoding structure of SIP messages. By using these methods, the typical SIP signaling data during session setup is reduced, as shown in the table below.

Table 6.x.3.3-1: Message size comparison

| SIP Message | Message Size/B | Message Size After Simplification/B | Compression Ratio/% |
| --- | --- | --- | --- |
| INVITE | 861 | 572 | 66.48 |
| 100 Trying | 638 | 397 | 62.23 |
| 183 Session Progress | 823 | 552 | 67.07 |
| 180 Ringing | 703 | 437 | 62.16 |
| 200 OK | 633 | 398 | 62.88 |
| PRACK | 639 | 397 | 62.13 |
| ACK | 570 | 359 | 62.98 |
| Average Compression Ratio |  |  | 63.69 |

6.X.3.5	 Call Procedure Optimization

To reduce call setup delay for users accessing via GEO satellites, the SIP call establishment procedure can be optimized. Given that the primary delay originates from signaling transmission between the UE and the core network, SIP message exchanges over the Gm interface can be reduce by delete unnecessary messages. This reduces both transmission delay and one-way propagation delay.

6.X.3.5.1 Mobile origination procedure

The following figure depicts the MO procedure in the optimized IMS call flow for NB-IoT access over GEO satellites.

Figure 6.X.3.5.1: Mobile origination procedure

- UE sends an initial simplified INVITE request to the P-CSCF.

- Upon receiving the simplified INVITE request, the P-CSCF first authenticates the originating UE's identity. The P-CSCF sends an AAR (AA-Request) to the PCRF to request call resources and the originating user's location information. The PCRF returns an AAA (AA-Answer). Resource reservation is completed on the originating side, and location information is obtained. The PCRF sends an RAR (Re-Auth-Request) to the P-CSCF to report the location information, and the P-CSCF returns an RAA (Re-Auth-Answer).

- P-CSCF converts the simplified INVITE request into a standard INVITE request and forwards it to the I/S-CSCF along the path determined by the originating UE’s latest registration process.

- I/S-CSCF sends a standard 100 Trying response to the P-CSCF. Before sending the message, the I/S-CSCF performs the following operations: stores the Contact header from the response message and sends the response based on the Via header content.

- P-CSCF sends a simplified 100 Trying response to the originating UE. Before transmission, the P-CSCF: stores the dialogue identifier from the response and associates it with the originating UE’s IMPU; stores the Contact, From, and To information.

- I/S-CSCF sends a standard 183 Session Progress response to the P-CSCF.

- P-CSCF forwards a simplified 183 Session Progress response to the originating UE.

- UE sends a simplified PRACK message to the P-CSCF.

- P-CSCF converts and sends a standard PRACK message to the I/S-CSCF.

- I/S-CSCF sends a standard 200 OK response to the P-CSCF.

- P-CSCF forwards a simplified 200 OK response to the originating UE.

- P-CSCF sends an AAR to the PCRF, which returns an AAA. The originating side modifies resource requests based on negotiation results and confirms successful resource reservation.

- P-CSCF initiates a standard UPDATE message to the I/S-CSCF for the called side.

- I/S-CSCF sends a 200 OK response to the P-CSCF.

- When the called UE rings, I/S-CSCF sends a standard 180 Ringing response to the P-CSCF. If the called supports color ringback tone service, the I/S-CSCF removes the P-Early-Media header from the 180 Ringing response and skips subsequent media playback procedures.

- P-CSCF sends a simplified 180 Ringing response to the originating UE.

- P-CSCF sends a standard PRACK message to the I/S-CSCF.

- I/S-CSCF sends a standard 200 OK response to the P-CSCF.

- After the called UE answers, the I/S-CSCF sends a standard 200 OK response to the P-CSCF.

- P-CSCF forwards a simplified 200 OK response to the originating UE.

- UE acknowledges the 200 OK response with a simplified ACK message.

- P-CSCF sends a standard ACK message to the I/S-CSCF.

6.X.3.5.2  Mobile termination procedure

The following figure depicts the MT procedure in the optimized IMS call flow for NB-IoT access over GEO satellites.

Figure 6.X.3.5.2: Mobile termination procedure

- Upon receiving the INVITE request, if the called party supports CRBT, the I/S-CSCF removes the P-Early-Media header field from the INVITE request before forwarding it to the P-CSCF.

- The P-CSCF, after receiving the INVITE request, checks whether the SIP message is intended for a satellite user. If not, it converts the INVITE into a simplified INVITE message and forwards it to the called UE.

- The called UE sends a simplified 100 Trying response to the P-CSCF.

- The P-CSCF converts and forwards a standard 100 Trying response to the I/S-CSCF.

- The called UE sends a simplified 183 Session Progress response to the P-CSCF.

- The called-side P-CSCF sends an AAR request to the PCRF for call resource reservation.

- The PCRF returns an AAA response to the P-CSCF.

- The PCRF sends an RAA request to the P-CSCF, which replies with an RAR response.

- The P-CSCF forwards a standard 183 Session Progress response to the I/S-CSCF.

- The I/S-CSCF sends a standard PRACK message to the P-CSCF.

- The P-CSCF converts and forwards a simplified PRACK message to the called UE.

- The called UE sends a simplified 200 OK response to the P-CSCF.

- The P-CSCF converts and forwards a standard 200 OK response to the I/S-CSCF.

- The called UE sends a simplified 180 Ringing response to the P-CSCF, which caches this simplified response.

- The I/S-CSCF sends a standard UPDATE message to the P-CSCF.

- After successful resource reservation via media negotiation with the PCRF, the called-side P-CSCF sends a standard 200 OK response to the I/S-CSCF.

- The called UE starts ringing, and the P-CSCF converts the cached simplified 180 Ringing into a standard 180 Ringing response and forwards it to the I/S-CSCF.

- The I/S-CSCF sends a standard PRACK message to the P-CSCF.

- The P-CSCF sends a standard 200 OK response to the I/S-CSCF.

- The called party answers, and the called UE sends a simplified 200 OK response to the P-CSCF.

- The P-CSCF converts and forwards a standard 200 OK response to the I/S-CSCF.

- The I/S-CSCF sends a standard ACK message to the P-CSCF.

- The P-CSCF converts and forwards a simplified ACK message to the called UE.

Based on the optimized MO call setup procedure and MT call answering procedure described above, the total message size transmitted over the Gm interface is as follows:

Table 6.X.3.5-1: Message size comparison

| Message Flow | Message Size/B | Message Size After Optimized/B | Compression Ratio/% |
| --- | --- | --- | --- |
| MO | 8167 | 5500 | 67.3 |
| MT | 8167 | 5500 | 67.3 |

By applying the SIP message simplification method to the optimized procedure, the overall simplification and optimization results are shown in the table below:

Table 6.X.3.5-2: Message size comparison

| Message Flow | Message Size/B | Message Size After Optimized/B | Compression Ratio/% |
| --- | --- | --- | --- |
| MO | 8167 | 3510 | 42.9 |
| MT | 8167 | 3510 | 42.9 |
| Total | 16334 | 7020 |  |

6.X.3.6	 SIP Timer Optimization

In GEO satellite access scenarios, if the default SIP timers (e.g., T1/T2 retransmission timers) designed for terrestrial networks are retained, the excessive propagation delay (~270 ms one-way, ≥540 ms round-trip) will cause premature timer expiration. This results in unnecessary signaling retransmissions, increasing network congestion and call setup failure rates. To avoid this issue, the timer thresholds should be extended to align with GEO latency characteristics, effectively preventing spurious retransmissions and ensuring reliable SIP protocol operation and session establishment in high-delay environments.

Table 6.X.3.6-1: SIP Timer Optimization Requirements

| Timer | Standard Value | Definition | Suggest Value | Optimization Information |
| --- | --- | --- | --- | --- |
| T1 | 0.5s | Round-trip message delay | 9s | The time required to send a SIP message: 8 × 0.7 KB (average SIP message size) / 1 kbps (satellite link rate) + 2.6 s = 8.2 s, recommended to adjust to 9 s. |
| T2 | 4s | Maximum retransmission interval for non-INVITE request messages and INVITE response messages | 18s | The SIP response message transmission time is approximately 9 s. It is recommended to set T2 ≥ 2 × SIP response transmission time (18 s), ensuring at least 1 retransmission. |
| T4 | 5s | Maximum lifetime of a SIP message in the network | 20s | The time to stop retransmissions must be greater than the T2 timer duration. |
| Timer A | T1 | Retransmission interval for INVITE messages when using UDP transport | 9s | Same as the T1 timer setting. |
| Timer B | 64T1 | INVITE transaction timeout duration | 72s | For the maximum total retransmission duration timer, the intervals are 9 s, 18 s, 36 s, 72 s, set to 72 s for a total of 4 retransmissions. Previously, 0.5 s, 1 s, 2 s, 4 s, 8 s, 16 s, 32 s allowed 7 retransmissions. |
| Timer C | 180s | Proxy server timeout duration for processing INVITE transactions | - | The maximum duration of an INVITE transaction, after process simplification, is less than 180 s. No adjustment is needed. |
| Timer D | 64T1 | Time to wait for retransmitted response messages after receiving a response | 72s | Same as the Timer B setting. |
| Timer E | T1 | Retransmission interval for non-INVITE request messages when using UDP transport | 9s | Same as the T1 timer setting. |
| Timer F | 64T1 | Non-INVITE transaction timeout duration | 72s | Same as the Timer B setting. |
| Timer G | T1 | Retransmission interval for INVITE response messages | 9s | Same as the T1 timer setting. |
| Timer H | 64T1 | Time to wait for an ACK after sending a final response | 72s | Same as the Timer B setting. |
| Timer I | T4 | Time to continue waiting for retransmitted ACKs after receiving an ACK | 20s | Same as the T4 timer setting. |
| Timer J | 64T1 | Time to continue waiting for retransmitted non-INVITE requests after sending a final response to a non-INVITE request | 72s | Same as the Timer B setting. |
| Timer K | T4 | Time to continue waiting for retransmitted response messages after receiving a response | 20s | Same as the T4 timer setting. |

6.X.4	Impacts to Services, Entities and Interfaces

UE:

-	Supports sending simplified SIP messages

-	Supports determining when to send simplified SIP messages based on the access type

P-CSCF:

-	Supports sending simplified SIP messages

-	Supports restoring simplified SIP messages to their original standard format

-	Supports determining when to send simplified SIP messages based on the UE's access type

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.101: "Service aspects; Service principles".

[10]	3GPP TR 22.887: “Study on satellite access - Phase 4”.

[11]	3GPP TS 22.261: " Service requirements for the 5G system".

[12]	IETF RFC 3261: "SIP: Session Initiation Protocol".

* * * End of Changes * * * *