3GPP TSG-WG SA2 Meeting #169	S2-2504980

Fukuoka, Japan, 19 – 23 May 2025

Source:	ETRI

Title:	KI#2, New Sol: NWDAF-assisted Analytics for Efficient User Plane Performance

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: This solution defines a practical methodology for KI#2, enabling AI/ML-assisted user plane traffic pattern and behavior analysis to support dynamic and efficient performance control in the user plane.

# 1. Introduction/Discussion

In clause 5.3 and 5.3.1 of TR 23.700-04 v0.1.0, the key issue (KI#2: AI/ML-assisted user plane traffic pattern and behaviour analysis to support efficient performance of the User Plane) identifies the need to study:

-	How to provide analytics (e.g. enhancing existing in TS 23.288 [5] or introducing new) including the aspects of required inputs/outputs to improve performance of user plan.

-	Which consumers can subscribe/request the analytics and how the consumers use the analytics to take actions to improve user plane performance.

-	Whether and how to train ML models and perform the inference for the user plane analytics, considering solutions that minimize the load on Control Plane NFs.

This contribution proposes a solution for FS_AIML_CN_Ph2 addressing KI#2.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-04

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| 6.X |  | x | x | x |

# * * * * second change * * * *

## 6.X	Sol #X: NWDAF-assisted Analytics for Efficient User Plane Performance

### 6.X.1	Description

This solution is based on the use cases and key issues defined in TR 23.700-04. It targets user plane traffic that is unpredictable, bursty, and difficult to manage with static rules. In 5GS, traffic behavior can change quickly depending on user activity, service type, or device pattern. Static PDR or QER settings may not be enough, especially when traffic volume or direction shifts suddenly. Examples include IoT devices, live media, encrypted applications, or moving UEs. These often produce flows that are short, bursty, or irregular. This makes shaping, buffering, or session control difficult. When traffic is encrypted, the network also cannot identify the service easily, so QoS may not match the actual traffic. Instead of detecting anomalies, this solution focuses on analyzing traffic patterns using NWDAF. The NWDAF can provide analytics that help other functions adjust parameters like DLDR timer, QER, or forwarding paths. This allows more flexible control, especially for services where traffic is variable or sensitive to how resources are handled.

NOTE1: This solution focuses on AI/ML-based traffic pattern analysis and does not aim to directly control network functions.

### 6.X.1.1	Input Data

To support AI/ML-based traffic analysis, NWDAF can collect data from network functions (NFs) and OAM, including data from MDT.

Table 6.x.1.1-1: NWDAF Input Data for UP Traffic Pattern Analysis

| Information | Source | Description |
| --- | --- | --- |
| UE IP Address | SMF, UPF | IP address assigned to the UE, used to associate traffic flows. |
| SUPI (within PDI) | SMF, UPF | Subscription Permanent Identifier in the Packet Detection Information, uniquely identifying the UE. |
| DL TEID | UPF | Downlink Tunnel Endpoint Identifier used in GTP-U for packet forwarding. |
| N6 DL Packet Info (5-tuple + header) | UPF | Contains source/destination IP addresses, ports, and protocol. Application-layer headers may be available depending on encryption. |
| Packet Count (UL/DL, N3/N6) | UPF, OAM | Number of packets observed on N3 and N6 interfaces, for uplink and downlink. |
| QoS Flow Info (5QI, QFI, ARP) | SMF, UPF | Parameters used to define QoS characteristics of each flow. |
| QER Info (GBR, shaping, discard, buffering) | UPF | QoS Enforcement Rule settings such as guaranteed bit rate, shaping behavior, discard handling, and buffer use. |
| FAR / BAR | UPF, SMF | PFCP rules used to forward or buffer packets as per session context. |
| DL Usage Threshold Crossing | SMF, UPF | Triggered when downlink usage exceeds a threshold; may help detect bursty flows. |
| Flow End | SMF, UPF | Marks the termination of a flow; used for resource release or analytics. |
| Congestion Notification | SMF, UPF | Indicates congestion in the UPF or RAN. May be inferred from queue depth or packet drops. |
| UL/DL Volume, Duration | UPF, OAM | Total traffic volume and session duration per direction. |
| Jitter | OAM, UPF | Variation in packet arrival delay. Can be derived per TS 29.244, depending on implementation. |
| UE Registration / Mobility State | AMF | Indicates whether the UE is registered, idle, connected, or suspended. |
| RRC Status | OAM, MDT | RRC state of the UE (e.g., RRC_CONNECTED, RRC_IDLE) from MDT or OAM logs. |
| Idle Duration | MDT, OAM | Duration of UE inactivity, useful for DLDR behavior prediction. |
| Paging Event Count | AMF, OAM | Number or frequency of paging events observed for the UE. |

NOTE2: Encrypted traffic (e.g., QUIC, TLS) is classified based on observable metadata such as 5QI, port, packet size, and timing—not by inspecting payload.

### 6.X.1.2 Output Analytics

The analytics include statistics and prediction results. These outputs can help improve traffic control in the user plane.

Table 6.x.1.2-1: UP Traffic Pattern statistics

| Information | Description |
| --- | --- |
| Throughput | Volume of data transferred per unit time (bps). |
| Packet rate | Number of packets observed per second (PPS). |
| Packet size distribution | Variation in packet sizes over a period of time. |

These statistics help understand how the traffic behaves. They do not always mean there is a problem, but some combinations may show something unusual or important.

- If jitter is high and throughput is low, the service quality may not be good. This can happen if buffering is too much or shaping is not well applied.

- If traffic is bursty and packets come in very fast, the network may get congestion or DLDR. It can happen with services like video upload or push.

- If latency goes up in flows with some 5QIs, maybe the QoS is not matching real traffic. It could be from queue delay or wrong configuration.

These are not strict rules, but they can be good hints. When used with ML or more data, they can support decisions like QER change or DLDR tuning

Table 6.x.1.2-2: UP Traffic Pattern prediction

| Information | Description |
| --- | --- |
| Traffic behavior classification | Indicates the ratio of normal vs. abnormal traffic per QoS Flow or PDU Session (e.g., 60% normal, 40% abnormal). |
| Idle session prediction | Probability of UE remaining inactive; used to optimize DLDR settings. |
| Encrypted traffic classification | Classification of encrypted traffic flows based on 5QI or observable patterns. |
| FAR adjustment | Derived from comparison of traffic pattern and the current PDR/PDI/FAR alignment. |
| QER adjustment suggestion | Suggested updates to QER settings based on observed QoS behavior. |

### Based on these results, some actions may help improve the user plane. These are not always required, but can be useful depending on the operator and deployment.

### DLDR timer tuning: If the system sees that some UEs are idle for long, the timer can be changed to reduce paging or improve downlink delay.

### Shaping intensity adjustment: For bursty traffic or unstable delay, changing shaping level may help.

### QER tuning: When QoS is not working well, QER settings (like GBR or drop rule) can be adjusted.

### QoS reconfiguration: Sometimes 5QI or ARP does not match the service anymore. QoS profile can be updated.

### FAR update: If flow routing is not efficient, FAR can be changed to use a better UPF or path.

### These are possible ways to use the analytics from NWDAF to support more dynamic traffic handling.

NOTE3: NWDAF does not modify QoS or forwarding rules directly. These actions are expected to be taken by consumer NFs (e.g., SMF, PCF) based on analytics.

### 6.X.2 Procedures

Figure 6.x.2.1-1: Data Collection and Analytics for UP Traffic Behavior using NWDAF

- A Consumer NF (e.g., PCF or AF) sends a Nnwdaf_AnalyticsSubscription_Subscribe request to the NWDAF to subscribe to a specific analytics service (e.g., QoS prediction, abnormal behavior detection).

- Upon receiving the subscription, the NWDAF determines the required data and subscribes to DCCF using Ndccf_DataManagement_Subscribe. This message instructs the DCCF to coordinate data collection from appropriate source NFs.

- The DCCF then sends subscription requests to the relevant data-producing NFs such as UPF, SMF, and OAM using either: Nupf_EventExposure_Subscribe, Nsmf_EventExposure_Subscribe depending on the nature of the data (e.g., flow stats, session info, KPIs).

- (Optional) If direct SBI subscription is not feasible (e.g., due to protocol mismatch or legacy systems), the DCCF may utilize the Messaging Framework Adaptor Function (MFAF) to intermediate data subscription using Nmfaf_3daDataManagement_Subscribe.

- The subscribed source NFs, OAM begin sending their reports to the DCCF using: Nupf_EventExposure_Notify, Nsmf_EventExposure_Notify.

- (Optional) Alternatively, if MFAF was used, it forwards the received data to DCCF via Nmfaf_3daDataManagement_Notify.

- DCCF aggregates or filters the collected data as needed and sends it to NWDAF via Ndccf_DataManagement_Notify.

- NWDAF processes the input data, possibly invoking trained internal ML models, and derives new analytics results. A new Analytics ID may be assigned to represent the computed analytics set.

- The analytics output can be optionally stored in the ADRF (Analytics Data Repository Function) via Nadrf_AnalyticsDataManagement_Store for reuse or historical analysis.

- Finally, NWDAF delivers the analysis result to the subscribing Consumer NF using Nnwdaf_AnalyticsSubscription_Notify.

NOTE4: OAM data is obtained via DCCF using implementation-specific interfaces, as defined in TS 28.552. No standard SBI exists for direct NWDAF-OAM interaction.

### 6.X.3	Impact on services, entities and interfaces

NWDAF

- Collects input data from UPF, SMF, OAM, and AF via DCCF or MFAF.

- Analyzes traffic patterns and behavior in the user plane using AI/ML models or statistical logic.

- Produces analytics such as burst level, DLDR timer recommendation, shaping and buffering trends, or pacing indicators.

- Provides analytics to consumer NFs through Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription.

- Congestion status, paging behavior, or UE idle state may be inferred from indicators such as discard count, MDT reports, or paging events—not from direct signaling.

Consumer NFs (e.g., SMF, AF, UPF)

- Request or subscribe to traffic-related analytics from NWDAF.

- Use the results to update control or policy settings, such as QER, shaping level, or DLDR configuration.

- Interact with other NFs to apply the adjustments when needed.

- Apply the updated traffic handling rules, including shaping, buffering, forwarding (FAR/QER), or pacing for DLDR.

# * * * * End of changes * * * *