3GPP TSG SA WG2 Meeting #169	S2-2504985

Fukuoka, Japan, 2025‑05-19 - 2025‑05-23	(revision of S2-220xxxx)

Source:	Apple, Ericsson

Title:	Dynamic detection of L4S traffic and enabling of ECN marking for L4S

Document for:	Discussion

Agenda Item:	19.3.2

Work Item / Release:	XRM / Rel-19

Abstract:

# 1. Background

3GPP Rel-18 has specified support for “ECN Marking for L4S” in TS 23.501 Clause 5.37.3. ECN Marking for L4S can be enabled on a per-QoS Flow basis.

ECN marking for L4S is enabled on a per QoS Flow basis in the uplink and/or downlink direction and may be used for GBR and non-GBR QoS Flows

ECN marking for L4S can be supported by either PSA UPF or NG-RAN.

The NOTE 3 in 5.37.3.1 of 23.501 states the different conditions for enabling a QoS Flow for L4S.

NOTE 3: A QoS Flow may be enabled with ECN marking for L4S requirement e.g. statically when a PDU session is established based on configuration in SMF or PCF, or dynamically based on detection of the L4S traffic (e.g. match packets with ECT(1) or CE (See RFC 9331 [160]) together with IP 5 tuple) in the IP header whereby SMF or PCF triggers a setup of a QoS Flow enabled for L4S, or by requests by an AF.

There are four different methods to enable L4S on a QoS Flow.

i). Static configuration in PCF

ii). Static configuration in SMF

iii). Dynamic detection of L4S traffic

iv). Explicit request from AF.

This discussion paper analyses the required framework in 5GC to realize the third method i.e., dynamic detection of L4S traffic.

# 2. Discussion

## 2.1 Dynamic detection of L4S traffic

This method requires interaction between SMF and UPF. This method makes use of the usage reporting rules and the N4 reporting procedures that are defined in clause 4.4.2 of TS 23.502.

An example follows for the interaction between SMF and UPF and for how SMF determines the N4 Rules (which are further described for this example in 2.2):

Figure 2.1 -1 N4 Session management interaction for detection of L4S traffic

- PCF provides PCC rules that require ECN marking for L4S. This could be during a SM Policy association creation/update for a PDU Session.

- SMF can decide whether to establish at this moment (i.e. statically) the QoS Flow(s) enabled for L4S or whether to establish them dynamically based on detection of L4S traffic on the QoS flow associated initially with the PCC rule. For dynamic QoS Flow establishment (i.e. based on detection of L4S traffic), SMF can make use of solicited application reporting as described in 5.4.11 of 29.244.

- SMF sends a N4 Session Establishment (or Modification) request that instructs UPF to report L4S capable traffic by provisioning PDRs that contain Usage Reporting Rules.

- In UPF, information in the Usage Report IE tells the UPF to send to SMF a Usage report when traffic is detected that matches the PDR (i.e. when detecting IP packets with the header bits set to ECT(1) or CE on the QoS Flow associated with the PCC rule).

- UPF sends a Usage Report to report back to SMF the start of the traffic corresponding to the PDR (i.e. L4S traffic).

- There are now two cases:

  - Case 1: no L4S capable QoS Flow exists that can be used: SMF adds a new QoS Flow with L4S enabled to the PDU Session and updates the QoS Rules and PDRs accordingly (see 2.2).

  - Case 2: an L4S capable QoS flow exists that can be reused: SMF binds it to this PCC Rule too and modifies the QoS Rule and PDRs accordingly.

Observation 1: Existing N4 session management procedures are enough for UPF to detect L4S traffic and report to SMF.

## 2.2 Specifics for the PDR configuration to UPF

For a PCC Rule, SMF can initially provision a set of PDRs (for UPF)/QoS Rules (for UE) that map the traffic to certain QoS Flow. They include rules for UPF to report when L4S traffic is detected (i.e. packet filters include to detect ECT(1) or CE marked packets) and take precedence over paired PDRs (no L4S related filters). When the UPF reports detection of L4S traffic in that QoS Flow, SMF can install new PDRs/QoS Rule that map that L4S traffic now to a “shadow QoS Flow” for that PCC rule (i.e., the new QoS Flow dynamically created with same QoS characteristics but additionally enabling L4S). Thus the new PDRs / QoS Rule have the same packet filters as the original PDRs/QoS rule and in addition have filters to detect ECT(1) or CET marked packets and take precedence over paired PDRs/QoS rule.

This scheme of enabling L4S will look like the figures below. Each PCC rule will translate to two QoS Flows with similar characteristics once L4S traffic is detected: one for non-L4S traffic and another one for L4S traffic. There will also be paired PDRs/QoS Rules – an L4S variant (higher precedence) and a non-L4S variant. PDRs are also needed to request the reporting

Observation 2: Paired PDRs/QoS Rules with appropriate rule precedence can help to isolate L4S traffic and non-L4S traffic to paired QoS Flows.

Figure below shows the rule definition for the case two QoS Flows are used for the PDU Session; one is established dynamically for L4S traffic when L4S traffic is detected and reported by UPF to SMF using solicited application reporting as described in 5.4.11 of 29.244.

Figure 2.2-1 Precedence of PDR Rules for L4S and non-L4S traffic

In the figure above, QoS Rule#1 /PDR1 correspond to the default match-all traffic and it is bound to a QoS Flow#1 with no L4S enabled (the QoS Flow for the default rule). For dynamic detection of L4S traffic, SMF also installs a PDR2 in UPF which takes precedence over PDR1. It includes an Application Id corresponding to a packet filter definition in UPF that will check ECT(1) /CE bits to detect L4S traffic. The PDR2 maps the traffic to same QoS Flow associated with PDR1 (i.e. QoS Flow#1). It contains a URR to request to UPF to report the start of traffic matching PDR2 (i.e. L4S traffic in QoS Flow#1) to SMF. Upon L4S traffic detection, SMF establishes QoS Flow#2 with similar QoS Characteristics as the QoS Flow #1 but L4S enabled. SMF provisions a new QoS Rule#2/ PDR2’ which takes precedence over QoS Rule#1/PDR2, have ECT(1) /CE bits present in the packet filter and map the traffic to the established QoS Flow#2. Towards UPF, SMF may modify PDR2 to forward the traffic to QoS Flow#2 instead of installing a PDR2’ that takes precedence.

In another example,

Figure 2.2-2 Precedence of PDR Rules for L4S and non-L4S traffic with application differentiation

In the figure above, QoSRule#1/PDR1 was derived from PCC Rule #1 and it is bound to a QoS Flow#1 with no L4S enabled. For dynamic detection of L4S traffic, SMF also installs a PDR3 which takes precedence and  includes an Application Id corresponding to similar packet filter components in UPF as PDR1 but additionally ECT(1) /CE bits are also present. PDR3 contains a URR to request to UPF to report the start of traffic matching the PDR3 (i.e. L4S traffic in QoS Flow#1) to SMF. The PDR3 maps the traffic to same QoS Flow associated with PDR1 (i.e. QoS Flow#1).

Upon L4S traffic detection, SMF establishes QoS Flow#3 with similar QoS Characteristics as the QoS Flow #1 but L4S enabled. SMF provisions a new QoS Rule#3/PDR3’ which takes precedence . PDR3’ includes the Application Id corresponding to similar packet filter components in UPF as PDR1 but additionally ECT(1) /CE bits are also present to map the traffic to the established QoS Flow#3. SMF may also modify PDR3 to forward the traffic to QoS Flow#3 instead of installing a PDR3’ that takes precedence. Packet filter in UPF L4S traffic detection Usage Report can be used for QoS Rule#3.

Similarly, QoSRule#2/ PDR2 was derived from PCC Rule#2 (for traffic matching a different traffic descriptor and with a different set of QoS characteristics) and mapped to QoS Flow#2 (in figure, PDR2 has a lower precedence than PDR#1). SMF configures UPF to report any L4S traffic detected on this flow by means of PDR #4. When L4S traffic is detected in that QoS Flow, SMF installs a new PDR (PDR#4’ in the figure, which takes precedence and includes an application Id corresponding to similar packet filter components in UPF as PDR2 but additionally ECT(1) /CE bits) that is associated L4S enabled QoS Flow (QoS Flow#4) for L4S traffic (or as an alternative modifies PDR#4 to be associated now to QoS Flow#4). Packet filter in UPF L4S traffic detection Usage Report can be used for QoS Rule#4.

It can also be seen that, an SMF may also create the L4S/non-L4S pairs of PDRs /QoS Flows statically without waiting for L4S traffic detection. The drawback with that approach is the creation of extra QoS Flows if certain QoS flow is not going to see any L4S traffic.

Observation 3: It is also possible to configure a L4S capable QoS Flow paired with the QoS Flow associated with the match-all PDR/QoS Rule.

# 2. Conclusion

As it has been shown, the Rel-18 3GPP functionality suffices to support in SMF for a PCC Rule, dynamic establishment of L4S capable QoS Flow triggered by detection of the L4S traffic (e.g. match packets with ECT(1) or CE (See RFC 9331 [160]) by UPF.