3GPP SA WG2 Meeting #169	S2-2505043

Fukuoka, Japan, May 19 – 23, 2025	(revision of S2-25xxxxx)

Source: 	LG Electronics

Title: 	KI#1&2, NewSol: Authorization to support sensing service request from UE and sensing result exposure to UE

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This paper proposes a new solution for KI#1 & KI#2 in TR 23.700-14.

1	Discussion

This paper introduces a new solution related to

-	KI#1: System Architecture to Support Sensing

-	KI#2: Authorization and Revocation to Support Sensing Service

-	In particular, aspect for authorization of sensing service requests and revocation of an on-going sensing service

2	Text Proposal

It is proposed to agree the following changes into TR 23.700-14v0.1.0.

* * * * Start of 1st Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[x]	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication".

* * * * Start of Next Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> |
| #Z | X | X |

* * * * Start of Next Change * * * *

!! All New Texts !!

6.Z	Solution #Z: Authorization to support sensing service request from UE and sensing result exposure to UE

6.Z.1	Description

The UE can request the core network to provide sensing result. Clause 5.23 "Use case on AMR collision avoidance in smart factories" of TR 22.837 [x] describes that the mobile system provides the sensing result to AMR (Autonomous Mobile Robot).

This solution proposes the following for UE to request the core network to provide sensing result and for the core network to provide sensing result to the UE:

1)	New NF called Sensing Support Function (SSF) is introduced to support sensing as illustrated in Figure 6.Z.1-1.

2)	The UE and the SSF communicate and interact each other via user plane for sensing service request and sensing result exposure.

3)	For a PDU Session to connect to the SSF, dedicated S-NSSAI and DNN are used.

4)	The UE is authorized based on its subscription data which includes the dedicated S-NSSAI for Sensing Service.

-	The authorization of the UE to establish connectivity with the SSF for Sensing Service is based on whether dedicated S-NSSAI for Sensing Service is received as part of Allowed NSSAI following existing procedures as described in clause 5.15 of TS 23.501 [3].

-	To revoke the authorization of UE's Sensing Service, the core network removes the dedicated S-NSSAI for Sensing Service from the UE subscription data. Then, the dedicated S-NSSAI for Sensing Service is removed from Allowed NSSAI.

5)	Address of the SSF (e.g. based on an FQDN) can be configured in the UE or provided by the core network (e.g. via Registration procedure, via Sensing Service related policy provisioning).

Figure 6.Z.1-1: System architecture to support Sensing Service

6.Z.2	Procedures

Figure 6.Z.2-1: Procedure for authorization to support sensing service request from UE and sensing result exposure to UE

1.	The UE sends a Registration Request message to the AMF.

2.	The AMF obtains UE subscription data from the UDM. The UE subscription data includes the dedicated S-NSSAI and DNN for Sensing Service.

3.	The AMF sends a Registration Accept message to the UE. The Registration Accept message includes the dedicated S-NSSAI for Sensing Service as part of Allowed NSSAI.

4.	The UE establishes a PDU Session for Sensing Service by using the dedicated S-NSSAI and DNN for Sensing Service based on the URSP.

5.	The UE establishes a connection for Sensing Service with the SSF over user plane via the PDU Session established in step 4.

NOTE:	The SSF can retrieve Sensing Service subscription data from the UDM to support Sensing Service for the UE. This aspect and details of steps 6 to 9 are not covered by this solution.

6.	The UE sends a Sensing Service Request message to the SSF to obtain sensing result.

7.	The SSF collects sensing data and the associated information.

8.	The SSF determines sensing result.

9.	The SSF provides sensing result to the UE.

6.Z.3	Impacts on Services, Entities and Interfaces

SSF (New NF):

-	Supports Sensing Service and the related operations.

-	Supports to handle sensing service request from UE and sensing result exposure to UE.

UE:

-	Supports for sensing service request to SSF and sensing result from SSF over user plane.

UDM:

-	Supports dedicated S-NSSAI and DNN for Sensing Service in UE subscription data.

* * * * End of Changes * * * *