3GPP TSG-SA WG2 Meeting #169	S2-2505075

Fukuoka, Japan, May 19 – 23, 2025

Source:	Samsung

Title:	Inventory Procedure Update

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The contribution proposes to update Inventory Procedure for AIoT.

1. Discussion

Inventory procedure needs to be described in the normative text in TS 23.369 based on LS from RAN3 (R3-252481) regarding the following editor’s note.

Editor's note:	Whether the Inventory Request sent to NG-RAN includes indication about whether there will be a follow up command or not needs to be determined.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369.

* * * Start of Change * * * *

6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory(AF ID, [External Target Area information], information about the target AIoT Device(s), [Approximate number of AIoT Devices]) service operation request to the NEF.

The External Target Area information is specified in clause 5.3.

Information about the target AIoT Device(s) may include Filtering Information, as described in clause 5.8, or include complete AIoT Device Identifier(s).

The approximate number of AIoT Devices, if provided, is used to determine the number of AIoT Devices expected to respond to this inventory service operation, which is sent by AIOTF to the NG-RAN in the assistance information for NG-RAN in step 7 for proper radio resource allocation.

Editor's note:	The parameters for the inventory service operation need further definition.

2.	The NEF may further authorize the AF request as specified in clause 5.6.

The NEF determines the Target Area information, and selects one or multiple AIOTF(s) to handle the request as specified in clause 5.3.1.

3.	The NEF invokes the Naiotf_AIoT_Inventory(AF ID, [Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices]) service operation towards to the selected AIOTF(s).

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. The AIOTF may perform authorization as specified in clause 5.6. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a Correlation ID corresponding to this AF service operation request.

The AIoT Identification Information to be provided to NG-RAN can include Filtering Information, as defined in clause 5.8, or a single AIoT Device Identifier.

AIOTF performs Reader Selection, see clause 5.3.3.

The AIOTF may also use the last serving Reader to assist with determining which Readers to use for an AFs request targeting for a specific AIoT Device.

The AIOTF determines assistance information as described in clause 5.4.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT Inventory service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT Inventory service operation request as specified in clause 8.3.

7.	The AIOTF sends the Inventory Request message including the Correlation ID, follow-on command indication  when there will be a follow up command, the AIoT Identification Information to be included in the paging message, and assistance information to the selected NG-RAN.

8.	The NG-RAN sends an Inventory Response to the AIOTF with the Correlation ID indicating that the Inventory Request is received successfully and will perform the service operation accordingly.

9.	Upon reception of the Inventory Request message from the AIOTF, the RAN Reader(s) will execute the inventory operation. The RAN Reader(s) broadcast the paging message that includes the AIoT Identification Information.

The AIoT Device determines whether it matches the AIoT Identification Information, as described in clause 5.8.

If an AIoT device matches the AIoT Identification Information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its AIoT Device ID.

Editor's note:	Whether and how the Device ID is concealed or encrypted will be determined and aligned with SA WG3.

10.	NG-RAN sends one or more Inventory Report messages to the AIOTF including the Correlation ID, Reader ID, RAN NGAP Device ID and the AIOT NAS message(s) from the AIoT Device(s). The AIOTF stores the mapping between the Reader ID and AIoT Device ID(s).

NOTE:	When to erase the stored mapping between the Reader ID and AIoT device ID(s) is up to implementation and local configuration.

11.	The AIOTF validates the results, using local stored device information or device profile data retrieved from the ADM. The AIOTF may aggregate the results.

12.	Optionally, if the NG-RAN detects that no more AIoT Devices will respond to the inventory procedure, the NG-RAN informs the AIOTF that the procedure is complete.

Editor's note:	The details how interactions between NG-RAN and AIOTF to indicate no further Inventory Reports will be sent and completion of the procedure need to be aligned with RAN.

13.	The AIOTF reports the progress of the AIoT inventory request to the NEF by sending the Naiotf_AIoT_Notify message including a list of AIoT Device Permanent Identifier (s). The AIOTF may send multiple reports. The AIOTF in the final Naiotf_AIoT_Notify message indicates it is the last report for this operation. If multiple AIOTFs are involved in the procedure, the NEF may receive the AIoT_Notify from multiple AIOTFs.

14.	When receiving the Naiotf_AIoT_Notify message from AIOTF, the NEF informs the AF of the outcome of the AIoT_Inventory request by sending the Nnef_AIoT_Notify message(s) including the AIoT Device Permanent Identifier(s). The NEF in the final Nnef_AIoT_Notify message indicates that it is the last report for this operation.

* * * End of Change * * * *