3GPP TSG|WG-SA2 Meeting #169 	S2-2505099

Fukuoka, Japan, 19 – 23 May 2025	(revision of xx-yyxxxx)

# Source:	Huawei, HiSilicon

# Title:	New WID on IP Address Range Allocation

# Document for:	Approval

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	IP Address Range Allocation

Acronym:	TEI20_IPARA

Unique identifier:

Potential target Release:	Rel-20

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification:

3	Justification

In Rel-19, UE initiates a PDU session modification procedure to associate the Non-3GPP Device Identifier behind UE with user plane address information, specifically the UE IP address combined with port ranges, to request for differentiated QoS of traffic for Non-3GPP Device Identifier. In case of IPv4, which is still widely used, in order to identify the traffic related to a specific Device and more than one application may run on the same device, multiple ports or port ranges have to be allocated to identify the Device, considering that each application needs at least one dedicated port. Always relying on multiple ports or port ranges to identify a Device is not flexible and introduces additional overhead in the signalling, especially from the UE to the network. It should be noted that these ports or port ranges have to be reserved even the applications may not always be used by the Device. To address this issue, it is proposed to enhance PDU Session establishment procedure to enable the network to send a range of IP addresses in case of IPv4 for the non-3GPP devices behind the UE.

In addition, Framed Routing is defined in 3GPP to support an IP network behind a UE, where a Framed Route information can be provided to the SMF by a DN-AAA server or UDM, which contains a range of IPv4 addresses or a range of IPv6 prefixes. However, so far the network does not send Framed Route information to the UE, and how the devices behind the UE gets the IP addresses is not defined by 3GPP specifications. One possibility might be that the UE requests the IP address from the DN-AAA through DHCP messages, which would introduce much signalling from the UE to the network for IP address request for each device and potentially security issues, e.g. DHCP spoofing. So it is beneficial to enable the network to send the Framed Route information to the UE through the NAS message.

4	Objective

The objective of this WID is to specify the support of IP range allocation for UE:

- Enable the SMF to allocate and send the range of the IPv4 address to the UE via NAS for the Non-3GPP Device Identifier(s) behind the UE during PDU Session Establishment procedure;

- Enable the SMF to send the Framed Route information to the UE via NAS for enterprise connectivity.

Note: This work will require 0.5 TU.

5	Expected Output and Time scale

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Specify that the network can send the range of IPv4 addresses and Framed Route information to the UE via NAS; | SA#110 (Dec 2025) |  |
| TS 23.502 | Update the PDU Session Establishment procedure to enable the SMF to send the range of IPv4 addresses and Framed Route information to the UE via NAS. | SA#110 (Dec 2025) |  |

6	Work item Rapporteur(s)

{Mandatory: <FamilyName>, <GivenName>, <Company>, <email address>}

1

7	Work item leadership

SA2

8	Aspects that involve other WGs

None identified.

9	Supporting Individual Members

Supporting IM name

Huawei

HiSilicon