TSG SA WG2 Meeting #169	S2-2505137

19 - 23 May, 2025, Fukuoka, Japan

Source:	Lenovo

Title:	Solution for KI#2: Authorization and Revocation of sensing service request

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing-ARC / Rel-20

Abstract of the contribution: Proposes new solution for service authorization and revocation

# 1. Introduction

This pCR proposes the solution for Key issue#2 Authorization and Revocation of sensing service request.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X |  | X |  |  |  |  |

## 6.X	Solution #X: < Authorization and revocation of sensing service request >

### 6.X.0	High-level solution Principles

The high-level principles for authorization and revocation of sensing service request are:

- NEF performs AF request authorization for sensing service based on SLA

- SF performs authorization of sensing service requested by the AF by checking the sensing requirements

- AF may request to revoke the sensing service to NEF and NEF informs SF to revoke the sensing service

### 6.X.1	Description

Subject to regulation and operator’s policies, the 5G network shall be able to authorize or revoke authorization of sensing service requested by the sensing consumer. Sensing service authorization mainly includes the following two parts:

- NEF performs AF request authorization for sensing service based on the SLA between the 3rd party AF and the 5GS of the mobile network operator, the operator policy and local configuration as in TS 33.501 [2].

- SF performs authorization of sensing service requested by the AF, by checking the sensing requirements, e.g., whether the specific sensing requirement can be fulfilled.

NOTE 1: only 3rd Application server is considered as the sensing service consumer in Rel-20.

AF may request to revoke the sensing service to NEF and NEF informs SF to revoke the sensing service.

### 6.X.2	Procedures

6.X.2.1	Procedures for sensing service authorization

Figure 6.X.2.1-1: Sensing service authorization procedure.

- Sensing service consumer (i.e., AF) sends Nnef_SensingService_Request message to the NEF, which includes sensing task ID, sensing requirement and sensing exposure requirement etc. Sensing requirement may include sensing accuracy, sensing resolution, confidence level, missed detection probability, false alarm probability, max sensing service latency, refreshing rate etc. Sensing exposure requirement may include notify method (e.g., one time, periodic or event trigger), reporting periodicity, waiting time etc.

- Upon receiving request from AF, the NEF authorizes the AF for request of the Sensing Service.

- The NEF performs sensing function discovery and selection.

NOTE 1: details of sensing function discovery and selection will be addressed by the solution of Key Issue#3.

- The NEF sends Nsf_SensingService_Request message to the selected SF, which includes sensing task ID, sensing requirement and sensing exposure requirement. NEF preserves the mapping of sensing task ID and SF.

- The SF performs authorization of sensing service requested by the AF, by checking the sensing requirements, e.g., whether the specific sensing requirement can be fulfilled.

- The SF sends Nsf_SensingService_Response message to the NEF, which includes sensing task ID and the result of authorization (e.g., accepted or rejected).

- The NEF sends Nnef_SensingService_Response message to the Sensing service consumer (i.e., AF), which includes sensing task ID and the result of authorization (e.g., accepted or rejected).

6.X.2.2	Procedures for sensing service revocation

Figure 6.X.2.2-1: Sensing service authorization procedure.

1. 	Sensing service consumer (i.e., AF) sends Nnef_SensingService_Revoke request message to the NEF, which includes the sensing task ID.

2.  The NEF sends Nsf_SensingService_Revoke request message to the corresponding SF based on the sensing task ID, which includes the sensing task ID.

3.  The SF sends Nsf_SensingService_Revoke response message to the NEF, which includes the sensing task ID and result.

4.  The NEF sends Nnef_SensingService_Revoke response message to the AF, which includes the sensing task ID and result.

### 6.X.3	Impacts on services, entities and interfaces

NEF:

- Support Sensing Function discovery and selection

- Interaction with AF and SF for sensing service authorization and revocation

Sensing Function:

- Interaction with NEF for sensing service authorization and revocation

AF:

- Interaction with 5GC for sensing service request and revocation

# * * * * End of changes * * * *