3GPP TSG-SA WG2 Meeting #169	S2-2505271

Fukuoka, Japan, 19 – 23 May 2025

Source:	ETRI

Title:	New solution for KI#1: New sensing NFs and functionalities

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This contribution proposes a solution for KI#1 with introducing new NFs and functionalities for sensing.

# 1. Discussion

This contribution addresses the work tasks identified in KI#1 of TR 23.700-14 by proposing new NFs and functionalities for sensing as summarized as follows:

- New NFs

  - SeMF: Sensing Management Function

  - SePF: Sensing Provisioning Function

- NF Roles

  - SeMF

    - Sensing entity management

      - registration, discovery/selection, configuration

    - Sensing data management

      - collection, processing, storage, content

  - SePF

    - Sensing service provisioning

      - authorization

      - policy, QoS enforcement

      - SeMF discovery/selection, sensing request

    - Sensing result exposure

      - content

      - type: one-time, periodic, event

- Key functionalities

  - Sensing entity management (SeMF)

    - registration

      - gNB/UE registers itself to SeMF via N2/N1

      - with sensing capability (examples below)

        - coverage: TA, Cell ID

        - role: transmitter or receiver or both

        - QoS: range, resolution, accuracy, latency

        - resource: energy

        - feature: pre-processing

      - serving SeMF is discovered/selected by AMF with TA, Cell ID

      - support of UE (as a sensing entity) mobility is FFS

    - discovery/selection

      - SeMF discovers/selects sensing entities

      - based on the sensing capabilities to meet the sensing request from SePF

    - configuration (as per KI#6, FFS)

      - SeMF configures sensing entnties (parameter provisioning)

      - based on the sensing request from SePF, or operator's policy

  - Sensing data management (SeMF)

    - collection

      - SeMF requests/collects sensing data to/from sensing entities

      - data transport:

        - CP or UP

    - processing/reply

      - SeMF generates sensing result from sensing data collected, and reply the result to the requestor (SePF or other NFs)

      - with contextual information

    - storage

      - SeMF manages sensing data collected in its own stoage

      - for re-use with different TTLs

      - for analytics by NWDAF (FFS)

  - Sensing service provisioning (SePF)

    - Authorization

      - SePF authorizes sensing service (request) via UDM, PCF

    - policy, QoS enforcement

      - SePF enforces QoS, policies on the sensing service (request) via PCF

    - SeMF discovery/selection

      - SePF discovers/selects SeMF via NRF

      - with sensing capabilities: TA, Cell IDs, etc.

    - sensing request triggering

      - SePF receives sensing request from AF via NEF

        - content: type (object detection, tracking, environment monitoring, etc.), target (object, area), QoS (accuracy, resolution, latency), assistance information (object information, area information)

      - SePF requests sensing results to the selected SeMF

      - with target information, assistance information from AF

  - Sensing result exposure (SePF)

    - SePF reply AF with sensing result

      - content: object type, location, velocity, direction, etc.

    - with contextual information

    - type: one-time, periodic, event

# 2. Proposal

It is proposed to add the following text to TR 23.700-14.

* * * Start of Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #y> |
| #1 |  |  |
| #2 |  |  |
| #x | O |  |

* * * Next Change * * * *

6.X	Solution #X: New NFs and functionalities for sensing

6.X.1	Description

This solution addresses Key Issue #1 by proposing a system architecture for supporting Sensing capabilities within the 5G System. The architecture introduces two new Network Functions (NFs): the Sensing Management Function (SeMF) and the Sensing Provisioning Function (SePF).

The SeMF (Sensing Management Function) is responsible for the core functionalities related to managing sensing resources and handling sensing data. Its key roles include:

- Sensing Entity Management:

This involves the registration of sensing-capable entities (gNBs or UEs or both), their discovery and selection for specific requests (as per KI#3), and their configuration (parameter provisioning, as per KI#6). Sensing entities register their capabilities including coverage (e.g., TA, Cell ID), role (e.g., transmitter, receiver, or both), QoS parameters (e.g., range, resolution, accuracy, latency), resource information (e.g., energy consumption), and features (e.g., pre-processing). The serving SeMF for a gNB/UE may be discovered/selected by the AMF (e.g., based on TA/Cell ID).

- Sensing Data Management:

This covers the collection of sensing data from sensing entities (e.g., via N1/N2 or N3), processing the collected data to generate sensing results (as per KI#4), and storing the sensing data for potential re-use or analytics (which is FFS)

The SePF (Sensing Provisioning Function) manages sensing services requested by 3rd-parties (e.g., AFs) via NEF. Its primary roles are related to provisioning and exposing sensing capabilities. Its key functionalities include:

- Sensing Service Provisioning:

This involves authorizing sensing service requests (as per KI#2), enforcing policies and QoS requirements, discovering and selecting the appropriate SeMF(s) to handle a request (e.g., via NRF, using sensing capabilities like TA), and triggering the sensing request to the selected SeMF(s) (as per KI#3). Mapping between geographical areas (specified in AF requests) and network-internal identifiers like TAs is needed (FFS, SePF/NEF involvement). Handling multiple SeMF selections for requests spanning multiple TAs is FFS.

- Sensing Result Exposure:

This involves receiving sensing results from SeMF(s) and exposing them to the requesting entity (e.g., AF via NEF, as per KI#5). The content of exposed results may include, for example, object type, location, velocity, direction, etc., but FFS. Results may be provided as one-time reports, periodically, or based on events.

6.X.2	Procedures

Figure 6.X.2-1: Overall procedure for handling sensing service request

A high-level procedure for a sensing service request may involve the following steps:

- (Assimption) Sensing entities are already registered to SeMF with their sensing capabilities, which may be covered in KI#3.

- An Application Function (AF) initiates a sensing service request towards the 5G System via NEF. The request includes details such as the sensing type (e.g., object detection, tracking, environment monitoring, etc.), target (e.g., object, area), required QoS (e.g., accuracy, resolution, latency), and potentially assistance information (e.g., object/area details).

- The NEF forwards the sensing request to the SePF.

- The SePF performs authorization and policy/QoS enforcement based on the request details and subscriber/service profile information (e.g., interacting with UDM and PCF).

- Based on the target area/object and required capabilities specified in the request, the SePF discovers and selects one or more suitable SeMFs. This involves mapping the geographical area (if specified) to network identifiers (e.g., TAs) and querying the NRF for SeMFs matching the area coverage and required sensing capabilities.

- The SePF triggers the sensing request towards the selected SeMF(s), including the relevant target information and assistance information derived from the AF request. If multiple SeMFs are selected, the SePF may need to coordinate the request distribution.

- Upon receiving the sensing request, the SeMF discovers and selects the specific sensing entities (gNBs/UEs) under its management that are capable of fulfilling the request for the specified target area/object and QoS.

- The SeMF configures the selected sensing entities with necessary parameters for the sensing request.

- The SeMF requests and collects sensing data from the configured sensing entities. Transport for the sensing data collection may occur over CP (N1/N2) or UP (N3).

- The SeMF processes the collected sensing data to generate the sensing results. The collected sensing data may be stored by the SeMF for further use in the following requests.

- The SeMF reports the sensing results, potentially with contextual information, back to the requesting SePF.

- The SePF receives sensing results from the SeMF and exposes the sensing result to the requesting AF via NEF.

6.X.3	Impacts on Services, Entities and Interfaces

The following NFs and interfaces are impacted:

SeMF:

-	a new network function for sensing entitiy management and sensing data management

-	new services and interfaces for sensing entity registration, sensing data collection, and sensing request

SePF:

-	a new network function for sensing service provisioning and sensing result exposure

-	new services and interfaces for sensing service request (including subscription)

NEF:

-	new services and interfaces for sensing service request (including subscription) from AF

NRF:

-	updates to NF profile management for discovery of SeMF and SePF

* * * End of Change * * * *