3GPP TSG-SA WG2 Meeting #169	S2-2505272

Fukuoka, Japan, 19 – 23 May 2025

Source:	ETRI

Title:	New solution for KI#3: Discovery and selection of sensing entities and functions

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This contribution proposes a solution for KI#3 with introducing the sensing entity management functionality for discovery and selection of sensing entities; and sensing function discovery via NRF.

# 1. Discussion

This contribution addresses the work tasks identified in KI#3 of TR 23.700-14 by proposing a new functionality for sensing entity management for discovery and selection; and how to manage the sensing functions for discovery via NRF as summarized as follows:

- New NFs (Architectural assumption as proposed in S2-2505271)

  - SeMF: Sensing Management Function

  - SePF: Sensing Provisioning Function

- Key functionalities

  - Sensing entity management (by SeMF)

    - registration

      - gNB/UE registers itself to SeMF via N2/N1

      - with sensing capability (examples as below)

        - coverage: TA, Cell IDs

        - role: transmitter or receiver or both

        - QoS: range, resolution, accuracy, latency

        - resource: energy

        - feature: pre-processing

      - serving SeMF is discovered/selected by AMF with TA, Cell ID for initial assignment during sensing entity registration.

      - support of UE (as a sensing entity) mobility is FFS

    - discovery/selection of sensing entities

      - SeMF discovers/selects sensing entities under its management

      - based on the sensing capabilities to meet the sensing request from SePF or other CN NFs

  - Sensing function discovery and selection (via NRF)

    - registration

      - SeMF instances register their NF profiles and capabilities with the NRF

      - With SePF capability (examples as below)

        - sensing coverage (list of TAs, Cell IDs)

        - supported sensing types, performance metrics, etc.

    - discovery/selection of sensing functions

      - SePF discovers and selects SeMF instances via NRF based on sensing request criteria, including target area (mapped to TA/Cell ID) and required SeMF capabilities

      - AMF discovers and selects initial serving SeMF for sensing entities via NRF based on the sensing coverage information (TA/Cell ID)

# 2. Proposal

It is proposed to add the following text to TR 23.700-14.

* * * Start of Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #3> | <Key Issue #y> |
| #1 |  |  |
| #2 |  |  |
| #x | O |  |

* * * Next Change * * * *

6.X	Solution #X: Discovery and selection of sensing entities and functions

6.X.1	Description

This solution addresses Key Issue #3 related to the discovery and (re-)selection of sensing entities (i.e., gNBs and/or UEs) and sensing functions. With the architectural assumption with SeMF (Sensing Management Function) and SePF (Sensing Provisioning Function) as per KI#1, this solution defines how sensing entities make their capabilities known to the network and how the SeMF discovers and selects them to fulfill sensing service requests. It also includes how the SeMF can be discovered by SePF or any other CN NFs via NRF.

For discovery and selection of sensing functions, the sensing function instances (i.e., SeMF) make their capabilities and service areas known by registering with NRF. Their capabilities include the sensing coverage (e.g., defined by lists of TAs or Cell IDs), supported sensing types, performance metrics, etc. Other NFs such as SePF for processing a sensing service request; or AMF for handling the signaling from sensing entities, query NRF based on the capabilties as defined above, especially with the sensing coverage relevant to the sensing request or sensing entities.

For discovery and selection of sensing entities, sensing entities register their specific sensing capabilities (e.g., the specific coverage area or location where they can perform sensing, supported modes, accuracy, range, processing features, role, etc.) with their serving SeMF. When an SeMF receives a sensing service request (from SePF or other CN NFs), it uses its internal database of registered sensing entity capabilities and status information to discover and select the most suitable sensing entities under its management to perform the sensing request.

This management includes handling updates to entity capabilities and status, re-selecting entities during an ongoing sensing task if needed. Due to mobility of sensing entities (i.e., UEs), re-selection of sensing entities may be also triggered but is FFS.

6.X.2	Procedures

Figure 6.X.2-1: Procedure for discovery/selection of sensing entities and functions

The discovery and selection processes for both sensing entities and sensing functions are supported by the following steps:

- (SeMF registration) SeMF registers its NF profile with the NRF, including its sensing capabilities (e.g., sensing coverage as TAs/Cell IDs, supported sensing types, performance, etc.).

- (Sensing entity registration) A sensing entity registers to a serving SeMF with its sensing capabilities (e.g., sensing coverage, supported modes, role, performance). This signaling is mediated by the AMF via N1 (NAS) for a UE or N2 (NG-AP) for a gNB to forward the registration request to SeMF which is discovered by AMF via NRF based on the sensing coverage (TA, Cell ID). The SeMF maintains the sensing entity information registered for discovery and selection later.

- (SeMF discovery) Upon receiving a sensing service request (e.g., from AF via NEF) with sensing target and requirements, SePF specifies the target area and mapping to network identifiers (e.g., TA/Cell ID list). It then queries the NRF to find a suitable SeMF based on sensing coverage and required capabilities.

- (Sensing entity discovery) SePF forwards the sensing request to the SeMF discovered. Then the SeMF identifies and selects suitable sensing entities which are already registered based on the sensing entites’ capabilities to meet the sensing request

- (Sensing trigger) The SeMF triggers the selected sensing entities to perform the requested sensing tasks using N1/NAS or N2/NGAP via AMF.

6.X.3	Impacts on Services, Entities and Interfaces

The following NFs and interfaces are impacted:

SeMF:

-	a new network function for sensing entitiy management and sensing data management

-	new services and interfaces for sensing entity registration, sensing data collection, and sensing request

NRF:

-	updates to NF profile management for discovery of SeMF and SePF

gNB/UE:

-	enhancement for registering their sensing capabilities to SeMF (via N1/N2).

Editor’s Note: The scope of RAN WGs.

* * * End of Change * * * *