3GPP TSG-SA WG2 Meeting #169	S2-2505303

Fukuoka, Japan, May 19 – May 23, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6329 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Energy Saving Indicator handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the LS reply from RAN3, there is no consensus in RAN3 to introduce Energy Saving Indicator in signalling from AMF to NG-RAN. So it is proposed to remove the descriptions about sending the Energy Saving Indicator from AMF to NG-RAN in release 19. However, considering that this indicator may help the NG-RAN handling for energy saving, it may be further studied or clarified in the later release. So it is proposed to add a NOTE to clarify this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to remove the descriptions about sending the Energy Saving Indicator from AMF to NG-RAN in release 19. Furthermore, the NOTE is added to clarify that Energy Saving indicator is not sent to NG-RAN in this release. This aspect may be further studied in the later release. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Editor’s note remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.51.5	Subscription information aspects

The Subscription data for a UE may include an Energy Saving Indicator that the UE is subject to network energy saving operation. The values of the Energy Saving Indicator point to energy saving behaviour configured in the AM-PCFs of the PLMN. The possible energy saving behaviours for a PLMN and the mapping of the Energy Saving Indicator values to the respective behaviour are operator-defined and outside the scope of this specification. The AMF receives the Energy Saving indicator in the UE Subscription data. The AMF forwards the Energy Saving indicator to the PCF.

NOTE:	Energy Saving indicator is not forwarded to NG-RAN in this release.

# * * * * End of changes * * * *