3GPP SA WG2# 169	S2-2505317

Fukuoka, Japan, May 19 – 23, 2025	(revision of xx-yyxxxx)

# Source:	Ericsson (Moderator)

# Title:	New SID on Study on Industrial Networks enhancements

# Document for:	Discussion

# Agenda Item:	30.2

Abstract: This document includes all WTs and detailed discussion and can be used as place holder for potential SI creation if agreements can be reached in any of the WTs to conduct study in Rel-20.

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on Industrial Networks enhancements?

{Free text. It has to be the same as in the "Title:" section above. Studies have to start by "Study on"}

Acronym:	FS_IndNwk_enh?

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-20

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

{Tick one or more box(es). The full structure of all existing Work Items is shown in the 3GPP Work Plan in https://ftp.3gpp.org/Information/WORK_PLAN}

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

{"Parent" Work Item refers to the related, earlier-Stage, Work Item, e.g. the related Stage 1 Work Item shall be indicated here when a Stage 2 normative Work Item or Study Item is presented. "Parent" Work Item can also refer to the related preceding Study Item e.g. the related Study Item and the earlier-stage Work Item shall be indicated here when a normative-work Work Items is started. List here all parent Work Items of which requirements are either fully or partially covered by the proposed Item. }

{This section is mandatory to be filled out by the rapporteur. This section is to be filled with care: it indicates to the companies monitoring the parent Work Item that it will be addressed in this study/work item.}

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as a Work Item in an earlier Release if further enhancing the feature from the previous Release)}

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 840050 | Enhancements for cyber-physical control applications in vertical domains | Work Item of Stage 1 requirements? |
| 820017 | Stage 2 of Vertical_LAN | Vertical_LAN? |
| 910059 | Support of Enhanced Industrial IIoT | IIoT? |
| 970011 | Stage 2 of Extensions to the TSC Framework to support DetNet | DetNet? |

Dependency on non-3GPP (draft) specification:

TBD

# 3	Justification

This section will be cleaned up once stable content has been achieved.

Justification for enhancing Hold and Forward buffering mechanism to support de-jittering function:

Latency characteristics of the 5G bridge deviate significantly from those of a wired bridge.

Wired bridge jitter in the range of 100s of nanoseconds.

5G bridge jitter in the range of milliseconds.

Leading to complex TSN scheduling and not scalable with increasing number of TSN streams. 802.1Qbv defines time instances when packets can be released at the egress gate, but it does not enable to de-jitter from the varying transmission delay that packets perceive in their transmission through the 5GS.

With a large jitter (compared to wired bridges) there will be an uncertain queueing and transmission order

within 5GS.

Queueing issues lead to some packets missing the opportunity for transmission in the current cycle, and are

delayed till next cycle –breaking the schedule.

Additionally, for delay critical applications and in general for TSC and DetNet (when IEEE 802.1Qbv is not

used), the large 5G-introduced jitter will prevent the service to guarantee on-time delivery of periodic

traffic as per requirements of the application. The focus is primarily on TSC, since DetNet is based on the TSC framework and could in principle apply this new feature if configured.

Expand the scope to include support of CQF in Annex T in IEEE 802.1Q can be used to improve the issue of large jitter in 5GS TSN bridge.

Justification in order to enable support loop detection and elimination in 5G LAN and enable support of 5GS native FRER. The WTs have been expanded to look at enhancement not restricted to 5G LAN and FRER only, but also to include IP option.

Loop in 5G-LAN: 5G-LAN defines Layer 2 Ethernet networking for 5GS, where Ethernet loop may occur in

the 5G-LAN network, which may result in broadcast storm and further network breakdown. For example,

haphazard adjustment of the device side networking may cause loop, since many end switches do not have the loop detection capability.

FRER can improve the connection availability and reliability, in the case of poor air interface quality or

temporary fault of a single UE module for multiple UE modules case, and reduce the service downtime caused

by transmission timeout. However, native support of FRER by 5GS is not supported, and the 5GS has to rely

on TSN and application level FRER which is hard to deploy.

Additional feedback is that to change the second case FRER) to the following: FRER for Ethernet and PREOF for IP can improve the connection availability and reliability, in the case

of poor air interface quality or temporary fault of a single PDU Session, and reduce the service downtime

caused by transmission timeout. However, native support of TSN FRER and DetNet PREOF by 5GS is

not currently available, and the 5GS has to rely on externally deployed TSN/DetNet and application level

FRER/PREOF which is hard to deploy. Scenarios both with single UE per device or multiple UEs per

device are possible. There are also request to remove multiple devices case and focus on single UE and multiple PDU sessions. Also questions raised what to solve and PDCP Layer solution already exists between RAN/UE. As well that previous attempts to address this has not been successful so may not be best use of time.

Additional feedback on Loop prevention: Loops should be prevented in advance, so we need a proactive solution, not a reactive one and as such adjust justification and WT accordingly. And to expand the scope significantly to include as per Siemens-WT#5 with its subtasks: Support of Multiple Spanning Tree Protocol (MSTP) for 5G System operating as logical bridge in integrated IEEE/3GPP networks.

In 2023 and 2024 5G-ACIA investigated potential gaps of 5GS when it acts as one or more IEEE 802.1Q bridges in an Industrial Automation network and identified the missing MSTP functionality as a critical issue, especially for non-stream IA traffic. (LS from 5G-ACIA “5G-ACIA-LS-2024-008” to SA and SA2)

Further, SA1 specified in TS 22.261 Rel-16 the requirement to support STP. This requirement was modified in Rel-17 to support RSTP, and in Rel-19 to support MSTP as defined in IEC/IEEE 60802

Justification for Enhancements on 5G System as Logical Bridge in integrated IEEE/3GPP networks

There is the need for 5G networks to be integrated into IEEE 802.1-based industrial networks. This includes the cases where the 5G system as logical bridge acts as a wireless link between the industrial backbone networks and mobile assets (e.g. robots and AGVs), or the cases where the 5GS logical bridge is connecting local industrial network segments as a wireless backbone. 5G-ACIA has addressed the required enhancements of 5GS Bridge model for flexible integration with IEEE 802.1-based networks and the industrial automation profile IEC/IEEE 60802 in the liaison SP-241819 (SA#106 plenary).

5GS Bridge that is integrated into an ethernet network may be part of a loop (e.g. when the 5GS Bridge resides between the ethernet switches that are part of a physical redundant path). The current specification has not addressed how the loop prevention operations of MSTP/RSTP can be supported on 5GS Bridge. It is desired to study the 5GS support for BPDU forwarding and processing, MSTP/RSTP configuration, and mappings of MSTP configurations with 5G VN Group(s). The above enhancements for 5GS Bridge model integrated with general IEEE 802.1 industrial communication (for non-time critical or legacy traffic) need to be studied.

Configuration by Netconf/YANG is mandated by IEC/IEEE 60802 "Time-Sensitive Networking Profile for Industrial Automation". For 5GS TSN Bridge, the support for CNC/CUC YANG modules, features and nodes needs to align with the current 5G TSN architecture and industrial automation profile. The more general architectural support of 5GS Bridge for Netconf/YANG configurations (for e.g. VLAN, MSTP, LLDP), and the corresponding enhancements on 5GS exposure need to be studied.

The IEC/IEEE 60802 has defined eight traffic classes and the mapping of traffic types to priorities and traffic classes for industrial automation. For some traffic types like Cyclic asynchronous streams, no PSFP information is derived from the CNC hence there are not enough parameters available to setup the appropriate QoS flow for such a traffic type. Study is needed on how the industrial automation IA traffic types as specified in IEC/IEEE 60802 can be supported without the PSFP configuration provided.

Enhancements on 5GS Bridge should address required LLDP extensions for supporting further IA protocols (e. g. IEC/IEEE 60802, OPC-UA).

Justification for avoiding interference by configuration domains using priority values

Frames that are received from a different configuration domain may use priority values that are reserved for TSN streams. To avoid interference with the traffic (especially TSN streams) in the configuration domain such priorities need to be downgraded as described in IEEE Std 802.1Q-2022, Annex U.2.

To clarify further on this, the changes are expected to be confined to extending the PMIC and UMIC from the TSN AF. Basically there would be frames received from different configuration domains using priority values that are reserved for TSN streams; so to enable avoiding interference with the traffic (e.g., TSN streams) in the configuration domain, such priorities need to be downgraded (described in IEEE Std 802.1Q-2022, Annex U.2) and could use the PMIC and UMIC for the sharing such an info and then carry out the priority re-generation.

Justification for Dynamic VN group based on geographical area

In the transportation industry, there are requirements to manage a dynamic VN  group containing UEs in a specific geographical area, to support group communications between the UEs in the area, e.g. multicast or broadcast of ship/port information between ships around the port, collision avoidance UAV-to-UAVs or vehicle-to-vehicles broadcasting communication within a specified relative distance to convey some information e.g. UAV flight route information, UAV or ship’s current speed and operating status. (Refer to clause 5.2.2 of TS22.125). This requires 3GPP to manage a dynamic VN group and support automatically adding UEs moving in or remove UEs moving out of the area based on UE location.

# 4	Objective

Draft objectives taking into account different views (not consensus):

WT#1: De-jitter for TSN

WT# 1.1 Study whether and how to extend the Hold and Forward buffering mechanism with a de-jittering functionality for TSN.

WT# 1.2 Study whether the support of CQF in Annex T in IEEE 802.1Q can be used to improve the issue of large jitter in 5GS TSN bridge.

WT#2: De-jitter for TSC/DetNet

WT#2.1: Study how to extend the Hold and Forward buffering mechanism with a de-jittering functionality for TSC.

WT#2.2: Study how to extend the Hold and Forward buffering mechanism with a de-jittering functionality for DetNet.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1.1 | 0.5 | 0.5 | No | self-contained |
| WT#1.2 (DCM) | ? | ? | No | ?? |
| WT#2.1 | 0.5 | 0.5 | No | self-contained |
| WT#2.2 | 0.5 | 0.5 | No | self-contained |

WT#3: Loop Prevention  (To be merged with Siemens-WT#1)

Alt 1: Study how to provide loop prevention functionality (i.e., MSTP/RSTP defined in IEC/IEEE 60802) when the 5G system is integrated in industrial Ethernet network as a bridge.

Alt 2:

- WT-3.1: How to define support of the Multiple Spanning Tree Protocol in the UPF for 5G VN scenarios to ensure loop prevention and enhancements for its activation/deactivation.

- WT-3.2: How to define support of Multiple Spanning Tree Protocol or Rapid Spanning Tree Protocol, defined in IEC/IEEE 60802, in UPF in Ethernet PDU Sessions which are not related to 5G VN.

Alt 3:

Support of Multiple Spanning Tree Protocol (MSTP) for 5G System operating as logical bridge in integrated IEEE/3GPP networks, including:

- Evaluate the topologies that create a loop

- Architectural enhancements of 5GS that supports MSTP operations (e.g. BPDU forwarding)

- How 5GS is configured for MSTP communication

MSTP (RSTP) works on physical topology that is below the active logical topology (5G-LAN), the “Support of MSTP for 5G VN group(s)” is based on aspects listed above.

NOTE: The solution should be compatible with MSTP as required by IEC/IEEE 60802 (RSTP + traffic engineering).

WT#4: Packet Replication & Elimination

Alt 1: Study how packet replication and elimination can be done within 5G system in order to establish redundant connection between UE and UPF; considering two PDU sessions for a device.

Alt 2: 3GPP entities natively support duplication of the packet data or elimination of redundant transmissions, re-using functionality of IEEE FRER or IETF DetNet where applicable. The replication procedure is controlled by the 3GPP system, and performed for two different PDU Sessions which may originate from a single device.

Contentious Issue: Whether it is for single UE, or this should be open to redundancy from Multiple UEs.

Proposed TU: 1.5 + 1.5 ?

Added WTs:

Siemens-WT#1: Support of Multiple Spanning Tree Protocol (MSTP) for 5G System operating as logical bridge in integrated IEEE/3GPP networks, including:

- Evaluate the topologies that create a loop;

- Architectural enhancements of 5GS that supports MSTP operations (e.g. BPDU forwarding);

- How 5GS is configured for MSTP communication.

Siemens-WT#2: Support of Netconf/YANG configuration profiled by IEC/IEEE 60802, including:

- Architectural enhancements of 5GS that supports Netconf/YANG configuration;

- How to translate IEC/IEEE 60802 configurations (information and data model) to the 5G VN-related and TSN-related configurations within 5GS;

- Enhancements of 5GS exposure to support Netconf/YANG configuration profiled by IEC/IEEE 60802.

Siemens-WT#3: Enhancements for 5GS bridge in integrated IEEE 802.1 industrial communication networks, including:

- Support of industrial automation (IA) traffic types specified in IEC/IEEE 60802, for setup of QoS flows for streams without PSFP configuration;

- Support for extensions for LLDP required by IEC/IEEE 60802.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| Siemens-WT#1 | 2 | 2 | No | self-contained |
| Simens-WT#2 | 2 | 2 | No | self-contained |
| Siemens-WT#3 | 2 | 2 | No | self-contained |

Nokia-WT#1: Study whether and how to implement different configuration domains to perform the priority regeneration according to IEEE Std 802.1Q-2022.

Estimated TU: 0.5 + 0.5 TU.

WT-CMCC-1: study enhancement of 5G LAN on support of location based dynamic group communication, to allow 5GC dynamically managing VN group containing UEs moving in and out a specific area based on UE location information.

WT_CMCC-2：How to support the configuration and management of 5G LAN group information locally.

WT_CMCC-3: How to support the 1:N mapping between DNN/S-NSSAI and 5G LAN groups.

Ericsson view on WT-CMCC 1-3:

Regarding the objective, it is already possible to e.g. use 5G VN with LADN, so you could define a 5G VN group (with a specific DNN/S-NSSAI) that is only available in a certain LADN area (i.e. set of TAIs).

Provisioning LADN services area from an AF was added in GMEC, and it is general so it can be applied also to 5G VN groups, e.g., as stated in 23.501:

“- The LADN service area can be provisioned for a group (e.g. 5G VN group) or an individual subscriber using UDM/NEF parameter provisioning service triggered by AF request as described in clause 4.15.6 of TS 23.502 [3].”

And to dynamically add and remove UEs from the 5G VN group as they move into or out of the area supported by the 5G VN group, not sure why that would be needed. Seems rather complex.

However, even if one wants to do that, couldn’t that be done by an AF monitoring the UE location using the monitoring API, and based on that trigger addition/removal of the UE in the 5G VN group using the 5G VN API? There is already a monitoring event for UAV moving into or out of an area, so an AF could subscribe to that and then trigger 5G VN addition/removal of UE to/from a group.

But we also question that it is related to UAV operation/use cases.

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT-CMCC -1 | 1 | 1 | No | self-contained |
| WT-CMCC -2 | 1 | 1 | No | self-contained |
| WT-CMCC -3 | 0.5 | 0.5 | No | self-contained |

# 5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| {Possible values: "TS" or "Internal TR" or "External TR". See Note 1} | {e.g. "22.XXX" or actual number if known} | {Title of the specification (as per TR 21.801 §6.1.1), to be aligned as much as possible with the WI/SI title} | {e.g. "TSG#87"} | {e.g. "TSG#89"} | {<FamilyName>, <GivenName>, <Company>, <email address>. See Note 2} |

{Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

{Note 2:	The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| {e.g. "22.281"} | {Possible values: - either free text (e.g. “CS aspects to be removed") - or “Specification to be withdrawn”} | {e.g. "TSG#89"} | {Free text, e.g. "This TS covers Stage 2" or "This TS covers Stage 3" or "This TS covers both stages 2 and 3"} |

# 6	Work item Rapporteur(s)

{Mandatory: <FamilyName>, <GivenName>, <Company>, <email address>}

{Optional: <FamilyName>, <GivenName>, <Company>, <email address>: Secondary task(s)}

{The first listed Rapporteur is the work item primary Rapporteur. The role of a Rapporteur is further described in www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec. By default, the primary Rapporteur shall ensure the production of the post-completion summary. 
Secondary Rapporteur(s) are possible for specific secondary task(s), such as: "Write the post-completion summary"; "In charge of a specific aspect of the work item (specify which)"; "Rapporteur for a secondary responsible WG (specify which)"}

# 7	Work item leadership

SA WG2

# 8	Aspects that involve other WGs

{This information is provided as best effort assumption, at the time of submission of the WID to TSG approval. It can be later changed without a need to revise the WID.

The “aspects” can be provided by topic (e.g. “security”, “multimedia”) and/or by specifying the WG(s) e.g.: "SA2, SA3, SA5, SA6. CT6 for storage, and potentially SA4". If not applicable, indicate "None" or "None identified yet"}

For a Stage 2 WID requiring Stage 3 to be done by another group: on a best-effort basis, indicate which potential WG is expected to specify the Stage 3: {possible values: "Not applicable", " unknown", "CT WGs", etc}

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name

Annex A Companies Views

WT#1.1, 1.2

CMCC:

DoCoMo:

DT:

Ericsson: Support 1.1, provides response to HW which was also included in the original SI proposal material.

Today H&F is only supporting Qbv. That is not enough to remove jitter. As explained previously (reference to original SID proposal): there is an uncertain queueing and wrong transmission ordering that leads to

packets missing the opportunity for transmission in the current cycle, and are delayed till next cycle –breaking the schedule.

Can we TEI20 if solution can be agreeable, no RAN impact.

For 1.2, this should not be about TSN transport, we are interested in the overall bridge jitter reduction. The N3 handling should be considered independently, not part of this study. There should not be a RAN dependency because of this.

Huawei:

-For 1.1 against. Not sure why current H&F is not enough as mentioned in NWM as well.

-If 1.2 this is related to N3 interface.; i.e. Study mechanisms for interworking with TSN transport networks using CQF in order to support of E2E determinism and low latency communication and efficient N3 transmission. Huawei would like to support. Then RAN dependency should be marked as Yes.

Mitsubishi: We support 1.2 with same context than Huawei

Nokia: Against. We prefer focusing on 2.1.

Qualcomm:

Samsung:

Siemens: We in general support to study the de-jitter for TSN for the purpose of reducing delays on TSN scheduled traffic.

ZTE: For 1.1, the existing 5GS per port pair delay is a range (identified by independentDelayMax, independentDelayMin, dependentDelayMax, dependentDelayMin). If the window is too small, the packet may miss the gate control. If the window is too large, it creates the jitter. Is the so-called "jitter" here caused by this reason?  If my understanding is correct, what mechanism do you want to solve it? If my understanding is incorrect, please clarify the problem.

For the 1.2, I will feedback during the meeting.

WT#2.1, 2.2

CMCC:

DoCoMo:

DT:

Ericsson: Support 2.1/2.2, provides response to HW which was also included in the original SI proposal material.

Today H&F is only supporting Qbv. That is not enough to remove jitter. As explained previously (reference to original SID proposal): there is an uncertain queueing and wrong transmission ordering that leads to

packets missing the opportunity for transmission in the current cycle, and are delayed till next cycle –breaking the schedule.

Can we TEI20 if solution can be agreeable, no RAN impact.

Huawei: Against. Not sure whether there is an issue to solve as mentioned in in NWM as well.

Mitsubishi:

Nokia:

Qualcomm:

Samsung:

Siemens:

ZTE:

WT#3

CMCC:

DoCoMo:

DT: support.

Ericsson: Neutral/Siemens-WT#1. Compatibility with IEEE should be clearly required. No point to discuss proprietary solutions that do not interwork natively with Ethernet. So this is not just about description, more about formulation of the scope.

Huawei: Against current STP based description, which is solution oriented . It would be OK if only mentioning the general purpose, i.e., loop prevention.

Mitsubishi: Support a study on loop prevention. Ok to write the work task in a more general manner, but we should not forget the compatibility with IEC/IEEE 60802. To be merged with Siemens-WT#1

Nokia: Support. This is a requirement from ACIA, as well as SA1. Additional details in https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/TSGS2_168_Goteborg_2025-04/Docs/S2-2503661.zip

Qualcomm:

Samsung:

Siemens: Cannot be TEI work item, need to be studied.

To Huawei’s comment: STP/RSTP/MSTP are defined in IEEE 802.1Q as the standards for loop prevention in time sensitive networks. MSTP is required by IEC/IEEE 60802 by which different industrial automation users and vendors can avoid loops when interoperating in bridged time-sensitive networks. When IEC/IEEE 60802 is required by industrial clients, how the 5GS Logical Bridge can interoperate with the bridged time-sensitive network that is the question. Support of MSTP is just another same level of the defined features (e.g. LLDP support) that makes the 5G bridge work with the Standardized TSN streams.

(Proprietary solutions that can not support global compliance and interoperability are not considered in this WT.)

ZTE: Support. At least including MSTP, RSTP.

WT#4:

CMCC: We support to change the WT#4 as following: Study how packet replication and elimination can be done within 5G system in order to establish redundant connection between UE and UPF; considering two PDU sessions (established with reusing TS 23.501 5.33.2) but with the same PSA UPF, for a single device.

DoCoMo:

DT:

Ericsson: Against restricting scope to single device.

Huawei: Prefer Alt 1 with CMCC proposal. Further suggest to change “device” into “UE”

Mitsubishi:

Nokia:

Qualcomm:

Samsung:

Siemens:

ZTE: Support. Similar view with CMCC.

Siemens-WT#1

CMCC:

DoCoMo:

DT: support.

Ericsson: Neutral, overall IEEE compatibility required.

Huawei: Against. MSTP is solution oriented. See above.

Mitsubishi: Support - To be merged with WT#3

Nokia: Support. As WT3.

Qualcomm:

Samsung:

Siemens: There may be various topologies creating the loop, so the first bullet needs to be studied first. The focus of the study – which needs to be addressed as we proposed here - is how 5GS bridge behaves and interoperates with MSTP communication, including handling of BPDUs and configurations. Cannot be TEI work item, need to be studied.

ZTE: It has been covered by WT#3

Siemens-WT#2

CMCC:

DoCoMo:

DT: support.

Ericsson: Neutral

Huawei: Against. Not sure about the gap.

Mitsubishi: Support, with a focus on the second bullet. More generally, we think it is important to check whether/how the 5GS can be integrated in a TSN network with IEC/IEEE 60802 profile.

Nokia: Against, our point is this could still be achieved by TSN AF learning.

Qualcomm:

Samsung:

Siemens: Cannot be TEI work item, need to be studied.

IEC/IEEE 60802 has specified the TSN profiles (for industrial automation) to be configured (from CNC to (logical) bridges) by NETCONF/YANG. Whether and how 5GS – if as an IA Station (logical bridge) – can support NETCONF/YANG configurations is the question, this includes whether the NETCONF server is located inside or outside of the 5GS, and hence where and how the required capabilities in YANG modules be translated in 5GS.

ZTE: Not sure the gap

Siemens-WT#3

CMCC:

DoCoMo:

DT: support.

Ericsson: Against, Not quite sure what this means.

Huawei: Against. Not sure about the gap.

Mitsubishi: Support, with a priority on first bullet.

Nokia: Against, please refer to the comment.

Qualcomm:

Samsung:

Siemens: Cyclic asynchronous streams is important and without corresponding QoS characteristics specified in 5GS. This needs to be studied, not TEI.

ZTE: Not sure the gap.

CMCC-WT#1

CMCC: We suggest to do this within this item.

DoCoMo:

DT:

Ericsson: Against, see overall comments below

Huawei: potential TEI20

Mitsubishi:

Nokia: Not clear and not immediate need ?

Qualcomm:

Samsung:

Siemens: By reading the Justification this is not the case for LAN-type services where geolocation-triggered dynamic group is not required. Clarifications needed whether this feature is for 5G-LAN or other type of VPN (mobile VPN?)

ZTE: support, or TEL20

CMCC-WT#2

CMCC: We suggest to do this within this item

DoCoMo:

DT:

Ericsson: Against, see overall comments below

Huawei: Not clear. Seems same as WT-CMCC-1.

Mitsubishi:

Nokia: Not clear.

Qualcomm:

Samsung:

Siemens:

ZTE:

CMCC-WT#3

CMCC: We suggest to do this within this item

DoCoMo:

DT:

Ericsson: Against, see overall comments below

Ericsson view on WT-CMCC 1-3:

Regarding the objective, it is already possible to e.g. use 5G VN with LADN, so you could define a 5G VN group (with a specific DNN/S-NSSAI) that is only available in a certain LADN area (i.e. set of TAIs).

Provisioning LADN services area from an AF was added in GMEC, and it is general so it can be applied also to 5G VN groups, e.g., as stated in 23.501:

“-           The LADN service area can be provisioned for a group (e.g. 5G VN group) or an individual subscriber using UDM/NEF parameter provisioning service triggered by AF request as described in clause 4.15.6 of TS 23.502 [3].”

And to dynamically add and remove UEs from the 5G VN group as they move into or out of the area supported by the 5G VN group, not sure why that would be needed. Seems rather complex.

However, even if one wants to do that, couldn’t that be done by an AF monitoring the UE location using the monitoring API, and based on that trigger addition/removal of the UE in the 5G VN group using the 5G VN API? There is already a monitoring event for UAV moving into or out of an area, so an AF could subscribe to that and then trigger 5G VN addition/removal of UE to/from a group.

But we also question that it is related to UAV operation/use cases.

Huawei: potential TEI20

Mitsubishi:

Nokia: Not clear.

Qualcomm:

Samsung:

Siemens:

ZTE:support, or TEL20

Nokia-WT#1

CMCC:

DoCoMo:

DT:

Ericsson: Not quite clear why this would be important. Priority remapping could be possible to handle using proprietary deployments as well.

Huawei: Against. Not sure about the problem to solve.

Mitsubishi:

Nokia: Support. Additional details in https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/TSGS2_168_Goteborg_2025-04/Docs/S2-2503661.zip

Qualcomm:

Samsung:

Siemens: Need to understand the scenarios for different configuration domains. In practical cases there is only a single configuration domain.

ZTE: