3GPP TSG-SA WG2 Meeting #169	S2-2505321

Fukuoka, Japan, May 19 – 23, 2025

Source:	Nokia

Title:	KI#2 new solution – Dynamic service adjustments based on review of EC Events from EIF

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes a new solution for KI#2 considering the energy consumption inputs from EIF.

1. Discussion

The proposed solution aims to implement dynamic service adjustments based on review of the EC Events from EIF.

2. Proposal

It is proposed to agree to have the following text in 3GPP TR 23.700-67.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solution# | 1 | 2 | 3 |
| --- | --- | --- | --- |
| X |  | x |  |

## 6.X	Solution #X: Dynamic service adjustments based on review of EC Events from EIF

### 6.X.0	High-level solution principles

This solution proposes below high-level principles which also re-uses most of the existing PCF capabilities.

PCF considers the following factors to determine the UEs and PDU Sessions for carrying out service adjustments.

- PCF subscribes to notifications from EIF as described in clause 5.51 of TS 23.501.

- PCF considers Events (Energy consumption percentile/ Energy consumption per bit) from EIF as described in clause 6.1.2 of TR 23.700-67.

- PCF analyses if the EC percentile / per bit is persistently about configured limits and accordingly chooses the UEs and/or PDU Session for carrying out service adjustments.

### 6.X.1	Description

Based on the operator policies and/or local configuration, the PCF determines to subscribe to notifications from the EIF for Energy Consumption information with the granularities as described in clause 5.51 of TS 23.501 and 6.1.2 of TR 23.700-67.

These events are used to identify the UEs that are in a higher percentile band or higher energy consumption per bit, persistently higher for N samples of energy related information received from the EIF, where N is the number of samples defined based on operator policies.

Editor's note:	Whether and how to consider Energy information for communication with CHF pertaining to Service Adjustments is FFS.

The service adjustment decisions by the PCF could result in providing updated AM, UE or SM policies for the selected UEs and/or PDU Sessions.

### 6.X.2	Procedures

#### 6.X.2.0	General

Existing procedures are re-used to implement the service adjustments.

### 6.X.3	Impacts on services, entities and interfaces

PCF:

-	Maintains the information of UEs which are persistently in a higher percentile EC band or higher in energy consumption per bit based on Event notifications from the EIF.

-	Based on operator policies, performs policy decisions relating to service adjustments for the UEs and/or PDU Sessions by considering the information maintained.

# * * * End of changes * * * *