3GPP TSG-SA2 Meeting #169	S2-2505407

# 19 - 23 May, 2025, Fukuoka, Japan

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6332 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supported TAIs information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Per RAN3 reply LS, RAN3 further clarifies the benefits on OAM configured TAIs in AMF. The RAN WGs and SA5 believes such information can help to reduce the RAN configuration updates and optimize the system. But in the DP S2-2503224 some issues were highlighted if there were two sources one from the NG-RAN and another from the OAM system for AMF. The issues are: a) Coordination across the signalling based and OAM solutions is not clear b) Violation of “plug n’ play” principles c) Change of functional split between RAN and CN (Please see S2-2503224 for further details) Thus its proposed while maintaining the essence of RAN/SA5 solution of timetable configuration in the AMF, instead of OAM configuring in the AMF, as a compromise the NG-RAN can directly signal to the AMF same information for e.g. as part of the NG Setup procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the descriptions of OAM configured TAIs for regenerative payload. AMF is signaled by NG-RAN this information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary higher signalling load from NG RAN to AMF due to change in supported TAI list information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.11.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

#### 5.4.11.9	N2 interface and connection management for regenerative satellite payload

The NG Removal procedure defined in TS 38.413 [34] can be used to remove the interface between a gNB and an AMF in a controlled manner, e.g. when the gNB is leaving the service area of an AMF when using regenerative satellite payload.

The serving AMF for a UE can remain the same during feeder link switchover.

As an optimization option, the AMF may be indicated with multiple subsequent time windows instances within which the NTN gNB ID and supported TAI list are specified in NGAP Setup procedure or the RAN Configuration Update procedures (see TS 38.413 [34]). In other words, the ‘timetable’ indicates which TAIs the gNB will support at what time and avoids the NG-RAN configuration for each time as the satellite moves.

NOTE 1:	If a gNB IP address changes due to feeder link switchover, then the procedures described in e.g. clause 5.21.1 can be used.

NOTE 2:	RAN transitioning between different NTN GWs, TAIs and potentially between AMFs can be supported by using existing procedures, e.g. the gNB releasing UEs to CM-IDLE before changing NTN gateway, or by using existing load re-balancing, triggering handover when it realizes it is going to leave a tracking area due to its own movement, etc.

NOTE 3:	The ‘timetable’ indicating which TAIs the gNB will support at what time, will be configured by OAM in the NG-RAN.

* * * * End of Changes * * * *