3GPP TSG-SA2 Meeting #169	S2-2505518

# 19 - 23 May, 2025, Fukuoka, Japan

Source: 	vivo

Title: 	KI#2: New Solution –Collocation of UPF and NWDAF containing AnLF to support efficient performance of User Plane

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes a solution for KI#2 to support efficient performance of User Plane by collocating UPF and AnLF.

1	Discussion

This paper proposes a solution to support efficient performance of User Plane by collocating UPF and AnLF.

Since UPFs in the network have plenty observed information and measurements data about UP traffic, they can be reported from UPFs to NWDAF for model training and model inference (i.e. to generates analytics). However, the data volume reported from UPFs to the NWDAF is big for the data reporting may be per UE or per PDU session or per service flow or even per packet, which may influence the UPF’s performance itself and may also cause the big load on Control Plane. Thus, it has been approved in the key issue description that the solution should consider to minimize the load on Control Plane NFs as much as possible. For model training, this issue can be avoided by offline data collection and reporting from the UPF to the NWDAF, however, for model inference this issue is still there. If the UPF is collocated with the NWDAF containing AnLF from architecture point, the issue can be avoided.

2. Proposal

It is proposed to add the following contents to TR 23.700-04.

Start of Change

6.0	Mapping of Solutions to Key Issues and Use Cases

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #1 | x |  |  |  |
| #2 | x |  |  |  |
| #3 | x |  |  |  |
| #4 | x |  |  |  |
| #X |  | X | x |  |

Next Change (all new text)

6.X	Solution #X: Collocation of UPF and NWDAF containing AnLF to support efficient performance of User Plane

6.X.1	High level principles

The NWDAF containing MTLF is used to train ML model of User Plane Traffic Exception,

The UPF is collocated with NWDAF containing AnLF, and it is used to generate analytics result of User Plane Traffic Exception based on the ML model from The NWDAF containing MTLF.

Based on the analytics result of User Plane Traffic Exception, the consumer performs corresponding mitigation actions to avoid abnormal or malicious traffic.

6.X.2	Description

This solution is to address Key Issue#2: AI/ML-assisted user plane traffic pattern and behavior analysis to support efficient performance of User Plane.

Since UPFs in the network have plenty observed information and measurements data about UP traffic, the NWDAF containing MTLF can use these input data from UPFs to train ML model of User Plane Traffic Exception . This ML model will be provided to NWDAF containing AnLF to generate analytics of User Plane Traffic Exception by performing ML model inference process.

Editor's Note: What input data should be collected to train ML model by the NWDAF containing MTLF is FFS.

However, considering latency and big data volume of input data reporting from UPF to the NWDAF containing AnLF during ML model inference process, this solution proposes to collocate the UPF and the NWDAF containing AnLF together. In this case, the ML model from the NWDAF containing MTLF will be offloaded to UPF collocated with AnLF, then the UPF can generate analytics result itself, and this result can assist the consumer to perform some corresponding mitigation actions, e.g. avoid abnormal or malicious traffic.

Editor's Note: What kind of User Plane Traffic Exception can be is FFS. What actions can be done by which consumers based on corresponding user plane traffic exception is FFS.

6.X.3	Procedures

6.x.3.1 Support of efficient performance of User Plane by collocating UPF and NWDAF containing AnLF

Figure 6.x.3.1-1:  Support of efficient performance of User Plane by collocating UPF and NWDAF containing AnLF

1. UPF collocated with AnLF subscribes to NWDAF containing MTLF for a trained ML Model of User Plane Traffic Exception, by invoking the Nnwdaf_MLModelProvision_Subscribe service operation.

2. To train the ML model of User Plane Traffic Exception, the NWDAF containing MTLF collects input data from UPF, by invoking the Nupf_EventExposure_Subscribe service operation.

3-4. The UPF collects data (e.g., UPF local data, N3 measurements, N6 measurements), and notifies the NWDAF containing MTLF by invoking the Nupf_EventExposure_Notify service operation.

Editor's Note: What specified input data should be collected to train ML model is FFS.

NOTE:  To avoid much impacts on UPF performance and reduce signalling load for data reporting, the data reporting from UPF for ML model training can be done during off-peak time.

5. NWDAF containing MTLF trains ML model of User Plane Traffic Exception  based on training data collected from data sources e.g., UPFs.

6.The NWDAF containing MTLF provides the trained ML model to the UPF which is collocated with NWDAF containing AnLF, by invoking Nnwdaf_MLModelProvision_Notify service operation.

7. Based on the ML model from the NWDAF containing MTLF, the UPF collocated with AnLF collects data, e.g. UPF local data, N3 measurements, N6 measurements, etc. as input data for ML model inference. Then

the UPF generates analytics result of User Plane Traffic Exception based on the ML model.

The consumer (e.g. the UPF) applies the corresponding policies and performs specific action related to the User Plane Traffic Exception.

6.X.4	Impacts on services, entities and interfaces

UPF collocated with NWDAF containing AnLF

- Retrieves ML model of User Plane Traffic Exception from NWDAF containing MTLF;

- Generates analytics of User Plane Traffic Exception based on the ML model.

NWDAF containing MTLF:

-	Trains and provides ML model of User Plane Traffic Exception.

End of Changes