3GPP TSG-WG SA2 Meeting #169	S2-2505629

Fukuoka, Japan, May 19 – 23, 2025	(revison of S2-2504671)

Source:	OPPO

Title:	New solution to KI#4 on sensing data and the associated information transport

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes a new solution to KI#4 on sensing data and the associated information transport for the FS_Sensing_ARC TR 23.700-14.

1	Discussion

This paper proposes a new solution to address KI#4 on sensing data and the associated information transport. This solution refers to the clause 6.18 of TS 23.273 on LCS-UP connection establishment procedure.

Based on the assumptions above, the following procedures are provided.

2	Proposal

It is proposed to include the following changes in TR 23.700-14 V0.1.0.

* * * * Start of Changes (All text new) * * * *

6.X	Solution #X: Sensing data and associated information transport via data connection

6.X.1	Description

This solution has the following assumptions:

1)	The sensing data and associated information are transmitted between the Sensing Entity and the Sensing Function via data connection.

2)	The data connection establishment procedure is carried out on control plane signalling interaction.

3)	The Sensing Function provides its connection information (e.g. IP address, end point) for data connection for data connection establishment and conditionally binding information (e.g. Sensing Entity ID, connection ID or specific binding ID) to the Sensing Entity.

4)	The Sensing Entity sends binding information via established data connection to sensing function for connection binding.

6.X.2	Procedures

Figure 6.x.2-1: Procedure to establish data connection between Sensing Function and Sensing Entity

1.	The Sensing Function determines to establish data connection for sensing data and associated information transport.

2a1.	[Conditional] The Sensing Function sends the connection information to AMF in a N1 or N2 container to indicate the selected sensing entity (i.e. UE or NG-RAN node) to utilize data connection for sensing. The connection information includes the address or end point information of the Sensing Function. The binding information (e.g. Sensing Entity ID, connection ID or specific binding ID) associated with the data connection is also included in the connection information.

2a2.	[Conditional] AMF sends the connection information to the Sensing Entity via N1 or N2 message.

2b.	[Conditional] If there is direct interface between the Sensing Function and Sensing Entity, the Sensing Function can send the connection information to the Sensing Entity directly.

NOTE1: For Sensing Enity as UE, only 2a1 and 2a2 will be performed.

3.	After the Sensing Entity receives the connection information, the Sensing Entity establish data connection with the Sensing Function according to the address of the Sensing Function. If the Sensing Entity is a UE, the UE shall establish a PDU Session or use an existing PDU Session to establish data connection with the Sensing Function. If the sensing entity is a gNB, the sensing entity establishes direct data connection with the Sensing Function. After the data connection has been established successfully, the Sensing Entity sends the binding information received in step 3 to Sensing Function via the established data connection to enable the Sensing Function to perform the correlation of the Sensing Entity or Connection ID with the data connection.

NOTE2: The PDU Session establishment procedure for the Sensing Enity as UE can refer to the LCS-UP connection establishment procedure in clause 6.18 of TS 23.273.

4a1.	The Sensing Entity sends an acknowledgement to Sensing Function to indicate a success of data connection establishment for sensing service.

4a2.	[Conditional] AMF sends the acknowledgement received in step 4a1 to the Sensing Function.

4b.	[Conditional] If there is direct interface between the Sensing Function and Sensing Entity, the Sensing Entity can send the acknowledgement to the Sensing Function directly.

5.	If Sensing Function or Sensing Entity determines to utilize the data connection for sensing and the data connection is established, messages are transferred between Sensing Entity and Sensing Function for sensing data and the associated information transport.

6.X.3	Impacts on services, entities and interfaces

The solution has impacts on the following entities:

AMF:

-	Support to send the connection information received from Sensing Function to Sensing Entity via N1 or N2 message.

Sensing Function:

-	Support to provide connection information to Sensing Entity directly or via AMF in N1 or N2 container.

-	Support to receive binding information from the Sensing Entity via data connection for connection binding.

-	Support to transfer sensing data and the associated information with the Sensing Entity via data connection.

Sensing Entity:

-	Support to establish data connection with the Sensing Function according to the connection information received from Sensing Function.

-	Support to send binding information to the Sensing Function via established data connection for connection binding.

-	Support to transfer sensing data and the associated information with the Sensing Function via data connection.

* * * * End of Changes * * * *