3GPP TSG-WG SA2 Meeting #169	S2-2505632

Fukuoka City, Fukuoka, JP, 19th May – 23rd May, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	NF Selection Update

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: address ENs about NF Selection.

1. Introduction/Discussion

Editor’s note: 	It is FFS whether and how the procedure is performed between AMF and AIOTF in order to provide the NG-RAN ID of NG-RAN from the AMF to the AIOTF.

The AMF capable of Ambient IoT can register its support of AIoT service as part of its NF profile in the NRF. The AIOTF can discover the AIoT capable AMF via NRF. The AIOTF selects the AMF to route the AIoT messages if the selected RAN node/reader is within the coverage of the AMF.

During NGAP setup procedure in case of indirect connectivity, the NG-RAN initiates the setup procedure towards the configured AMF. The same configuration can also apply to the AIOTF, thus the AIOTF can determine the correct AMF that is configured to connect to the selected node/reader.

-	Even if the NGAP connection fails to setup between the AMF and target NG-RAN, the AMF can reject the delivery request from AIOTF with appropriate code. And this failure shall not happen in the operation phase (when the network is in use), the failure can be detected as unsuccessful operation (NG SETUP FAILURE) during network deployment phase, thus the configuration will be adjusted before the network is put in use.

-	In the case that NG-RAN nodes or AMF instances are newly added or released, the configuration shall be updated according, both at the NG-RAN and the AIOTF.

Proposal 1: The AIOTF knows the relationship between the NG-RAN and the AMF by means of the configuration, same as the configuration used by NG-RAN to setup NGAP association towards the AMF.

Editor's note:	The further investigation of factors for the ADM selection will be needed and other factors are FFS.

Domain information of AIoT device permanent ID contains the PLMN ID, NID or A third party identifier used. Normally, the domain information together with NRF is used to locate the entity storing the AIoT device profile data of an Ambient IoT Device, if device ID is used, then the NRF query will be very inefficient considering the long length of the permeant device ID. Other parts (e.g., ID type or ID identification) are not clear how to use NRF to select or discover the ADM.

Proposal 2: Clarify AIoT device permanent ID is used for ADM selection in case of local configuration.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change * * * *

5.3.4	AMF Discovery and Selection

For indirect Connectivity via AMF (see clause 4.2), AMF discovery and selection functionality is implemented in AIOTF.

The AIOTF performs AMF discovery and selection in order to forward AIoT service operation messages to the target NG-RAN node via the selected AMF.

When the AIOTF selects a NG-RAN node during the NG-RAN Node and RAN Reader selection procedure as defined in clause 5.3.3, the AIOTF selects the AMF based on local configuration which contains the AMF and NG-RAN relationship.

NOTE:	The network operator needs to ensure the local configuration works successfully for NG SETUP operation between NG-RAN and AMF during network deployment phase. And it is left to stage 3 to handle the case that the selected AMF cannot forward the messages received from AIOTF towards the target NG-RAN.

# * * * * End of changes * * * *