SA WG2 Meeting #S2-169	S2-2505635

19 - 23 May, 2025, Fukuoka, Japan	(revision of S2-2505543/S2-2504967)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5499 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the UPF Energy Consumption information collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When the SMF collects the Energy Consumption information, it needs to collect the data volume in the gNB, I-UPF and Anchor UPF. When the I-SMF is involved in a PDU session, the SMF of PDU session will interact with I-SMF to obtain the data volume on the I-UPF which is controlled by I-SMF. There are two mechanisms for A-SMF to interact with I-SMF: 1) Nsmf_PDUSession_Update. The A-SMF sends the corresponding N4 information to I-SMF in Nsmf_PDUSession_Update Request as described in clause 4.23.5 of 23.502. And the I-SMF uses N4 Session Modification towards I-UPF to collect the data volume. 2) Nsmf_EventExposure service.The A-SMF subscribe the data volume with I-SMF, and I-SMF send the N4 Session Modification towards I-UPF to collect the data volume. In the previous meeting, it was agreed that the UPF event exposure is not apllied in the Energy Consumption information collection. The N4 session will be used between SMF and UPF. It means the I-SMF only can use the N4 session to control the I-UPF for data volume collection. In the exsiting procedure, the N4 information is not created by I-SMF itself, the A-SMF send the Nsmf_PDUSession_Update Request to I-SMF with corresponding N4 information. So to minimize the impact, this CR proposes the method 1.The A-SMF sends the Nsmf_PDUSession_Update Request to I-SMF for Energy Consumption information collection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying the I-UPF ID and gNB ID information collection when I-SMF is involved |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclar how A-SMF collects the data volume on the I-UPF controlled by I-SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.29.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 4.29.2	Energy Consumption information collection

Figure 4.29.2-1 shows the procedure for EIF to collect the Energy consumption information.

Figure 4.29.2-1: Energy Consumption information collection

1.	The Consumer NF (i.e. trusted AF or 5GC NF) subscribes to the energy consumption event with the EIF by using Neif_EventExposure_Subscribe service for the required granularities (UE, PDU Session and/or QoS flow) as described in clause 5.51.2.3 of TS 23.501 [2].

If the Consumer is untrusted AF, the subscription is sent via NEF. If the event subscription is authorized by the NEF, then the NEF sends the event subscription to the EIF via Neif_EventExposure_Subscribe.

2.	The EIF invokes Nudm_UECM_Get service operation to retrieve the SMF ID for the required granularity.

3.	The EIF invokes the Nsmf_EventExposure_Subscribe request to the SMF to request the information as defined in clause 5.51.2 of TS 23.501 [2].

If the I-SMF is involved in the PDU session, the SMF of PDU session requests to I-SMF to report the ULI and I-UPF ID.

4.	The SMF informs the UPF with requesting the PSA UPF to report the data volume of the required granularity.

5. The SMF provides the information for energy consumption calculation (defined 5.51.2 of TS 23.501 [2]) to EIF by invoking Nsmf_EventExposure_Notify service operation for the required granularity.

NOTE:	The SMF can derive gNB ID(s) according to the AMF provided ULI as described in clause 4.3.2.

6. If there is no related Node-level energy consumption information and Node-level data volume, the EIF obtains the Node-level energy consumption information and Node-level data volume from OAM with providing gNB ID(s) and UPF ID(s).

7.	The EIF performs the energy consumption information calculation as described in clause 5.51.2.3 of TS 23.501 [2] and then reports the energy consumption information of the required granularity to the Consumer NF via Neif_EventExposure_Notify.

END OF CHANGES