3GPP TSG SA WG2 Meeting #169	S2-2505693

# 19 – 23 May, 2025, Fukuoka, Japan                                      			    (revision of S2-2504830)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0733 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to UE LCS privacy profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In previous meetings it was debated whether LMF should check the UE LCS privacy profile when collecting UE location data for model training and performance monitoring given previous agreements where LMF checked user consent. In this CR, an update of the UE LCS privacy profile is proposed to explicitly account for consent to collect UE location data. It is indicated however that such updated privacy profile is utilized, it cannot be provided or updated by the AF but rather by the UE itself. The above is proposed as basis for a potential way forward where the check of the updated privacy profile may suffice for training/performance monitoring/inference in AI/ML-based positioning |  |  |  |  |  |  |  |  |
| Summary of changes: |  | The following changes are proposed: - Updated definition of UE LCS privacy feature including the option to allow or disallow UE location data collection based on consent from the user, but unlike the legacy UE LCS privacy profile this cannot be provided/updated by an AF. - Updates to LPI indicating new settings to allow or disallow the above options - Updates to the LPI provisioning procedure including LMF as an option - Other miscellaneous changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The deadlock on user consent and privacy for AIML-based positioning will not be resolved |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1, 5.4.2.1, 5.4.2.3, 5.4.3, 6.12.1, 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

## 5.4	UE LCS privacy

### 5.4.1	General

An LCS client or AF may or may not be authorised to retrieve the UE location, e.g. for commercial use. UE LCS privacy is a feature which allows a UE and/or AF to control which LCS clients and AFs are and are not allowed access to UE location information. In addition, based on consent from the user, the UE LCS privacy feature allows or disallows

the collection of location data by the network from the UE for a purpose related to UE location estimation, e.g. UE positioning calculation or training of a model for LMF-based AI/ML positioning.

UE LCS privacy can be supported via subscription and via UE LCS privacy profile handling.

With subscription, privacy preferences for a UE are stored in a UE LCS privacy profile as part of UE subscription data in the UDM and queried from the UDM by another NF such as GMLC, LMF or NEF. The UDM may also store the UE privacy profile in the UDR. In this release of the specification, subscription of privacy preferences is restricted to the Call/Session unrelated Class as defined in clause 5.4.2.2.3 and the PLMN Operator Class as defined in clause 5.4.2.2.4.

With UE LCS privacy profile handling, the UE and/or AF can provide and update part of the UE privacy profile and provide it to the network as an update to the UDR. In this release of the specification, UE LCS privacy profile handling is restricted to the Location Privacy Indication as defined in clause 5.4.2.3.

The UE LCS privacy profile is used to indicate whether LCS requests from LCS clients and AFs and collection of location data from the UE are allowed or disallowed, together with the POI as defined in clause 5.4.4.

NOTE 1:	In clause 5.4, even if the UE LCS privacy detail is only described for LCS client, the same detail is also applicable for AF, if no exception statement.

NOTE 2:	In UE LCS privacy, consent to collect location data from a UE cannot be provided by an AF.

* * * Next Change * * *

### 5.4.2	Content of UE LCS Privacy Profile

#### 5.4.2.1	General

The UE LCS privacy profile shall include information related to classes of LCS client, referred to as "privacy classes", which are permitted, or conditionally permitted, to obtain location information for the UE. Privacy classes are defined in clause 9.5.3 of TS 23.271 [4], but not all classes defined in TS 23.271 [4] are supported in this specification. Privacy classes are supported as described below. The differences between the Privacy classes for 5GS and those for EPS are described in Annex A.

The UE LCS privacy profile also includes the Location Privacy Indication, as defined in clause 5.4.2.3, which can be provided and updated by the UE and/or AFs.

NOTE:	Tthe UE LCS privacy profile cannot be provided or updated by an AF when it includes consent to collect location data from the UE.

* * * Next Change * * *

#### 5.4.2.3	Location Privacy Indication (LPI)

The Location Privacy Indication is not defined in TS 23.271 [4]. The Location Privacy Indication defines whether LCS requests for UE from any LCS clients are allowed or disallowed. The Location Privacy Indication also defines whether location data can be collected by the network from a UE for a purpose related to UE location estimation, e.g. UE positioning calculation or training of a model for LMF-based AI/ML positioning.

The LPI at least includes one of the following global settings (for all LCS clients and AFs):

-	Location for UE is disallowed (location for UE not allowed to any LCS client except where POI applies).

-	Location for UE is allowed (default setting, and LCS requests for UE from LCS clients are authorized based on their associated privacy classes as defined in clause 5.4.2.2).

The LPI may also include at least one of the following additional global settings:

-	Collection of UE location data is disallowed (collection of UE location data not allowed except when POI applies).

-	Collection of UE location data is allowed (default setting, and LCS requests for UE from LCS clients are authorized based on their associated privacy classes as defined in clause 5.4.2.2).

NOTE:	Additional LPI values may be supported for additional differentiation of location request types.

The LPI also allows the following optional settings:

-	Valid time period for LPI, including start time and end time.

The LPI takes precedence on the subscribed privacy classes as defined in clause 5.4.2.2. The LPI allows a UE to override the location preference of the subscribed privacy classes. The usage of LPI is described in clause 6.1.2.

* * * Next Change * * *

### 5.4.3	Provision of UE LCS privacy profile

A generation or change to the LPI in UE LCS privacy profile is determined by the UE and provided to the network using N1 NAS message. It may be updated by UE any time.

An authorized AF is allowed to provision the LPI in UE LCS privacy profile for specific UE(s) via NEF.

NOTE:	The AF allowed to provision the UE LCS privacy profile is different from the AF sending location requests.

The LPI in UE LCS privacy profile may be provided or updated by the target UE during the 5GC-MT-LR and Deferred 5GC-MT-LR Procedure for Periodic, Triggered and UE Available Location Events. The updated profile is stored into the UDR by the UDM after the interaction with the AMF. The LPI in UE LCS privacy profile shall include an indication if location is allowed or disallowed and may include a valid time period for LPI as described in clause 5.4.2.3.

In addition, a notification is sent by the UDM in order to notify the subscribed consumer i.e. GMLC, LMF and/or NEF about the change of UE LCS privacy profile:

-	Target UE identity (one or both of GPSI and SUPI);

-	Updated UE LCS privacy profile.

* * * Next Change * * *

### 6.12.1	UE Location Privacy Setting Procedure Initiated by UE

Figure 6.12.1-1: UE Location Privacy Setting procedure initiated by UE

0.	NF (e.g. GMLC, NEF, LMF) may subscribe to UDM notifications of UE LCS privacy profile updates.

NOTE 1:	NF other than LMF can decide to subscribe to such notifications while a deferred location request is on-going.

NOTE 2:	LMF can decide to subscribe to such notifications while the procedure for data collection for model training or performance monitoring for LMF-based AI/ML Positioning in clause 6.22.3 is on-going.

1.	If the UE has generated or updated the UE Location Privacy Indication, the UE sends the Location Privacy Indication to the AMF via UE Location Privacy Setting Request in N1 NAS message. The UE Location Privacy Indication indicates whether allows or disallows the subsequent LCS requests for the UE, as defined in clause 5.4. If the UE has generated or updated the event report expected area and optionally the area usage indication, e.g. based on UE power status, the UE Location Privacy Setting Request includes the event report expected area and the area usage indication.

2.	The AMF invokes a Nudm_ParameterProvision_Update (LCS privacy) service operation towards the UDM and the service operation carries the Location Privacy Indication information and may include event report expected area and the area usage indication. The UDM stores or updates the UE LCS privacy profile in the UDR by invoking a Nudr_DM_Update (SUPI, Subscription Data) service operation accordingly.

3.	The AMF responses to the UE via UE Location Privacy Setting Response in N1 NAS message.

4.	UDM notifies the subscribed Network Function (e.g. GMLC, NEF, LMF) of the updated UE LCS privacy profile via Nudm_SDM_Notification Notify message.

5.	NF (e.g. GMLC, NEF, LMF) may unsubscribe to UDM notifications of UE LCS privacy profile updates e.g. if a deferred location procedure is cancelled.

* * * Next Change * * *

## 7.1	UDM

For each UE subscriber the UDM stores LCS related data as part of the Subscriber Data Management (SDM) service as defined in clause 5.2.3.3.1 of TS 23.502 [19].

The privacy profile data is defined in table 7.1-1 containing data for the privacy classes for which location of the target UE is permitted. For the meaning of each LCS privacy profile data type and included data, refer to clause 5.4.2.

Table 7.1-1: LCS privacy profile data stored in the UDM for a UE Subscriber

| Privacy Profile Data Type | Presence | UDM data |
| --- | --- | --- |
| Location Privacy Indication | M O O | Indication of one of the following mutually exclusive global settings: - Location is disallowed. - Location is allowed (default). Indication of one of the following mutually exclusive global settings: - Collection of UE location data is disallowed. - Collection of UE location data is allowed (default). Time period when the Location Privacy Indication is valid. |
| Call/session Unrelated Class | M O O O O O O O O O O O O O | For any LCS client or AF not in the external LCS client list or otherwise identified for the Call/session Unrelated Class, the following data may be present: - One of the following mutually exclusive options: - Location not allowed (default case) - Location allowed with notification. - Location with notification and privacy verification; location allowed if no response. - Location with notification and privacy verification; location restricted if no response. - Time period when positioning is allowed. - Geographical area where positioning is allowed. - Indication that codeword shall be checked in UE or one or more codeword values to be checked in GMLC. External LCS client list: a list of zero or more LCS clients, AFs and LCS Client groups with the following data for each entry: - One of the following mutually exclusive options: - Location allowed without notification (default case). - Location allowed with notification. - Location with notification and privacy verification; location allowed if no response - Location with notification and privacy verification; location restricted if no response - Time period when positioning is allowed - Geographical area where positioning is allowed Service types list: a list of one or more service types for which the LCS client is allowed to locate the particular UE. The possible service types are defined in TS 22.071 [2]. The following data may be present for each service type in the list: - One of the following mutually exclusive options: - Location allowed without notification (default case) - Location allowed with notification - Location with notification and privacy verification; location allowed if no response. - Location with notification and privacy verification; location restricted if no response. - Time period when positioning is allowed. - Geographical area where positioning is allowed. - Indication that codeword shall be checked in UE or one or more codeword values to be checked in GMLC. |
| PLMN Operator Class | O | LCS client list: a list of one or more generic classes of LCS client that are allowed to locate the particular UE. The following classes are distinguished: - LCS client broadcasting location related information. - O&M LCS client in the HPLMN. - O&M LCS client in the VPLMN. - LCS client recording anonymous location information. - LCS Client supporting a bearer service, teleservice or supplementary service to the target UE. - NWDAF in the HPLMN (when the UE is currently being served by the HPLMN). - NWDAF in the VPLMN. - LMF in the HPLMN. |
| User Plane Connection between UE and LCS Client or AF | O | Indication of one of the following mutually exclusive global settings: - UE is allowed to report periodic or triggered location events via user plane to an LCS Client or AF - UE is not allowed to report periodic or triggered location events via user plane to an LCS Client or AF (default). |
| Event report expected area | O | Presents a geographical area generated by UE, which is used by GMLC to determine event report allowed area for the UE. |
| Area usage indication | O | Indication of one of the following mutually exclusive global settings on using event report allowed area: - Inside reporting (default). - Outside reporting. |
| GMLC address list | O | Addresses of GMLC located in local network(s) which are allowed to be used for the UE positioning. The GMLC address is used to identify the local network for verification of location service in PNI-NPN. |

The Mobile Originating data is defined in table 7.1-2 containing the LCS MO-LR services that a UE can receive.

Table 7.1-2: LCS Mobile Originated data for a UE Subscriber

| MO-LR Data | Presence | UDM data |
| --- | --- | --- |
| Mobile Originated data | M | List of MO-LR services allowed for a UE subscriber: - Basic Self Location (UE can receive its own location). - Autonomous Self Location (UE can receive location assistance data). - Transfer to Third Party. |
| List of Assistance Data Types for MO-LR | O | A list of one or more types of location assistance data that may be provided to the UE in the MO-LR procedure. |

The LCS broadcasting data is defined in table 7.1-3 containing a list of assistance data types for which ciphering keys should be provided to the UE if requested by the UE.

Table 7.1-3: LCS broadcasting data for a UE subscriber

| Broadcasting Data | Presence | Description |
| --- | --- | --- |
| List of Assistance Data Types | O | A list of one or more types of location assistance data for which ciphering keys should be provided to the UE if requested by the UE when the assistance data is broadcast using ciphering. |

Other UE LCS subscriber data is defined in table 7.1-4 containing UE's subscription data related with LCS.

Table 7.1-4: UE LCS subscriber data

| UE LCS subscriber data | Presence | Description |
| --- | --- | --- |
| PRU indication | O | When present, it is used to indicate that the UE is allowed to serve as a PRU. |
| LPHAP indication | O | When present, it indicates a UE requires low power and high accuracy positioning. |
| LMF ID | O | An LMF ID which presents an LMF deployed in local network to support location service for the UE. |
| Indication of user plane positioning between UE and LMF | O | When present, it indicates that the UE is allowed to use the user plane positioning between UE and LMF. |

* * * End of Changes * * *