3GPP TSG-SA WG2 Meeting #169	S2-2505785

19 – 23 May, 2025, Fukuoka, Japan

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6254 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EIF discvoery and selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Node-level EC information from OAM can be used by EIF for multiple Consumer NFs that subscribe to the EIF for the UE being served by the Node. EIF discovery/selection does not support the selection of the same EIF for the same UE, which can reduce duplicated signaling (e.g., OAM). |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Update NOTE so that the the Node-level Energy Consumption information received from OAM could be used by EIF for one or more Consumer NFs -Add SUPI range, GPSI range as a factor to be considered for EIF selection/discovery |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.2.2.3, 6.3.27 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.51.2.2.3	Energy Consumption information collection from OAM

Table 5.51.2.2.3-1 provides the list of information received from OAM. EIF requests the energy consumption and data volume information per NF level from OAM by providing the serving gNB ID(s) and (I-)UPF ID(s) received from SMFs.

The node-level reporting interval from the OAM is the PLMN wide configurable starting time and interval T which is the same as data volume reporting interval from SMFs.

Table 5.51.2.2.2-1: Information from OAM for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| gNB energy consumption | The Energy consumed by a gNB over the configured time interval T based on clause 6.7.3.4.2 of TS 28.554 [209]. |
| gNB data volume | The UL/DL data volume handled by a gNB over the configured time interval T based on clause 6.7.1.1 of TS 28.554 [209]. |
| UPF energy consumption | The Energy consumed by a UPF over the configure time interval T based on clause 6.7.3.1 of TS 28.554 [209]. |
| UPF data volume | Data volume consumed at a UPF. |

When the serving gNB and/or the (I-)UPF(s) of the UE are changed, the serving gNB ID and UPF ID will be updated to the EIF through SMF(s).

NOTE 1:	The Node-level Energy Consumption information received from OAM could be used by EIF for all the UEs beng served by the NF Node. Also, if the EIF has subscriptions from one or more Consumer NFs for the same UE, the Node-level Energy Consumption information received from OAM could be used by EIF for one or more Consumer NFs that subscribed to the EIF.

NOTE 2:	In this Release, only the energy related information of user plane communication is supported and that of control plane signalling is not supported.

The EIF acts as the enforcement point for user consent for collection and processing of energy-related information depending on local regulations. For support of user consent (for collection and processing of energy-related information), the EIF communicates with the UDM as specified in Annex V of TS 33.501 [29].

# * * * * Second change * * * *

6.3.27	EIF discovery and selection

The NF consumers shall utilise the NRF services to discover EIF instance(s) unless the EIF selection information is locally configured in NF consumers. The NF consumer selects an EIF instance based on the available EIF instances obtained from the NRF or locally configured information in the NF consumer.

The following factors may be considered by the NF consumer for EIF selection:

-	Location (see locality in clause 6.1.6.2.2 of TS 29.510 [58]).

- 	SUPI: The NF consumer selects an EIF instance based on the SUPI range the UE's SUPI belongs to or based on the results of a discovery procedure with NRF using the UE's SUPI as input for EIF discovery.

-	GPSI: NF consumers which manage network signalling not based on SUPI (e.g. the NEF) select a EIF instance based on the GPSI range the UE's GPSI belongs to or based on the results of a discovery procedure with NRF using the UE's GPSI as input for EIF discovery.

# * * * * End of changes * * * *