3GPP TSG-RAN WG2 Meeting #130	R2-2503565

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.7.1

Source:	Nokia (Rapporteur)

Title:	Stage 2 Open Issues

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

During post RAN2#129bis email discussion #501, one open issue was identified on PDU set handling. This contribution echoes past discussions, highlights the problems and suggests sending an LS to SA2 to solve it.

# 2	PDU Set Handling

At RAN2#128, the following was discussed:

R2-2409533 LS for PDU Set Information Marking Support without QoS parameters (S2-2411253; contact: vivo) SA2 LS in Rel-19 XRM_Ph2 To:RAN2, RAN3 Q1:SA2 would like RAN2 to provide feedback (e.g. the usefulness) on the PDU Set based handling without PDU set QoS parameters. Noted Discussion on the reply LS on PDU Set Information R2-2409767 Discussion on SA2 LSs on XRM and FEC vivo discussion Rel-19 NR_XR_Ph3-Core Noted Proposal 1: RAN2 confirms that it is useful for gNB to have PDU Set Information marking without PDU Set QoS parameters, e.g. for PSI based SDU discard. Proposal 2: RAN2 to discuss the draft reply LS in Annex A for PDU Set Information Marking Support without QoS parameters. DISCUSSION on P1: - OPPO thinks SA2 only asks about DL and asks if it applies for UL as well. - Vivo thinks LS mentions DL only. - LGE thinks both LSes from SA2 focus only on DL. LGE agrees with the proposal. - OPPO agrees this is only for DL and then agrees with the proposal. - Sony agrees with the proposal, it is useful. Agreement for the reply to SA2 on PDU set information: - RAN2 confirms that it can be useful for gNB to have PDU Set Information marking without PDU Set QoS parameters.. [AT128][501][XR] Reply LS on PDU set (vivo) Scope: Reply LS Intended outcome: Reply LS in R2-2411001 Deadline: For offline approval on Friday 2024-11-22 09:00

The agreed reply LS stated the following [R2-2411001]:

RAN2 response: RAN2 has discussed RAN handling on DL data based on PDU Set Information marked in the GTP-U header without PDU Set QoS parameters configured in NGAP signaling. RAN2 confirms that it can be useful for gNB to have PDU Set Information marking without PDU Set QoS parameters.

As a result of this agreement:

1.	SA2 agreed a CR against 23.502 entitled Support PDU Set based handling without DL PDU Set QoS parameters[S2-2502673], capturing amongst other things the following:

If the N2 SM information includes the PDU Set Based Handling Support Indication and either there are PCC Rules with PDU Set QoS parameters for DL or the SMF has sent the DL PDU Set Information Marking Support Indication in step 3, the SMF configures PSA UPF to activate PDU set identification and marking for the QoS flow as described in clause 5.37.5.3 of TS 23.501 [2].

2.	In post RAN2#129 email discussion #508, the following Note was added to the Stage 2 running CR:

NOTE: PDU Set Information can be provided without PDU Set QoS Parameters.

During post RAN2#129bis email discussion #501, whether PDU set based handling is linked to the provision of PDU Set QoS parameters was discussed. For instance, 23.501 states that “At least one of the following shall be sent to the NG-RAN/N3IWF/TNGF/W-AGF to enable PDU Set based handling: 1) a PSIHI and/or 2) both PSDB and PSER”, seemingly contradicting RAN2 agreement and what 23.502 describes. In order to clarify this point, an LS should be sent to SA2.

# 2	Conclusion

One open issue has been identified:

Stage 2-1: PDU Set Based Handling without QoS parameters.

And to solve it, the following was proposed:

Proposal 1 (Stage2-1): send an LS to SA2 to clarify PDU set handling when the gNB is not provided with any PDU Set QoS Parameters from the SMF but still being provided with PDU Set information from the UPF.