3GPP TSG-SA2 Meeting #170	S2-2506311

# Goteborg, Sweden, August 25 - August 29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2150 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UIA_ARC Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-08-10 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR makes editorial corrections in the clauses that specify the QoS differentiation of traffic for Non-3GPP Device Identifier feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editorial mistakes would remain in the description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

Annex C (informative):
QoS differentiation of traffic of individual non-3GPP devices behind 5G-RG

This Annex describes how the traffic of individual non-3GPP devices behind a 5G-RG can be identified and provided with differentiated QoS.

Figure C-1: Example scenario for mapping traffic of individual non-3GPP devices to a PDU Session

In this example, two non-3GPP devices are mapped to PDU Session A and initially use the default QoS Flow (i.e., QFI 1); when differentiated QoS is requested for one device, the 5G-RG binds the traffic of the device to a Non-3GPP Device Identifier, and the traffic is mapped to a separate QoS Flow (i.e., QFI 2). Four other non-3GPP devices are mapped to PDU Session B based on their association with Connectivity Group ID X and initially use the default QoS Flow (i.e., QFI 3); when differentiated QoS is requested for two of these four devices, the 5G-RG binds their traffic to Non-3GPP Device Identifiers, and their traffic is mapped to separate QoS Flows (i.e., QFI 4 and QFI 5). Similarly, three other non-3GPP devices are mapped to PDU Session C based on their association with Connectivity Group ID Y and initially use the default QoS Flow (i.e., QFI 6); when differentiated, but the same, QoS is requested for two of these three devices, the 5G-RG binds their traffic to Non-3GPP Device Identifiers, and their traffic is mapped to a separate QoS Flow (i.e., QFI 7).

Figure C-2 illustrates a procedure which enables the 5GS to identify the traffic of individual non-3GPP devices initially using the same PDU Session behind a 5G-RG and provide differentiated QoS.

Figure C-2: Example procedure for enabling QoS differentiation for individual non-3GPP devices behind 5G-RG

0a.	Non-3GPP device 1 is connected to the 5G-RG.

0b.	To provide connectivity to non-3GPP device 1, the 5G-RG implements the existing behaviour of either using the URSP rule (optionally containing the Connectivity Group ID as described in clause 4.10b), or using UE Local Configuration, to map the traffic of the non-3GPP device 1 to a PDU Session.

0c.	Non-3GPP device 2 is connected to the 5G-RG.

0d.	To provide connectivity to non-3GPP device 2, the 5G-RG implements the existing behaviour of either using the URSP rule (optionally containing the Connectivity Group ID as described in clause 4.10b), or using UE Local Configuration, to map the traffic of the non-3GPP device 2 to the same PDU Session as non-3GPP device 1.

1.	The 5G-RG subscription owner or an authorized user, using mechanisms out of scope of 3GPP, requests differentiated QoS for the non-3GPP device 2 through the AF.

NOTE 1:	The request for differentiated service can be made through an operator portal hosted either in the 5G-RG or in the AF.

2.	The AF provisions the Non-3GPP Device Identifier Information for non-3GPP device 2 into the UDR, as defined in clause 4.15.6.15 of TS 23.502 [3].

NOTE 2:	Provisioning of the Non-3GPP Device Identifier Information into the UDR is done only for the non-3GPP devices that require differentiated QoS. This provisioning could be done before a device is connected to the 5G-RG.

3.	The 5G-RG requested PDU Session Modification procedure is triggered, as defined in clause 7.3.2.

NOTE 3:	For already existing QoS Flows of a Non-3GPP Device Identifier for whom there is a subsequent update of Non-3GPP Device Identifier Information, PCF may initiate the SM Policy Association Modification procedure, as defined in clause 4.16.5.2 of TS 23.502 [3].

4.	In response to step 1, using mechanisms out of scope of 3GPP, the operator portal returns a response to the 5G-RG subscription owner or the authorized user about the completion of the differentiated QoS request for non-3GPP device 2.

NOTE 4:	In the case of an operator portal deployed on the AF, the 5G-RG may indicate to the AF about the completion of step 3 using methods that are out of 3GPP’s scope.

# * * * * End of changes * * * *