SA WG2 Meeting #170	S2-2506480

25 – 29 Aug, 2025, Goteborg, SE

Source:	China Mobile

Title:	[WT#1.1] WT and KI for enhancement of UE and Core Network interaction

Document for:	Approval

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes motivation of 6G architecture design to support flexible interaction between UE and Core Network.

1. Justifications

The interaction between UE and Core Network is a key technology for mobile network. In 6G, multiple new types of service are introduced, and how to enable the new service to be introduced without much impact to existing NAS mechanism should be studied.

- Current network issues

- All NAS signalling is transferred by AMF, including a large number of NAS message which is not processing in AMF.

AMF is responsible for user connection management and mobility management, while other signalling such as SM, SMS, UE policy, and LCS are carried on NAS-MM.  Therefore, AMF process too much NAS signalling which is not terminated in AMF. If AMF is failed, all related functionalities will break down.

- The AMF is usually deployed in central network, which cannot meet the requirements of efficient signalling transmission.

For uplink NAS signalling, UE sends NAS signalling to AMF deployed in central network, then AMF transfer NAS signalling to target NF (e.g., L-SMF, LMF, etc.) deployed in edge network. This lead to low efficiency of NAS transmission, and also cannot meet requirements that signalling is transmission locally.

- New services requirements

6GS will support for beyond connectivity. These new services will introduce new parameters in signalling between UE and network. It may have impact on NAS mechanisms.

- Computing services will introduce new message or parameters in NAS signalling

For computing service, UE may send computing request as well as computing task to 6GC, in order to support traffic processing in CN. There are some interactions between UE and CN. It may lead to new NAS message or new NAS parameters.

- Sensing service will introduce new message or parameters in NAS signalling between UE and new NF

For sensing service, the UE and CN need to negotiate to support the UE based sensing. For examples, the UE may send sensing capability, location information to 6GC in order to register itself in Sensing Control Function. In these procedures, new parameters or new NAS message will be introduced in signalling between UE and CN.

For beyond connectivity, if reuse 5G NAS mechanism, every new service and related parameters should be specified and supported by UE, which may lead some difficulties in UE side to supporting the new features. The flexibility for new service introduction in UE side should be studied. In this KI , it needs to study how to introduce new service flexible and has lowest impact on existing functionalities.

**** First Change ****

Annex A.X. WT#1.1 Scope

WT#1.1 Study the support for control signaling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

**** Second Change ****

5.X. Key Issue #X: Enhancement of UE and Core Network interaction

### X.1	Description

In order to improve the control signalling transmission between UE and Core Network, the NAS mechanism may need to be updated. This key issue includes following aspects:

- How to categorize NAS functionalities e.g., basic functionalities,

- Whether and how to identify enhanced NAS functionality for beyond connectivity

- Whether and how to support for flexible and efficient introduction mechanism of new NAS functionality without any impact on other NAS functionality.

- Study the approaches for UE and Core Network interaction e.g., NAS enhancement or over user plane, Service based interface between UE and Core Network.

**** End of Changes ****