3GPP TSG SA WG2 Meeting #170	S2-2507450

25 – 29 Aug 2025, Goteborg, SE

# Source:	Apple

# Title:	New WID: UE power saving when processing packets for closed IP flows

# Document for:	Approval

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: UE power saving when processing packets for closed IP flows

Acronym: TEI20_UEPS

Unique identifier:

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

It has been experienced in the field that "straggler packets" from closed or partially closed IP flows continue to be sent to the UE, even when they hold no value to the UE anymore. When the UE is in idle mode, the UE shall be paged, initiate a Service Request procedure, enter CM-CONNECTED state, in order to receive these packets. After UE processing, the UE will eventually drop these packets as they belong to closed or partially closed IP flows.

This procedure results in waste of energy and unnecessary signaling messages. An optimized procedure is sought that enables the UE to inform the Core Network not page the UE for MT traffic that corresponds to certain port numbers or port number ranges.

# 4	Objective

This new WID aims to enable the following extensions to the Paging restriction feature:

WT-1: Define a new Paging Restriction Information that enables the UE to request the AMF to restrict paging for certain UDP or TCP port number(s) or port number ranges.

WT-2:  Define a new Paging Restriction Information that enables the UE to request the AMF to restrict all paging except for certain UDP or TCP port numbers(s) or port number ranges.

NOTE:	The use of Paging restriction feature is assumed to be allowed for non-MUSIM UEs.

This work item will require 1 TU to discuss and agree on the corresponding CR(s).

TU estimates and dependencies:

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT-1 |  | 0.5 | No | None |
| WT-2 |  | 0.5 | No | None |

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Extensions to the Paging restriction feature | TSG#111 (Mar 2026) |  |

# 6	Work item Rapporteur(s)

Apple

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

Apple