3GPP TSG-SA WG2 Meeting #170	S2-2507723

Goteborg, SE, Aug 25 - 29, 2025	(revison of S2-2506283)

Source:	OPPO

Title:	New Solution for KI#5: Sensing result delivery to the consumer

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution on Sensing result delivery to the consumer.

- Discussion

KI#5 have been defined in TR 23.700-14 as below:

This key issue addresses the exposure of sensing results to support sensing services. The aspects to be studied in this key issue include: - Study what types of sensing results with or without the sensing contextual information defined in TS 22.137 [2] could be exposed from the Core Network to the sensing service consumer. - How the Core Network exposes sensing results to the sensing service consumer (e.g., one time, periodic or event trigger).

For sensing result exposure, new sensing result may be derived from sensing data. Data collection often triggers sensing measurements. However, since multiple requests from diverse applications may seek identical results, reusing stored outcomes avoids redundant processing. This approach not only streamlines operations but also aligns with energy efficiency goals by minimizing unnecessary data generation and computation.

Sensing data storage preserves historical and real-time environmental context. Stored data ensures consistency for cross-service validation (e.g., AI training, network optimization) and supports regulatory requirements. For result exposure, centralized storage simplifies access via standardized APIs, allowing third parties or internal systems to retrieve processed insights (e.g., obstacle maps, interference patterns) without raw data duplication. It also facilitates on-demand querying, reducing redundant data collection. By decoupling storage from processing, stored datasets enable scalable, repeatable analytics, ensuring high-quality sensing results are dynamically exposed for applications like autonomous mobility or predictive network maintenance.

Sensing services, as a type of data service, can share the same data collection and capability exposure architecture with other data services (e.g., AI, positioning). However, due to the unique requirements of sensing services, there are also dedicated operations like sensing entity selection and sensing data processing. Thus, sensing service management is jointly implemented by the DSMF (Data Service Management Function) and SSMF (Sensing Service Management Function). The DSMF manages unified data collection and capability exposure. The SSMF handles sensing entity selection, sensing result generation, and storage.

This solution primarily focuses on a DSMF-based capability exposure framework, leveraging shared infrastructure for efficiency while ensuring specialized processing for sensing-specific tasks through the SSMF. By integrating sensing into the broader data service ecosystem, operators avoid redundant architectures while maintaining tailored support for sensing requirements like real-time entity coordination and environmental context handling.

- Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-14.

* * * Start of Change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | X |  | X | X | X |  |
| #2 |  | X |  |  | X |  |
| #3 | X | X | X | X | X | X |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  | X |  |  |
| #6 | X | X | X |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |
| #10 |  | X |  |  |  |  |
| #11 |  | X |  |  |  |  |
| #12 |  | X |  |  |  |  |
| #13 |  |  | X |  |  |  |
| #14 |  |  | X |  |  |  |
| #15 |  |  | X |  |  | X |
| #16 |  |  | X |  |  |  |
| #17 |  |  | X |  |  |  |
| #18 |  |  |  | X |  |  |
| #19 | X | X |  |  | X |  |
| #20 |  |  |  |  |  | X |
| #X | X |  |  | X |  |  |

* * * Next Change (All new text)* * * *

## 6.X	Solution #X: Sensing result delivery to the consumer

### 6.X.0	High-level solution Principles

This solution is for Key Issue#1 and Key Issue#5 on Sensing result delivery to the consumer.

- DSMF (Data Service Management Function) is introduced to control sensing data and the associated information collection and sensing result exposure, which is centrually deployed.

- SSMF (Sensing Service Management Function) is introduced to handle Sensing Entity selection, sensing result generation and storage, which is deployed in a distribution manner.

- Based on operator configuration, the sensing result can be stored either in the SSMF or a centrally deployed NF.

- The sensing service consumer(s) can be a UE or AF.

Editor's note:	Any potential privacy and security feasibility aspects regarding sensing result exposure to the UE are FFS and requires coordination with SA WG3.

Editor's note:	sensing result exposure to the UE needs to be checked with SA WG1.

- The sensing result can be transferred over Control Plane (CP) or User Plane (UP) / Data connection for storage and exposure.

- If there is available sensing result matching the service request (i.e. the sensing result fufills service requirement indidated in the service request) in the centrally deployed data storage NF, it can be provided to the consumer without triggering sensing procedure to the Sensing Entity.

### 6.X.1	Description

To enable sensing result exposure, the DSMF needs to:

- Determine the sensing result provider who transfers the sensing result to the consumer:

  - Based on operator configuration, DSMF may first check with the centrally deployed NF if there is available sensing result matching the service request

  - If there is available sensing result matching the service request, the centrally deployed NF storing sensing result is the sensing result provider;

  - Otherwise, SSMF is the sensing result provider

- Determine the sensing result transferring mode: Control Plane (CP) or User Plane (UP) / Data connection.

- Contral delivering the sensing results from the sensing result provider to the sensing service consumer(s).

To enable sensing result storage, the DSMF needs to:

- Determine the sensing result transferring mode: Control Plane (CP) or User Plane (UP) / Data connection.

- Contral delivering the sensing results from the SSMF to the sensing result storage NF if it is deployed.

If User Plane (UP) / Data connection is required to deliver sensing results to the consumer and the sensing result storage NF, a User Plane (UP) / Data connection needs to be established before transferring the sensing result.

DSMF determines sensing result transferring mode based on the following info:

- Capability of the sensing result provider for transferring sensing result over UP/data connection

- Capability of the sensing service consumer or the centrally deployed NF storingsensing result for transferring sensing result over UP/data connection

- Sensing result format

- Sensing service type

- Operator configuration

### 6.X.2	Procedures

Figure 6.X.2-1: Sensing result exposure and storage service flow

- DSMF receives a sensing service request, and the service request is successfully authorized. The request includes:

- Consumer ID/Address

- Sensing service Ttype

- Target sensing service area

- Expected sensing result，including sensing result format, time availability and QoS requirements

- Capability of the sensing service consumer for transferring sensing result over UP/data connection

If the consumer is UE, the service request is transmitted via AMF.

If the consumer is AF, the service request is transmitted via NEF.

- DSMF checks with the centrally deployed NF storing sensing result if there is available sensing result matching the service request, by sending sensing service type, target sensing service area and expected sensing result  received in step 1.

Editor's note:	More parameters to determine the stored sensing result matching the service request is FFS.

If there is matching sensing result, the sensing result storage NF responds to DSMF including either

Sensing result or

Indication of transferring sensing result over UP/data connection and optionally

the sensing result storage NF address

- If neither sensing result nor indication of transferring sensing result over UP is provided in step 3, DSMF sends the sensing service request to the SSMF, including what is received in step 1

- SSMF checks locally if there’s locally available sensing result matching the sensing service request. If no, the DSMF performs sensing transmitter/receiver discovery and selection.

- SSMF responds to DSMF.

- If there’s locally available sensing result matching the sensing service request, the SSMF includes in the request either the sensing result or

- indication of transferring sensing result over UP/data connection

- Otherwise, the SSMF includes in the response the selected sensing entity ID/address

- DSMF determines the sensing result provider:

  - If there is available sensing result matching the service request, the centrally deployed NF storing sensing result is the sensing result provider;

  - Otherwise, SSMF is the sensing result provider

- DSMF determines sensing result transferring mode (CP or UP/data connection) based on the following information:

- Indication of transferring sensing result over UP/data connection received at step 2 (from NF storing sensing data) or step 5 (from SSMF)

- Capability of the sensing service consumer for transferring sensing result over UP/data connection, sensing result format and sensing service type received at step 1

- Operator configuration

- whether sensing result is already received at step 2 or step 5.

- If sensing result is already received at step 3 or step 6, DSMF responds to sensing service consumer of the sensing result, and the rest steps are skipped; if DSMF determines to transfer sensing result over UP/data connection, it responds with the Indication of transferring sensing result over UP/Data connection and sensing result provider ID/Address. The respond is sent via NEF to the AF or via AMF to the UE.

- DSMF configures the sensing result provider (SSMF in 9a or sensing result storage NF in 9b) with one or multiple following parameters:

- Indication of transferring sensing result

- Indication of transferring sensing result over UP/Data connection

- The sensing service consumer ID/Address

- If Indication of transferring sensing result over UP/Data connection is received and there’s no available UP/Data Connection, new UP/Data Connection is established between SSMF and sensing service consumer (10a) or between sensing result storage NF and sensing service consumer (10b).

- SSMF triggers the sensing transmitter/receiver to perform sensing.

- Sensing data collection and result generation are performed.

- the sensing result is transferred to the sensing service consumer over UP/data connection (13a) or CP (13b)

- Based on operator configuration, SSMF stores the sensing result

- Based on operator configuration, the SSMF notifies sensing result storage NF about the sensing result over CP or data connnection.

- Sensing result storage NF stores the sensing result.

### 6.X.3	Impacts on services, entities and interfaces

DSMF:

  - Determine the sensing result provider(s).

  - Determine the sensing result transferring mode.

  - Contral delivering the sensing results from the sensing result provider(s) to the sensing service consumer(s).

  - Determine the sensing result storage entity(ies).

  - Contral delivering the sensing results from the sensing result provider(s) to the sensing result storage NF.

SSMF:

  - Support sensing result generation

  - Support sensing result storage

Receiver:

  - Support sensing data collection to SSMF

  - Support sensing measurement

Data storage NF:

  - Support sensing result storage

* * * End of Change * * * *