3GPP TSG-SA WG2 Meeting #170	S2-2506984

Goteborg, Sweden, 25 – 29 August 2025

Source:	ETRI

Title:	Solution 6.4 Update: Hierarchical SeMF Support

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The proposal enhances the discovery procedure and selection efficiency in multi-coverage, multi-domain sensing environments by introducing a hierarchical/multi-SeMF structure.

# 1. Discussion

This contribution proposes to update solution 6.4 of TR 23.700-14 by adding support for hierarchical SeMF deployment, addressing KI#1 (architecture) and KI#3 (discovery).

In the proposed solution, a Main SeMF can discover, select, and coordinate one or more Sub SeMFs to extend sensing coverage and capabilities across multiple domains. This enables more flexible and scalable handling of sensing service requests, especially when a single SeMF cannot cover the required sensing target area or capabilities.

Example scenarios where hierarchical SeMF support is used include:

- Fragmented coverage areas where multiple Sub SeMFs must be coordinated to provide a single sensing service.

- Multi-domain operations where SeMFs in different administrative or operational domains are coordinated by a Main SeMF.

- Dynamic selection of Sub SeMFs during an ongoing sensing session, for example, replacing or adding Sub SeMFs to adapt to changes in sensing coverage caused by the movement of sensing targets, or to change sensing entities to meet sensing quality requirements.

Key benefits include:

- Improved discovery flexibility by allowing a Main SeMF to retrieve and evaluate NF profiles of candidate Sub SeMFs.

- Support for coverage extension and multi-domain operation with minimizing the changes in the system architecture and interfaces for sensing services.

- Enhanced scalability and role separation in sensing service operation.

- Aggregation of sensing data/results from multiple Sub SeMFs under a unified policy determined by the Main SeMF.

# 2. Proposal

It is proposed to add the following text to TR 23.700-14.

* * * Start of Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | X |  | X | X | X |  |
| #2 |  | X |  |  | X |  |
| #3 | X | X | X | X | X | X |
| #4 | X |  | X |  |  |  |
| #5 | X |  |  | X |  |  |
| #6 | X | X | X |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |
| #10 |  | X |  |  |  |  |
| #11 |  | X |  |  |  |  |
| #12 |  | X |  |  |  |  |
| #13 |  |  | X |  |  |  |
| #14 |  |  | X |  |  |  |
| #15 |  |  | X |  |  | X |
| #16 |  |  | X |  |  |  |
| #17 |  |  | X |  |  |  |
| #18 |  |  |  | X |  |  |
| #19 | X | X |  |  | X |  |
| #20 |  |  |  |  |  | X |

* * * Next Change * * * *

6.4	Solution #4: Localized Sensing Management Architecture per Sensing Coverage

6.4.0	High-level solution Principles

The solution is based on the following general principles to support sensing:

-	The Sensing Management Function (SeMF) is newly introduced to support core sensing capabilities including sensing entity management and sensing data management,

-	The SeMF may be distributed to provide the sensing capabilities using the sensing entities within a specific sensing coverage,

-	The NEF manages sensing services requested by 3rd-parties (e.g. AFs) to support sensing service provisioning and sensing result exposure,

-	The SeMF may operate in a hierarchical structure, aggregating the sensing data or results from other SeMFs that serve dedicated coverages or domains.

It addresses the Key Issue #1 and #3. Any further details are covered in the other Key Issues.

6.4.1	Description

6.4.1.1	General

This solution addresses Key Issue #1 and #3 by proposing a distributed sensing management architecture per sensing coverage for supporting sensing capabilities within the 5G System. The architecture introduces a new Network Function (NF): the Sensing Management Function (SeMF) which is responsible for managing sensing entities and sensing data within a specific sensing coverage; and leveraging NEF functionalities for sensing service provisioning. The SeMF is responsible for the core functionalities related to managing sensing resources and handling sensing data. Its key roles include:

-	Sensing Entity Management:

This involves the registration of sensing-capable entities (gNBs), their discovery and selection for specific requests (as per KI#3), and their configuration (parameter provisioning, as per KI#6). Sensing entities register their capabilities including coverage, role (e.g. transmitter, receiver, or both), sensing-related performance parameters (e.g. range, resolution, accuracy, latency), resource information (e.g. energy consumption), and features (e.g. pre-processing).

-	Sensing Data Management:

This covers the collection of sensing data from sensing entities (e.g. via N1/N2 or N3), processing the collected data to generate sensing results. It also includes storing the sensing data and/or result for purposes such as enabling analytics for predictions, historical analysis, or re-use for sensing with static objects minimizing resource consumption (as per KI#4).

The NEF manages sensing services requested by 3rd-parties (e.g. AFs). Its primary roles are related to provisioning and exposing sensing results. Its key functionalities include:

-	Sensing Service Provisioning:

This involves authorizing sensing service requests (as per KI#2), enforcing policies and service-specific requirements, discovering and selecting the appropriate SeMF(s) to handle a request (e.g. via NRF, using sensing coverage), and triggering the sensing request to the selected SeMF(s) (as per KI#3). It also includes mapping between sensing target areas (e.g. geographical area) specified in AF requests and network-internal identifiers

-	Sensing Result Exposure:

This involves receiving sensing results from SeMF(s) and exposing them to the requesting entity (e.g. AF as per KI#5). The content of exposed results may include, for example, object type, location, velocity, direction, etc. Results may be provided as one-time reports, periodically, or based on events.

6.4.1.2	Hierarchical SeMF Support

This subclause specifies support for hierarchical deployment of multiple SeMFs to enhance discovery, (re-)selection, and coordination across multiple sensing coverages or domains.

An SeMF may coordinate with other SeMFs in different roles depending on the relationship:

- A Main SeMF receives sensing service requests from NEF (or trusted AF) and coordinates with one or more Sub SeMFs whose sensing coverage corresponds to portions of the required sensing coverage area.

- A Sub SeMF manages sensing entities within its dedicated coverage or domain and provides sensing data or results to the Main SeMF that coordinates it.

- A Sub SeMF may itself coordinate other Sub SeMFs, acting as a Main SeMF in that relationship.

Key functional aspects include:

- Discovery and Selection of sub SeMFs:

A Main SeMF maps the requested sensing target area to its own sensing coverage, determines if multiple Sub SeMFs are required, and discovers candidate Sub SeMFs via NRF using NF profile attributes such as coverage, domain ID, capabilities, and load. Selection is based on matching portions of the required area and fulfilling service requirements and policy constraints.

- Coordination between Main SeMF and Sub SeMFs:

The Main SeMF sends sensing requests to selected Sub SeMFs, including additional parameters for coordination such as assigned coverage area, required capabilities, aggregation policy (e.g., raw data or pre-processed result), and reporting mode.

- Data and Result Handling from Sub SeMFs:

The Main SeMF determines whether each Sub SeMF sends pre-processed results or raw data, according to the aggregation policy. The Main SeMF aggregates and processes the received data or results to produce the final sensing result.

- Dynamic Re-selection of Sub SeMFs:

The Main SeMF may replace or add Sub SeMFs during an ongoing sensing session to adapt to changes in sensing coverage (e.g., movement of sensing targets) or to change sensing entities to meet sensing quality requirements.

The signalling procedures and interfaces remains same but with the following additions or extensions for hierarchical SeMF support:

- SeMF discovery (Main SeMF → NRF): additional NF profile attributes for Sub SeMF such as (sub-)coverage, domain ID, capabilities, and load.

- Sensing request (Main SeMF → Sub SeMF): carry coordination parameters including required capabilities, aggregation policy (e.g., raw data or pre-processed result), and reporting mode.

- Sensing result reporting (Sub SeMF → Main SeMF): content (data vs. result) determined by aggregation policy.

- Sensing data processing (by Sub SeMF): optional pre-processing before reporting to Main SeMF, according to the specified aggregation policy.

- Data/result aggregation (by Main SeMF): combines sensing data or results from multiple Sub SeMFs.

- Dynamic re-selection (by Main SeMF): replaces or adds Sub SeMFs during an ongoing session when coverage or sensing quality changes.

6.4.2	Procedures

6.4.2.1	General

Figure 6.4.2-1: Overall procedure for handling sensing service request

A high-level procedure for a sensing service request may involve the following steps:

0.	(Assumption) Sensing entities are already registered to SeMF with their sensing capabilities, which may be covered in KI#3.

Editor's note:		The details of sensing entity registration procedure is FFS.

1.	An Application Function (AF) initiates a sensing service request towards the 5G System via NEF. The request includes details such as the sensing type (e.g. object detection, tracking, environment monitoring, etc.), target (e.g. object, area), required service-specific performance (e.g. accuracy, resolution, latency) and potentially assistance information (e.g. object/area details).

2.	The NEF performs authorization based on the request details and sensing service profile information (e.g. interacting with UDR).

Editor's note:	The use of UDR for sensing service authorization is FFS.

3.	Based on the target area/object and required capabilities specified in the request, the NEF discovers and selects one or more suitable SeMFs. This involves mapping the sensing target area (if specified) to sensing coverage and querying the NRF for SeMF(s) matching the sensing coverage and required sensing capabilities.

4.	The NEF triggers the sensing request towards the selected SeMF(s), including the relevant sensing target information and assistance information derived from the AF request.

5.	Upon receiving the sensing request, the SeMF discovers and selects the specific sensing entities (gNBs) under its management that are capable of fulfilling the request for the specified target area/object and service-specific performance.

6.	The SeMF configures the selected sensing entities with necessary parameters for the sensing request.

7.	The SeMF requests and collects sensing data from the configured sensing entities. Transport for the sensing data collection may occur over CP (N1/N2) or UP (N3).

8.	The SeMF processes the collected sensing data to generate the sensing results. The collected sensing data and/or sensing results may be stored by the SeMF for further use.

9.	The SeMF reports the sensing results, potentially with contextual information, back to the requesting NEF.

10.	The NEF receives sensing results from the SeMF and exposes the sensing result to the requesting AF.

6.4.2.2	Procedures for Hierarchical SeMF Support

This subclause describes the procedure for supporting hierarchical deployment of SeMFs, as introduced in clause 6.4.1.2. The procedure extends the general procedure in clause 6.4.2.1 by adding the discovery, selection, and coordination steps between a Main SeMF and one or more Sub SeMFs.

Figure 6.4.2-2: Procedure for hierarchical SeMF operations

The procedure for a sensing service request with hierarchical SeMF support may include the following steps:

- The AF sends a sensing service request to the NEF, including parameters such as sensing target area, required capabilities, and reporting conditions. (same as step 1 in clause 6.4.2.1)

- The NEF authorizes the request, discovers and selects the appropriate Main SeMF, and triggers the sensing request to the selected Main SeMF. (same as step 2-4 in clause 6.4.2.1)

- The Main SeMF queries the NRF to discover candidate Sub SeMFs that match the service requirements, using NF profile attributes such as sensing coverage, domains, capabilities, load, etc.

- The Main SeMF sends the sensing requests to the selected Sub SeMFs for their coverages, including additional coordination information such as required capabilities, aggregation policy (e.g., raw data or pre-processed result). The Main SeMF may re-select one or more Sub SeMFs during the sensing session to address coverage changes (e.g., due to mobility of sensing entities or sensing targets), QoS degradation, load balancing, or failure recovery.

- Each Sub SeMF performs localized discovery and configuration of sensing entities in its coverage and initiates sensing measurements. (same as step 5-7 in clause 6.4.2.1)

- The Sub SeMF may optionally pre-process the collected sensing data before reporting to the Main SeMF, in accordance with the Main SeMF’s aggregation policy.

- The Sub SeMF sends either raw sensing data or pre-processed results to the Main SeMF as instructed.

- The Main SeMF aggregates and/or processes the sensing data or results collected from multiple Sub SeMFs to produce the final sensing result.

- The Main SeMF provides the final sensing result to the NEF. (same as step 9 in clause 6.4.2.1)

- The NEF exposes the final sensing result to the AF, following the requested reporting mode. (same as step 10 in clause 6.4.2.1)

6.4.3	Impacts on Services, Entities and Interfaces

The following NFs and interfaces are impacted:

SeMF:

-	a new network function for sensing entity management and sensing data management under its sensing coverage.

-	new services and interfaces for sensing entity registration, sensing data collection, and sensing request.

-	support for hierarchical operation, where a Main SeMF discovers, selects, and coordinates one or more Sub SeMFs, and receives sensing data or results according to an aggregation policy.

NEF:

-	enhancement for sensing service provisioning and sensing result exposure.

-	new services and interfaces for sensing service request (including subscription) from AF.

NRF:

-	updates to NF profile management for discovery of SeMF.

* * * End of Change * * * *