3GPP TSG-WG SA2 Meeting #170	S2-2507298

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[WT#1.2, Architecture Simplification and Impacts for Localized Service Access] Justifications and Proposals for 6G architecture simplification

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract: This document provides a gap analysis of 5G architecture and argues for the need to improve flexibility, scalability and resilience of the 6G architecture. It is proposed to achieve this goal by simplifying the overall 3GPP system, in particular through optimization of inter-functionality or inter-function dependencies, study of NF logic and data separation and architecture support for PLMN offered localized service access.

1. 	Introduction

6G provides the opportunity for operators to achieve CAPEX/OPEX reduction by introducing improvements to the overall 3GPP system performance, e.g., by means of simplifying the overall system, integrating of new technologies, etc. The 6G study goal is to create lean and streamlined standards for 6G, e.g., by dimensioning an appropriate set of functionalities, minimizing the specification of multiple options for the same functionality, avoiding excessive configurations, etc.

5G core enabled a cloud-native deployment, and promoted business opportunities by unlocking the possibility of providing services to verticals alongside the traditional consumer services. To shape the 6G architecture, this document first provides a gap analysis and identifies the limitations of 5G and subsequently provides architectural motivations as the justifications for studying the optimization of inter-functionality or inter-function dependencies, separation of NF logic and data and architecture support for PLMN offered localized service in 6G. It is expected that the study outcome will contribute towards designing a 6G architecture with enhanced flexibility, scalability and resilience by means of a simplified overall system.

2. 	Justification for optimization of inter-functionality dependencies

In 5G networks, there are many dependencies between network functions as illustrated by these for examples: 1) the SMF relies on AMF for UE context, mobility state, connected state and SMF selection, triggering the session release. 2) the SMF consults the PCF for QoS and policy rules before setting up user plane resources. 3) the AMF consults the PCF for access and mobility control before granting UE access during UE registration procedure. 4) NFs like SMF, AMF, and PCF rely on NRF for discovering other NFs. This web of dependencies means that new features often cannot be supported by introducing just a new NF, but require the upgrade of interdependent NFs. For example, upgrading or customizing downstream NFs also requires upstream NFs to adapt the selection and invocation of downstream NFs. Figure 2.2-1 shows that most of the new features introduced in Release 18 required support in multiple NFs. The consequence is that for the deployment of these new features besides the new NFs a number of existing NFs needed to be upgraded.

Figure 2.2-1: Impact of New Features on NF

A particularly problematic example is shown in figure 2.2-2. NF discovery and selection heavily depends on coordination among the producer NF, consumer NF, SCP, and NRF. This interdependency, compound with different selection logic and the four communication models (A, B, C, and D) used to implement the selection logic makes upgrade of the involved NFs, SCP, and NRF for NF discovery and selection extremely difficult especially when the network elements are provided by different vendors.

Figure 2.2-2 Impact of NF discovery and selection

For 6G SA1 has proposed new requirements to reduce signalling and minimize connection setup time for the 6G system as described in clause 5.9.4 of TS 22870: [PR 5.9.4.2-1] The 6G system shall be able to support means to simplify network operation and service delivery compared to 5G system, e.g., in order to reduce signalling, minimize connection setup time. This goal is very difficult to achieve by re-using the 5G design which is based on HTTP and Restful API since the actual end-to-end procedures still utilize a message serialization design, which means that each step must wait for the previous step to complete before proceeding. As the number of NFs, services, and interfaces within the network continues to increase, the number of signalling steps for a single procedure also grows, leading to increased signalling latency and further enhancing the coupling between NFs.

Observation 1: The existing coupling NFs based architecture and serial procedure, is not flexible and scalable for support of new features or new services, network upgrade, network failure recovery, scaling in/out and so on.

Proposal 1: Study how to optimize the inter-functionality dependencies and serial procedure for 6G architecture, e.g., via dimensioning an appropriate set of functionalities to support network robustness and resiliency, network simplification.

3. 	Justification for study on separation of NF logic and NF data

In 5GC, stateful design faces several key challenges, including poor resilience and scalability, difficulty in maintaining state consistency, performance bottlenecks, and high maintenance costs. As the number of users and service complexity increase, the need for state synchronization and persistence across nodes becomes more demanding, leading to difficulties in system expansion, prolonged service downtime during failures, data conflicts, and increased latency. Additionally, the complexity of state management significantly raises operational and maintenance efforts.

As shown in Figure 3.2-1, different type of UE contexts (e.g., MM context, SM context) are defined based on NF types, and only one type of UE context is allowed to be shared among the NF instances in the same Set, provided that those instances are from the same vendor. This arrangement results in a large amount of duplicated data between different type of UE contexts, for example DNN, S-NSSAI, UE location, session status, connected status, reachable status may be stored in several NFs at once. This duplication leads to the challenge of keeping data consistency requiring large amount of UE context data to be transferred between different NFs through NF interaction signalling. This synchronisation also aggravates the problem of coupling between NFs discussed in the previous clause.

Figure 3.2-1: Data Duplication and Redundancy between NFs

Observation 2.1: Duplication between different types of UE context brings challenges to keep data consistency between NFs and aggravates the coupling between NFs, apart from waste of network resources.

In the 5G architecture CP anchors such as the SMF play a critical role in the context synchronisation as they maintain UE state information, ensuring session continuity, mobility management, and efficient signalling procedures. Those anchors, once selected, e.g., the SMF selected during PDU session establishment will need to receive all SM messages creating congestion risks, adding delay, sub-optimal routing of signalling and additional functional impacts on CP anchors in case of localized service upgrade or provisioning.

Furthermore, the UE associations need to be maintained among NFs in 5G to serve the UE, e.g., AM Policy Association, MM Policy Association, SM Policy Association, AMF-SMF association, N5 association etc. Due to stateful design in 5G, after an initial interaction between NF consumer and producer, NF producer stores the context data, and correspondingly NF consumer stores the instance ID of the NF producer so that subsequent requests have to be sent to the same instance holding the context data. During scaling in or out, e.g. for the purpose of load balancing, before new SM messages can be successfully distributed a large exchange of messages is needed since all the NF consumers need to be notified so that they can update the NF producer instance ID. This introduces scalability challenges and is not flexible to support network upgrade, network failure recovery, scaling in/out.

Statefulness of 5G architecture and binding of the context data to a specific NF also makes it difficult to prevent signalling storms which is instead a clear SA1 requirement (see clause 5.6.5 of 3GPP TR 22.870, [PR 5.6.5.6-1] The 6G network shall provide a mechanism to prevent signalling storm arising from recovery of network failure). Signalling storms should also be prevented when rolling out new services, (see clause 5.9.5 of 3GPP TR 22.870), or provisioning of new localized service as part of the PLMN  (see clause 5.6.2 of 3GPP TR 22.870).

Figure 3.2-2: Signalling Storm Due to NF Outage

As shown in Figure 3.2-2, when the AMF detects SMF/UDM failure, the AMF would deactivate a large number of affected users. These users will attempt to reconnect within a short period, ultimately leading to a signalling storm. In addition, the AMF upgrade may also cause a signalling storm because the AMF updates the links with a large number of affected RANs under the wide AMF service area in a short period.

Observation 2.2: Stateful NFs limit the network scalability and resilience, increase inter NF traffic, restricts load balancing, result in signalling detour and makes the architecture prone to signalling storms.

Proposal 2: It should be studied how to separate the data management and service logic processing to enable efficient distribution of message requests, facilitate data sharing among different NFs, e.g., with load balancing and without extra delay and congestion risk, and to assist prevention of signalling storm.

NOTE: 	Data framework, to be studied in WT#5, can be leveraged to achieve separation of UE context data and service logic.

4. 	PLMN offered localized service access

4.1 	Localized service access

Localized services can be offered via PLMN to support various services in public area, for example:

-	Provisioning local data connectivity service and voice service in the disaster area in response to an urgent event (e.g., disaster, emergency and DDoS events) to improve network resilience, as described in clause 5.6.2 of TR 22.870.

-	Verification of the new network services (e.g., XR services) rolling out on the local area network, running test on the selected users in response to new business contract to enable agile and zero-downtime rollout of new services, as described in clause 5.9.5 of TR 22.870.

-	Offering customized local area network service (e.g., customized capabilities, and more localized management) for vertical scenarios like factories or campuses, adapting to differentiated and changing demands on network services of various local stakeholders, to simplify the network and improve the network flexibility, as described in clause 5.9.6 and 11.9 of TR 22.870.

Offering localised services requires the operator to configure part of the PLMN as a local area network. The local areas network should be easy to deploy, offer the adequate performance and efficient costs, be sufficiently flexible to meet different demands etc. Taking these points into consideration, the PLMN offered localized service access can be characterized by the following attributes:

-	it is possible to instantiate millions of PLMN offered localized networks

-	the requirements for the PLMN offered localized services become increasingly diverse across difference scenarios and are dynamically changeable

-	flexible enough to enable agile service delivery

-	allow freedom to enable local control: local subscription, local signalling control, and local traffic forwarding for users, under the lightweight/no control of the PLMN

-	changes of PLMN offered localized network are confined locally

-	flexible inter-interactions or inter-connectivity among networks of the same PLMN operator

4.2 	Gap Analysis

SA1 has proposed new requirements to support certain level of local control for PLMN offered localized service access, TR 22.870, clause 5.6.2, [PR 5.6.2.6-1] Subject to operator policies and service level agreements, the 6G system shall enable operators to provision network services as part of the operator’s PLMN network on-demand, e.g., in response to an urgent event (e.g., disaster, emergency and DDoS events), with certain level of local control and specific functionalities in a given area during a specific time period. 5G defined many features that could be used to provide localized service towards verticals such as e.g., NPN, EDGE, Slicing etc., however, those features only provide the centralized control for CP signalling, user subscription, policy management. Figure 4.2-1 illustrates how these features can be used to create localized service access but they also suffer from a number of shortcomings. In particular:

-	PNI-NPN requires PLMN to offer UDM, PCF, CAG and slicing services;

-	EDGE requires PLMN to offer AMF, PCF, UDM and SMF services;

-	SLICING requires PLMN to offer network resources such that AMF, SMF, UDM, PCF, UPF are shared among tenants;

-	SNPN can be deployed by PLMN operator, but SNPN architecture uses underlay-overlay arrangements to support simultaneous connection to localized services and PLMN services. This arrangement is both inefficient and cumbersome for fulfilling the interworking requirement proposed in clause 5.6.2 of 3GPP TR 22.870: [PR 5.6.2.6-2] Subject to operator policies and service level agreements, the 6G system shall enable a network operator to authorize a UE, that is subscribed to local network services, to access services from the PLMN of the same operator.

Figure 4.2-1: Architecture for PNI-NPN, Edge, Slicing, and SNPN

5G also attempts to define the certain level of local control on dedicated aspects: e.g., session management in ETSUN or information exposure in UPEAS. However, those features only allow very limited local control:

-	In case of ETSUN, when the UE is outside of the SMF service area, an I-SMF and I-UPF are inserted to locally serve the UE location, and SMF and UPF anchor services are still remained for the PDU session.

-	UPEAS only enables the local exposure for certain information (e.g., Private UE IP, Usage Data Usage etc.) via local NEF and UPF.

Observation 3.1: PLMN offered localized service access with the desired level of local control cannot be fully supported in the context of 5G architecture.

As described in clause 11.9 and 5.6.2 of TS 22.870, the UE that is subscribed to the localized services can also use PLMN services or other localized services deployed by the same operator, [PR 11.9.6-2] Subject to an agreement between a PLMN operator and authorized 3rd party, operator policies and regulatory requirements, the 6G network shall support a mechanism to authorize and authenticate UEs owned by a trusted authorized 3rd party to access services from the PLMN operator. When the NRF is deployed at slice-specific level and the localized services are bound to a specific slice, there is no possibility to enable the interactions between the NFs tied to localized services (e.g., for verticals) and other NFs in the PLMN. When the NRF is deployed at PLMN-level or shared-slice level as the centralized NRF, in order to discover NFs tied to localized services by other NFs (e.g., AMF, SMF) in the network (or vice versa), following the current 5G mechanism, those NFs should register their NF profiles directly to the centralized NRF, or indirectly via an intermediate NRF serving the corresponding localized services, as specified in clause 5.2.2.2.3 of 3GPP TS 29.510.

c) the registering NRF may include nrfInfo which contains the information of e.g. udrInfo, udmInfo, ausfInfo, amfInfo, smfInfo, upfInfo, pcfInfo, bsfInfo, nefInfo, chfInfo, pcscfInfo, lmfInfo, gmlcInfo, aanfInfo, nfInfo and nsacfInfo in the nfProfile locally configured in the NRF or the NRF received during registration of other NFs, this means the registering NRF is able to provide service for discovery of NFs subject to that information;

This means the centralized NRF holds the profile for all NFs (not only NFs for PLMN services, but also NFs tied to localized services), causing extra load on NRF. And more importantly, this brings security risks because any topology changes (including incorrect or malicious topology information) from substantial localized services are updated to the centralized NRF, and also this potentially affects other PLMN/localized services in the PLMN network.

Observation 3.2: If localized service uses the current 5G NF management mechanism, it might bring heavy load on the centralized NRF and any changes of localized services will cause wide impacts across the entire network.

In case of cross-domain interaction in 5G, e.g., roaming between visiting and home network as shown in Figure 4.2-2, NF discovery request and service request are both rerouted over the home network as shown in the below figure, causing extra signalling and procedure delay. This problem also exists when a UE, that is subscribed to local network services, accesses PLMN services, as described in clause 11.9 and 5.6.2 of TS 22.870. In this case, the local network services and PLMN services can be treated as visiting PLMN and home PLMN respectively.

Figure 4.2-2: NF discovery and service request signalling in roaming cases

Observation 3.3: Current 5G NF discovery and selection mechanism is inefficient for cross-domain message forwarding and routing in case of localized service access.

When UE accesses localized services, local NFs, i.e., AMF and SMF in 5G, need to discover the centralized UDM for UE subscription data retrieval. As shown in Figure 4.2-3, local NFs tied to localized services can subscribe to changes of the UDM registered in the central NRF via the local NRF. In case of network upgrade, the centralized NRF needs to notify all subscribed NFs including local NFs, of NF profile changes of the upgraded NF, while such NF profiles can be the UDM that includes lots of SUPI ranges. This results in signalling storm of notification messages with large payload in the whole network.

Figure 4.2-3: Subscribe-Notify with intermediate NRF

In order to apply dedicated SLA for localized services, UE’s PLMN subscription shall be revised. This may not be authorized due to the limitation for the subscriber’s agreement with its operator. In addition, even if the subscription modification is authorized, since the localized services are offered within specific area and/or time, such frequent modification of PLMN subscription (e.g., due to UE’s mobility or the expiry of localized services) is not efficient and large signalling would be caused.

Observation 3.4: Centralized subscription management cannot support local policy control efficiently. If localized service uses the current 5G NF management mechanism, it might cause signalling storm with large payload packet when the network upgrades, which happens frequently for localized service.

Study how to design 6G architecture to support the PLMN offered localized service access with certain level of local control is necessary. The motivation is not only to satisfy the SA1 requirements, but also, this promotes business opportunities for PLMN operators, and extremely enlarges the monetization space for operator’s PLMN network. In addition, this can help gain benefits on the operators’ CAPEX/OPEX reduction when providing localized services, e.g., by signaling overhead reduction in case of cross-domain interactions and avoiding signalling storm in case of network upgrade or network scaling.

Proposal 3: It is proposed to study how to enable the 6G architecture to support certain level of local control for PLMN offered localized service access.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.801-01.

# * * * * First change * * * *

Annex A.X. WT#1.2 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

Define the overall 6G architecture as collection of capabilities and high-level functionalities considering the following sub work tasks to support 6G access network:

-	Study the 6G architecture simplification for lean and streamlined design, and architecture impacts for localized service access, see clause 5.X and 5.Y.

Editor's Note: The scope is to be updated due to impact from other WTs, such as WT#3, #4, #5, #6.

Annex A.X. WT#5 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

-	Support the management of the UE context type data.

# * * * * Second change * * * *

5	Key Issues

Editor's Note: This clause identifies key issues to be addressed.

5.X	Key Issue #X: Architecture simplification to support lean and streamlined design

The existing NFs interdependency, stateful NF design and serial procedures create challenges on the path to improve network resilience, network scalability and network simplification. Roll out of new features or new services, network upgrades, network failure recovery, and scaling in/out, might produce a chain of affected network functions/functionalities, association update signalling storm, inefficient and imbalanced message distribution etc.

In order to create lean and streamlined standards it is advantageous to simplify the overall system by dimensioning an appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations. This key issue will study the simplification of the 6G system architecture taking into consideration the following aspects:

-	how to support dimensioning an appropriate set of functionalities to minimize the inter-functionality or inter-function dependencies, e.g., minimize the signalling latency by adopting parallel processing for orthogonal functionality and inter-function interaction reduction via high cohesive network functions.

-	how to support uniform message forwarding and routing, NF discovery and selection.

-	how to support separation of NF data and logic, for example to minimize the signalling arising from network upgrade, network dynamic scale-in /scale-out, new feature/ service delivery, network failure recovery etc.

Editor's Note: New KIs, derived from other WTs, such as WT#3, 4, 5, 6, are to be added.

5.Y	Key Issue #Y: Architecture impacts to support localized service offered by PLMN

Localized services can be offered via PLMN to support several services in public area, for example: 1) Provisioning local data connectivity service and voice service in the disaster area in response to an urgent event (e.g., disaster, emergency and DDoS events) to improve network resilience, as described in clause 5.6.2 of TR 22.870[xx]. 2) Verification of new network services (e.g., XR services) rolling out on the local area network, running test on the selected users in response to new business contract for fast service delivery, as described in clause 5.9.5 of TR 22.870[xx]. 3) Offering customized local area network service (e.g., customized capabilities, and more localized management) for vertical scenarios like factories or campuses, adapting to differentiated and changing demands on network services of various local stakeholders, to simplify the network and improve the network flexibility, as described in clause 5.9.6 and 11.9 of TR 22.870[xx].

To offer localized services, the operators need to configure part of the PLMN as a local area network. The local area network should be easy to deploy and use, provide adequate performance, be cost effective, sufficiently flexibile to meet demands etc. This key issue will study how to enable the 6G architecture to support certain level of local control for PLMN offered localized service access, including:

-	How to enable the architecture to support local control for control plane signalling, user subscription, policy management, topology management etc.

-	How to minimize the impacts towards PLMN when performing NF discovery and selection, message forwarding and routing between localized services dedicated network functions and PLMN services dedicated network functions.

-	How to support fast delivery of the new or updated services/capabilities for localized services with no/minimal disruption to existing services.

-	How to enable a UE to access the localized services, which are provided by dedicated network function instances within the PLMN.

-	How to authorize the UE, that is subscribed to PLMN and to localized services, to access the localized services when UE is in specific area, with minimum impacts to UE’s home network.

Editor's Note: New KIs, derived from other WTs, such as WT#3, 4, 5, 6, are to be added.

# * * * * End of changes * * * *